SA WG2 Meeting #142e	S2-2008792

# Nov 16th – Nov 20th, 2020 ; Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	Conclusions for PC5 DRX operation

Document for:	Approval

Agenda Item:	8.12

Work Item / Release:	FS_eV2XARC_Ph2 / Rel-17

Abstract of the contribution: This paper adds two conclusions to be added in case RAN WG provides positive feedback to the provisioning of PC5 DRX configuration assistance info in the form of default PC5 DRX schedules.

# 1 Introduction

This paper adds two conclusions to be added in case RAN WG provides positive feedback to the provisioning of PC5 DRX configuration assistance info in the form of default PC5 DRX schedules and based on considerations about the level on which such default schedules shall be provided.

# 2 Proposal

It is proposed to update TR 23.776-020 as follows:

* * * * First Change * * * *

## 7.2	Conclusions for PC5 DRX operations

For Key Issue #1 (Support of QoS aware NR PC5 power efficiency for pedestrian UEs), regarding NR PC5 DRX operations the following principles are taken as the interim conclusion:

-	The V2X application layer provides inputs for V2X layer to determine PC5 DRX assistance information.

Editor's note:	It is FFS whether the inputs described in TS 23.287 [5] (e.g. V2X Application Requirements) are sufficient or additional inputs (e.g. traffic pattern) are needed from the V2X application layer.

-	The V2X layer may provide the PC5 DRX assistance information to the AS layer.

-	The PC5 DRX assistance information may include default PC5 DRX schedules per V2X service type (or per combination of V2X service types)

Editor's note:	It is FFS whether PC5 DRX assistance information, which is provided to the AS layer by the V2X layer, is defined in 3GPP and what is the content of the PC5 DRX assistance information.

Editor's note:	It is FFS if the PC5 DRX assistance information is per mode of communication (i.e. unicast/broadcast/groupcast).

-	The Access Stratum (AS) layer determines the PC5 DRX for V2X communication over PC5 reference point to enable pedestrian UE power saving.

-	The AS layer provides the applied PC5 DRX information to the V2X layer.

-	For unicast, the PC5 DRX may be negotiated between the UEs in AS layer, pending on the feedback from RAN2.

Editor's note:	The design principles need to be evaluated by RAN WGs.

Editor's note:	It is FFS whether DRX related parameters are needed and what is the contents, which is (pre-)configured on the UE to assist the V2X layer determine the PC5 DRX assistance information.

* * * * End of Changes * * * *