3GPP TSG-WG SA2 Meeting #142E 	S2-2008811

16 - 20 November, 2020, Electronic meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR |  | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | GPSI translation based on UE IP address |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI17 |  |  |  |  |  | Date: |  |  | 2020-11-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In some use cases, it may be reasonable to expect that the Application Client (AC) within the UE can provide the GPSI (e.g. MSISDN) to the EAS as part of the application layer protocol, however, typically this may not be the case (i.e. the AC is not aware of its GPSI, neither MSISDN nor External Identifier). Yet in some other use cases, the user due to privacy concerns may not be comfortable to share his/her MSISDN with certain applications. This means, there are many scenarios where the application is unaware of the UE identity and yet to serve the user/UE, the application server (i.e. EAS) would need to query for certain information about the UE (e.g. location). Additionally, the AC within the UE may not be a trustworthy source for identifiers, thus a network provided identifier is preferred. Under such circumstances, the EAS that is not aware of UE identifier(s) can only provide UE's IP address (over the API) as the identifier of the UE. Therefore, It is required to provide exposure of network services to AFs using a static identifier when only an IP address of the UE is known to AF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to return GPSI as a static UE identifiers when the static UE identifier is requested by AF using Nbsf_Management_Discovery service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5GC operator is unable to provide network exposure service to AF that is unaware of UE identifier. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.13.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (5.2.13.2.4) * * * *

5.2.13.2.4	Nbsf_Management_Discovery service operation

Service Operation name: Nbsf_Management discovery

Description: Discovers the PCF and PCF set selected for the tuple (UE address(es), SUPI, GPSI, DNN, S-NSSAI).

Inputs, Required: UE address (i.e. IP address or MAC address), DNN [Conditional], S-NSSAI [Conditional].

NOTE:	For TSN scenarios the UE address contains the DS-TT port MAC address.

Inputs, Optional: SUPI, GPSI, request for a static UE identifier.

Outputs, Required: PCF address(es), PCF instance ID [Conditional, if available] and PCF Set ID [Conditional, if available], level of Binding [Conditional, if available] (see clause 6.3.1.0 of TS 23.501 [2]), GPSI [Conditional, if the static UE identifier is requested and GPSI is available] (see clause x.x.x of TS 23.503 [20].

Outputs, Optional: None.