3GPP TSG-SA/WG2 Meeting #141e	S2-2008816

# Elbonia, 16-20 Nov, 2020

Source:	ZTE, Convida Wireless, Nokia, Nokia Shanghai Bell

Title:	KI#2 conclusion on UE cache

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution proposes conclusion for KI#2

1	Discussion

The key issue#2 has been concluded in last meeting. However there are still some FFS needs to be resolved. These EN need to be resolved and further conclusions are proposed

For clause 9.2.2 Conclusions for Key Issue #2: UE DNS cache. The title should be changed to reflect that this conclusion is for UE based EAS rediscovery. The conclusion should clarify the relationship between the UE based EAS rediscovery and the AF based EAS rediscovery as described in clause 9.2.3. It is proposed the AF sends information to the SMF(similar concept as solution#34), so the SMF can determine to clear the DNS cache in the UE via NAS. Also the AF need not be able to know the DNS cache entry in the UE side, so it is proposed that the AF sends the trigger for EAS relocation to the SMF and the SMF makes the decision whether all DNS caches should be removed when the UE receives such indication. Also an NOTE is added to clarify that the application may determine to flush the DNS cache for this application at any time.

2	Proposal

It is proposed to agree the changes:

/********************Start of Change***************/

9.2.2	Conclusions for Key Issue #2: UE based EAS rediscovery

The new DNS query may be performed to reselect a new EAS due to application internal logic. Before the new DNS query the DNS cache in the UE needs to be cleared. For UE DNS cache related issue, it is concluded:

-	The AF may send EAS relocation information to SMF, and the SMF makes the decision to clear the DNS cache in the UE. The SMF may provide DNS cache clear indication within NAS message to UE as described in Solution #32 and #34 (only DNS cache related part). Based on the received DNS cache clear indication, the UE cleans the DNS cache related to the PDU Session. This does not impact the application client.

NOTE 1:	If connectivity is still available to the source EAS, then the application can go on interfacing the source EAS and can select a new EAS after the DNS caching is flushed by the applications or cache timer expires.

NOTE 2:	If the UE does not support DNS cache clear triggered by 5GC, the application will continue to be served by the old EAS until the UE Application Layer DNS cache timer expires or the applications determines to flush the DNS cache.

NOTE 3:	This DNS cache clear indication does not impact the UE Application Layer DNS caching.

-	For SSC mode 2/3 case, the UE remove the cached DNS records associated with the released PDU Session and can reselect a new EAS after it is allocated with a new IP address.

/*******************End of Change****************/