3GPP TSG-WG SA2 Meeting #142E e-meeting	S2-2008836

Elbonia, November 16 – 20, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#3, Sol#35: Clarification on authorization information provided to NG-RAN

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract: This contribution proposes the clarification on authorization information provided to NG-RAN.

# 1. Introduction/Discussion

This paper clarifies that the authorization information provided to NG-RAN only applies to Layer 2 Relay case.

For Layer 2 Relay case, NG-RAN needs to be aware of the UE being UE-to-Network Relay UE or Remote UE.

For Layer 3 Relay case, NG-RAN does not need to be aware of the UE being UE-to-Network Relay UE or Remote UE.

Thus, for all the related procedures, authorization information provided to NG-RAN only happens in case of Layer 2 Relay.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.752.

# * * * * First change * * * *

#### 6.35.2.2	Procedure Enhancement for providing relay authorization information to NG-RAN

The Registration procedure for UE is performed as defined in TS 23.502 [8] with the following additions:

-	The AMF determines whether the UE is authorized to be a 5G ProSe UE-to-Network Relay based on UE's 5G ProSe capability and the subscription data related to relay service authorization from the UDM. In the case of Layer 2 Relay, the AMF indicates to NG-RAN that the UE is authorized to be a 5G ProSe UE-to-Network Relay.

-	In case of Layer 2 Relay, the AMF also determines whether the UE is authorized to be a Remote UE accessing via a Layer 2 UE-to-Network Relay based on the UE's 5G ProSe capability and the subscription data related to relay service authorization information from the UDM. The AMF indicates to NG-RAN that the UE is authorized to be a 5G ProSe Remote UE.

In the case of Layer 2 Relay, the Service Request procedure for UE in CM-IDLE state is performed as defined in TS 23.502 [8] with the following additions:

-	The AMF indicates NG-RAN that the UE is authorized to be a 5G ProSe UE-to-Network Relay.

-	The AMF indicates NG-RAN that the UE is authorized to be a 5G ProSe Remote UE.

In the case of Layer 2 Relay, the handover procedure for UE is performed as defined in TS 23.502 [8] with the following additions:

-	The source NG-RAN indicates target NG-RAN that the UE is authorized to be a 5G ProSe UE-to-Network Relay.

-	The source NG-RAN indicates target NG-RAN that the UE is authorized to be a 5G ProSe Remote UE.

# * * * * End of changes * * * *