3GPP TSG-WG SA2 Meeting #142E e-meeting	S2-2008920

Elbonia, November 16 – 20, 2020

Source:	Samsung

Title:	KI#7 conclusion update

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This contribution proposes an update to the conclusion of KI#7.

# Discussion

This contribution proposes an update to the conclusion of KI#7.

# Proposal

It is proposed to add the following changes into TR 23.757.

******************************** START OF CHANGES *********************************

## 8.6	Key Issue #7: Reliable delivery method switching between unicast and multicast

For delivery method switching due to mobility, the following principle are agreed,

-	When the UE moves from a NG-RAN node that supports 5MBS to a RAN node that does not support 5MBS, the network and UE shall support switch from 5GC Shared MBS traffic delivery method to 5GC Individual MBS traffic delivery method.

-	When the UE moves from a RAN node that does not support 5MBS to a NG-RAN node that supports 5MBS, the network and UE shall support switch from 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

Editor's note:	It is FFS before the handover procedure, whether the associated PDU Session needs to be activated and the UE and NG-RAN need to be aware of the linkage between the MBS Session and the associated PDU Session before mobility. If yes, 5GC needs to provide this information to UE and NG-RAN.

-	During the handover from RAN not supporting 5MBS to NG-RAN supporting 5MBS, PDU sessions, including the one associated with MBS session, are handed over to target RAN. After the handover, the MBSF and MB-SMF are notified and the 5GC delivery method switch is triggered from the 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

-	For the handover from NG-RAN supporting 5MBS to RAN not supporting 5MBS, the MBS Session flow is converted to the QoS flow within the associated PDU session at the target RAN.

Editor's note:	It is FFS whether the support for lossless handover with data forwarding from source NG-RAN to the target NG-RAN is needed, which needs confirmation by RAN.

For delivery method switching not due to mobility, the following principle are agreed,

-	Switching between PTP and PTM delivery methods for 5GC Shared MBS traffic delivery shall be supported. NG-RAN is the decision point for of switching the PTP and PTM delivery methods.

-	If the NG-RAN node support 5MBS, the network shall use the 5GC Shared MBS traffic delivery method for MBS Session packet transfer.

NOTE:	When a UE is handed over from an NG-RAN node not supporting 5MBS, Individual delivery may still be used.

Switching between multicast delivery and unicast delivery using individual UE and application server addresses is not specified in normative work.

******************************** END OF CHANGES **************************************