SA WG2 Meeting #142E	S2-2008977

16 November - 20 November 2020, Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#11, Sol.#72 update and conclusions

Document for:	Discussion/Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Adding conclusion related to sol#72, plus few clarifications on sol#72.

# 1		Discussion

Solution #72 update:

- Updates step 3 to explicitly mention that NF1 is triggered to transfer the analytics.

- Update to wording in Impacts clause

Proposed conclusion with principle related to extending existing UE Context transfer services with UE Analytics information.

# 2	Proposal

It is proposed to modify TR 23.700-91 as follow.

*** Change 1 ***

## 6.72	Solution #72: Efficient Analytics Transfer in mixed Deployments

### 6.72.1	Description

This solution addresses aspects of Key Issue #11: increasing efficiency of data collection.

In a mixed and multiple NWDAF deployment scenario where some NWDAFs are also co-located with 5GC Network Functions such as AMF and SMF, it is beneficial to share analytics data performed in one of the co-located NWDAF with other NFs, so that the latter are relieved from contacting NWDAF again for collecting data and, if needed, further computing analytics. Also, each NF interested in analytics for a UE or a group of UEs requests analytics from NWDAF specifically for its uses. However, once analytics for UE(s) are available to an NF consumer (e.g. AMF), it would be useful that these are made common to other entities, thus avoiding that several 5GC NFs request analytics to one or several NWDAFs for the same UE or the same group of UEs.

For example, if a co-located NWDAF or an NF Consumer has a UE mobility related analytics data either generated by itself or received using Nnwdaf services, it can be re-used by other NFs which are interested in the same UE mobility analytics data, e.g., those related to same PDU session and so on. With co-located NWDAF, and in particular for the scenarios when UE is moving, the analytics are lost for the UE as soon as the UE moves to a different AMF unless it is uploaded or sent to some other network function(s).

In cases where a co-located NWDAF or NF which has received analytics data using Nnwdaf services has analytics information (for example, UE mobility Analytics, UE Communication pattern Analytics specific to a UE or a group of UEs), then this Analytics ID and / or analytics information is shared along with UE Context information with other interested 5GC NFs. Those NF(s) which support receiving these analytics, e.g., along with UE Context information, may explicitly request these analytics when they initiate the UE related interaction with the NF that currently maintains these analytics. Therefore, a source NF can use this information before transferring the analytics to destination NFs.

### 6.72.2	Procedures

Figure 6.72.2-1 shows the procedure where a Network Function NF2 gets UE context transfer message from NF1 which additionally includes UE analytics information, e.g., mobility analytics or UE Communication pattern Analytics.

Figure 6.72.2-1: UE context transfer between 5GC Network Functions

1-2.	A Network Function NF1 (e.g. AMF or SMF) has available UE Analytics Info, e.g., UE Mobility analytics. This information could be based on Nwdaf_AnalyticsInfo Request response with NWDAF and/or NF1 could be co-located with the NWDAF.

3.	NF1 gets a trigger to UE Context transfer to another Network Function NF2, for example, this could be triggered as part of Mobility registration procedure or PDU session establishment procedure. Another example trigger is NF2 sending a UE context transfer request message to NF1.

4.	NF1 transfers UE Context information, and it additionally includes UE analytics information, e.g., mobility analytics, UE Communication pattern Analytics, and/or other relevant analytics data that it already received from NWDAF at step 2 or, if it is co-located with an NWDAF that it generated based on the received UE data. For example, in case NF1 and NF2 being AMFs, then this message would correspond to Namf_Communication_UEContextTransfer and in case of NF1 being AMF and NF2 being SMF, it would correspond to using the Nsmf_PDUSession_CreateSMContext message.

NOTE:	NF1 and NF2 may be the same type, e.g., when UE context is transferred between and "old" AMF and a "new" AMF. However, also the case of NF1 and NF2 being different NF types is possible.

### 6.72.3	Impacts on services, entities and interfaces

5GC NF with UE context information:

-	Extend UE Context transfer (e.g. as specified for AMF in clause 5.2.2.2.2 of TS 23.502 [3] and for SMF in clause 5.2.8.2.5 of TS 23.502 [3]) to support transferring UE analytics information along with the UE Context information.

*** Change 2 (all new text) ***

### 8.11.x	Signalling reduction via parametrization and services changes

The following principles are proposed for the normative work on "Signalling reduction via parametrization and services changes":

- Existing UE Context transfer services (e.g. as specified for AMF in clause 5.2.2.2.2 of TS 23.502 [3] and for SMF in clause 5.2.8.2.5 of TS 23.502 [3]) are extended to support transferring information related to UE Analytics along with the UE Context information.

NOTE:	The "information related to UE Analytics" comprises, e.g., UE mobility Analytics, UE Communication pattern Analytics, and/or other relevant analytics data available at the source NF.

Editor's Note: this list of principles is not complete.

*** End Change ***