SA WG2 Meeting #142E (e-meeting)	S2-2009089

# Nov 16 – Nov 24, 2020, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | DRAFT | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identification of satellite backhaul QoS characteristics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TNO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2020-09-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to Solution#11 ‘Backhaul QoS handling based on AMF and UPF information’ the 5GS is capable to determine the possible QoS limitations either by means of AMF or UPF. The identified limitations are provided to SMF and used by SMF in the process of PDU Session Establishment or PDU Session Modification procedure. RAN3 response to LS S2-2008308 states that RAN3 did not discuss the satellite backhaul during the NTN Study Item and that the backhauling is out of scope of the on-going Rel-17 RAN Work Item. Taking the RAN3 response into account this contribution proposes the fashion in which QoS properties (in particular Packet Delay Budget - PDB) of a satellite backhaul link are determined based on AMF and UPF information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Determining QoS properties (PDB) of satellite backhaul links based on AF and UPF information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Backhaul QoS handling faces difficulties. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x(New), 5.x.3(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.x	Support for integrating satellite access into 5GS

Editor's note:	this clause is intended as a placeholder for high level NR satellite access functionality.

### 5.x.1	General

This clause describes the support of satellite access in 5GS.

NR satellite access affects the system behaviour as follows:

-	NR satellite access is identified by the AMF which informs also other NFs of the CN RAT Types (NR(LEO), NR(MEO), NR(GEO), NR(OTHERSAT)

### 5.x.2	Support of RAT types defined in 5GC for satellite access

CN RAT Types "NR(LEO)", "NR(MEO)", "NR(GEO)" and "NR(OTHERSAT)" apply for satellite access to distinguish the different NR satellite access types in 5GC (see clause 5.4.X).

When a UE is accessing to the network via satellite access, the AMF determines the RAT type as specified in clause 5.4.X.

* * * First change * * *

### 5.x.3	Backhaul QoS handling based on AMF and UPF information

Making use of satellite backhauling can result in latencies that are larger compared to latencies experienced in terrestrial networks. Packet Delay Budget (PDB) is one of 5G NR QoS characteristics that defines an upper bound for the time that a packet may be delayed between the UE and the UPF that terminates the N6 interface. PDB in case of satellite backhauling can be determined by:

- AMF: either by configuration of the AMF or by exploiting information signalled between AN node and 5GC. In the latter case the AMF becomes aware of the capabilities of the 5G RAN during the N2 setup. The AMF is capable of converting the information acquired from the RAN (e.g. global RAN Node ID, global gNB ID) into a PDB value exploiting the values preconfigured in the 5GS. The extracted PDB value is further transferred to the SMF and used in the PDU session establishment/modification procedure.

- SMF may activate the end to end UL/DL packet delay measurement between UE and PSA UPF for a QoS Flow during the PDU Session Establishment or Modification procedure. Using this information the 5GS can estimate the PDB and exploit it in the selection of appropriate QoS Flow (5QI).

* * * End of changes * * *