3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2009170

Elbonia, November 16 – 20, 2020	(revision of S2-2008638r04)

Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, ITRI

Title:	KI #2: Solution #39 conclusion

Document for:	Discussion/Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract: This paper provides discussion and conclusion for solution #39

# Discussions

Solution #39 proposes seamless application relocation by 3GPP layer, i.e. UPF proxied L4 connection. This solution relies on defining two MPTCP subflows of one MPTCP session over two local PSA UPFs, (hence the MPTCP termination in the network is not unique) such as:

-  Two MPTCP subflows of one MPTCP session are established, one is between the UE and old local PSA, the other is between the UE and new local PSA. SMF triggers to establish second MPTCP subflow which implies network initiated JOIN. How this can be realized is described in RFC8684.

-  N4 is enhanced to support providing the MPTCP session context from the source PSA to the target PSA via the SMF.

-  When the EAS relocation is completed, the source PSA is indicated to trigger path switching by using the MPTCP mechanism, which makes no impact on the UE.

This solution introduces new mechanism, some of them relying on RFCs, that might require further clarifications:

- It introduces new functionality referred to as “Virtual MPTCP host” which has MPTCP proxy functionality distributed in two PSA UPFs. The MPTCP functionality required to make the solution work is described in RFC8684. The, logic in this virtual host that describes the handling of the MPTCP session context, e.g., transferring MPTCP context in PSA1 to PSA2, and any need to synchronise the TCP states on the upstream connection between the moving MP TCP proxy and EAS, might require further description.

- The proposed MPTCP context transfer between source PSA and target PSA would involve transfer of the keying material which may have a security risk e.g. sharing session keys should generally be discouraged, might need to be assessed by SA3

- the solution has some aspects of ATSSS, but many other aspects are different, as ATSSS is about an unique MP-TCP termination in one UPF in the network.

- the solution would not work for either UE(s) or EAS not supporting MPTCP and would not work for non TCP traffic (QUIC)

# Proposal

It is proposed to include the following in the TR 23.748.

* * * * Start of Change * * * *

### 7.1.6	Evaluation for Key Issue #2: Solution #39

Solution #39 is a network-based solution and UE application layer is agnostic with EAS relocation. It does not dependent on any application layer mechanism.

The solution relies on MPTCP proxy on UPF and requests relocation of MPTCP contexts when UPF relocation happens. There are concerns raised on whether the MPTCP context relocation is feasible or not, and potential issues on how to make the relocation seamless. It was concluded Solution #39 is not recommended in normative work in this study.

9. 	Conclusions

9.x	Conclusions for Solution #39

Solution #39 is not recommended in normative work in this study.

* * * * End of Change * * * *