SA WG2 Meeting #142E	S2-2009361

16 November - 20 November 2020, Elbonia	(revision of S2-2008978r1)

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#2, Updates to evaluation clause 7.2.5 and conclusion clause 8.2.2

Document for:	Discussion/Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Update evaluation in clause 7.2.5 to align with sol.59 updates. Resolve the Editor's Notes in clause 8.2.2.

# 1		Discussion

The following changes are proposed on clause 7.2.5:

- Update the evaluation of sol.#59 according to the updates proposed for solution #59.

# 2	Proposal

It is proposed to modify TR 23.700-91 as follow:

*** Change 1 ***

Table 7.2.2-3 lists solutions that concern with the re-selection of NWDAFs, e.g., in case of mobility. Those solutions assume a setup with multiple NWDAF instances, therefore are marked as related to Key Issue #2, while the solutions focus on NWDAF re-selection and context handover aspects, and therefore can be applied to any kind of NWDAF architecture. Main differences are in the services used for the re-selection and in which entity is triggering the re-selection and informing the service consumer.

Table 7.2.2-3: Solutions concerning re-selection of NWDAF

| Sol. | Aspect covered | Impact to NWDAF | Impact to other NFs | Additional notes |
| --- | --- | --- | --- | --- |
| 10 | Without DCCF: Re-selection of NWDAF triggered by AMF handover (clause 6.10.2.4). With DCCF: Re-selection of NWDAF is using knowledge in DCCF. | Re-use request/response service operations to transfer UE related statistics | Without DCCF: Extend AMF services to inform target AMF about the NWDAF ID used by the source AMF for UE related Analytics. Target AMF determines whether to keep source NWDAF instance or select new target NWDAF instance. With DCCF: Extend to inform DCCF on what UE related Analytics ID is used in which NWDAF. Utilize knowledge DCCF has when switching NWDAFs. | Without DCCF: Source AMF (as NWDAF service consumer) determines and triggers NWDAF re-selection. Target NWDAF requests UE related statistics from source NWDAF. With DCCF: DCCF has knowledge on what source NWDAF has been used for UE related Analytics IDs. |
| 26 | Re-selection of NWDAF. | New service to: - transfer subscription data - transfer analytics data and collected data | NWDAF service consumer: support subscription renewal with new NWDAF instance. New parameter for Notify message indicating reselection of NWDAF instance. | Source NWDAF determines and triggers NWDAF re-location. Source NWDAF informs the NWDAF service consumer about the new NWDAF ID that will serve the consumer. |
| 57 | Re-selection of NWDAF in case of UE mobility. | New service to: - transfer subscription data - transfer analytics data and collected data | NWDAF service consumer: support subscription renewal with new NWDAF instance. | Source NWDAF triggers the re-location. Target NWDAF informs the NWDAF service consumer about the handover. |
| 59 | Re-selection of NWDAF based on trigger or for handover preparation. | Extend subscription service to: - inform other NWDAF about analytics handover or prepared/cancelled handover Extend AnalyticsInfo service to: - transfer subscription data - transfer analytics data and collected data |  | Source NWDAF determines and triggers the analytics handover or the prepared analytics handover. Target NWDAF informs the NWDAF service consumer about the completed handover. |

*** Change 2 ***

### 7.2.5	Reselection of NWDAF

Solution #10, #26, #57 and #59 address the reselection aspect of KI #2 in the multiple NWDAF deployment scenario:

-	Solution #10, clause 6.10.2.4: Handling of mixed and distributed NWDAF deployments.

-	Solution #26: Reselection of NWDAF.

-	Solution #57: Reselection of NWDAF due to mobility change.

-	Solution #59: Analytics handover.

Clause 6.10.2.4 in Solution #10 defines a set of procedures to support change of the serving NWDAF when the change of NWDAF is triggered by NF change. Whether a new NWDAF is required or the old serving NWDAF can be reused is decided by a target NF. When a new NWDAF is selected by a new target NF, the new selected NWDAF may retrieve analytics context information (e.g., analytics data and collected input data) from the old serving NWDAF using Nnwdaf_AnalyticsInfo service operations. The solution without DCCF also defines a procedure that supports updating NWDAF service consumers with information of a new NWDAF instance by extending Namf_Communication_UEContextTransfer. The new (target) serving NWDAF notifies the NWDAF service consumer about the relocation.

Solution #57 defines a set of procedures to support change of the serving NWDAF instance when the change of NWDAF instance is triggered by the mobility of the UE. The serving NWDAF is informed of the UE mobility event by the old (source) AMF or a new (target) AMF, and decides the NWDAF reselection. The serving NWDAF may send analytics context information (e.g., stored analytics data and collected input data) to the new NWADF during the reselection procedure using a new Nnwdaf_AnalyticsRelocation service. The new (target) serving NWDAF notifies the NWDAF service consumer about the reselection.

Solution #26 defines a procedure to support change of the serving NWDAF instance when the change of NWDAF instance is triggered by the old serving NWDAF. The old serving NWDAF may select a new NWDAF instance based on NF profile obtained from the NRF and the new serving NWDAF informs the consumer NF after the reselection is finished. The new selected NWDAF may retrieve analytics context information (e.g., analytics data and collected input data) from the old serving NWDAF using a new Nnwdaf_AnalyticsInfo service.

Solution #59 defines a set of procedures to support change of the serving NWDAF when the change of NWDAF is triggered by the old serving NWDAF, as like in Solution #26. The old serving NWDAF may select a new NWDAF instance based on NF profile received from the NRF. In the procedure, the old serving NWDAF inform the target NWDAF about the analytics handover, indicating to it/them the availability of analytics context information (e.g., collected historic data and stored analytics) using an extended Nnwdaf_AnalyticsSubscription service operation. The solution also defines the option that the old serving NWDAF sends a prepared analytics handover request to the potential candidate NWDAFs and selecting a final target NWDAF when the actual handover is triggered. The target NWDAF(s), when informed about the availability of analytics context information (e.g., collected historic data and stored analytics) available at the source NWDAF, will retrieve this information from the source NWDAF using Nnwdaf_AnalyticsInfo service operations. Finally, the new (target) serving NWDAF notifies the NWDAF service consumer about the successful reselection.

All solutions have in common that analytics data and input data collected by the old serving NWDAF can be transferred from the old serving NWDAF to the new target NWDAF. Solution #10 and #57 are handling the scenario that the serving NWDAF is changed due to change of the consumer NF which again is triggered by UE mobility. Solution #26 is handling the scenarios that the serving NWDAF is changed by the reselection of NWDAF instances to support, e.g., offloading, load balancing and network resource configuration. Solution #59 is addressing both cases. In solutions #10, #26, #57, #59 the NWDAF service consumer is informed by the target NWDAF, which is inline with AMF relocation procedures as specified in Release-16.

*** End Change ***