SA WG2 Meeting #142E e-meeting	S2-2009362

November 16 – 20, 2020	(revision of S2-200xxxx)

Source:	Ericsson, Samsung, NTT DOCOMO, Nokia

Title:	KI#2 – Solving ENs in 8.2 Conclusion on reselection of NWDAF plus some additions

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes how to solve ENs in conclusion for KI#2.

# 1 Discussion

In last meeting two ENs were introduced in the conclusion on reselection of NWDAF. This contribution proposes how to solve these.

First EN states:

Editor’s Note: It is still to be decided if information shall be pushed via Nnwdaf_AnalyticsSubscription_Subscribe request message or re-use existing Rel-16 meachanism via a pull from target NWDAF

There are proposed some scenarios when a reselection of NWDAF is possible.

- At UE-mobility either caused by:

A1) Change of Analytics consuming NF

A2) UE moves into an area not part of source NWDAF’s serving area (e.g. when Analytics consumer is OAM, AF)

- Scenarios including e.g. load balancing assumed not done inside NF sets or not perfectly utilization of NRF prioritization list or when an NWDAF will soon initiate a procedure that requires UEs to be re-allocated to a new NWDAF

The information sent might be rather large compared to what usual is sent in signalling procedures. Therefore, it is deemed that no attempt to perform signalling reduction need to be applied and it is also deemed better to strive for re-using existing services than introduce new. Also better to follow conclusion on KI#11 on how to send information.

For A1 it was agreed in last meeting to send information (Data and/or Analytics) in a response message to a pull request from target NWDAF using Rel-16 existing services or in KI#11 defined services for data collection.

When AMF change happens information may be sent in extended existing messages. Further when a DCCF is deployed it may be used to support the historical data transfer at reselection of NWDAF when change of Analytics consumer change happens. This is described in clause 6.10.2.4 in solution#10.

For A2 it is proposed to let the source NWDAF use NRF to find a suitable NWDAF that can take over and start serving the UE. This is preferably done by introducing an indication in an Nnwdaf_Analytics message (imitating the trigger of an NWDAF relocation message as stated in solution#57). If needed the source NWDAF can indicate for how long it may store the data for the UE, to letting the target NWDAF now how fast it must act. Or this is an operator pre-defined time. Then the target NWDAF initiate the collection of information when it has time to do so. And it will re-use the agreed pull message for A1 scenarios to do so.

Scenarios in B) are also suitable to be done in NF sets or make use of existing mechanisms described in 4.26 in TS 23.502. Therefore, it is deemed that these scenario uses similar mechanisms as described above for group A), or they make use of existing mechanisms such as NF sets and context transfer mechanisms.

The second EN states:

Editor’s Note: Bullet 2 above depends on decisions on if and how Data collection from NWDAF shall be done.

And the second bullet, which the EN refers to is:

- Historical data available at the source NWDAF and related to the Analytics to be handed over to the target NWDAF (including not only the raw data collected from the data source(s), but also "analytics metadata" such as the time period of the collected data, information about the data source(s), etc.).

It is proposed, as also stated for the first EN above, to further discuss the service used for data collection in the conclusion for KI#11.Since the information retrieved from the source NWDAF, here named Analytics context includes historical data the service used for Analytics context transfer is removed in this conclusion. The service used for Analytics transfer is still Nnwdaf_AnalyticsInfo_Request response service, according to the reasoning for the first EN above. New addition of the Analytics context (e.g. Information related to entities that are subscribed to receive analytics) is to be added to this service.

If the NWDAF or even DRF (where source NWDAF may have stored its data for target NWDAF to collect according to KI#11) are distributed purging of UE related data in combination with or shortly after a UE has stopped being served by the source NWDAF is a good principle. Whether long term storage is performed in distributed DRF or if central DRF collects data (directly from NF or distributed NWDAF) to be stored long term is an operator specific principle, but both are deemed possible.

## 2 Text Proposal

It is proposed the following changes to TR 23.700-91.

# * * * * Changes * * * *

### 8.2.2	Re-selection of NWDAF (category #2)

The following principles are proposed for the normative work on "Re-selection of NWDAF":

a)	Extend or re-use existing Release-16 service operations, i.e. do not introduce new AnalyticsRelocation service.

b)	Support different triggers conditions for the NWDAF re-selection corresponding to UE mobility, load balancing, etc.

c)	Support the handover of analytics subscriptions between NWDAFs.

In case NWDAF re-selection is initiated by the source NWDAF, the source NWDAF will make use of NRF to find candidate NWDAF(s) and the request(s) for handover of analytics subscriptions in a Nnwdaf_Analytics message to the selected target NWDAF(s). The request(s) include information related to the subscriptions to be handed over, including for each subscription at least the subscribed entity, the Analytics ID and corresponding parameters (as defined in clause 6.1.3 of TS 23.288 [x]), as well as an information about additional analytics context available at the source NWDAF (see bullet e)).

d)	Support the transfer of analytics context:

-	NF consumer was not changed, i.e. NWDAF re-selection was initiated by the source NWDAF: the target NWDAF initiates the collection of information related to the subscriptions handed over.

-	NF consumer is changed, i.e. NWDAF re-selection is initiated by the target consumer NF: the analytics context is requested by the target NWDAF from the source NWDAF. When AMF change happens, information (such as source NWDAF ID) can be sent in extended existing message Namf_Communication_UEContextTransfer. Further, when a DCCF is deployed it can be used to support the context transfer at reselection of NWDAF when change of a consumer NF happens.

e)	Analytics context can include the following information parts:

-	Analytics related:

Information related to entities that are subscribed to receive analytics.

Pending Analytics (i.e. not yet notified to the consumer) or historical Analytics Output information - if available at the source NWDAF.

-	Historical data:

The target NWDAF may retrieve historical data that is available at the source NWDAF and related to the Analytics to be handed over to the target NWDAF (including not only the raw data collected from the data source(s), but also "analytics metadata" such as the time period of the collected data, information about the data source(s), etc.).

NOTE 1: If DRF and DCCF are deployed, the target NWDAF can retrieve historical data from the DRF either directly or via a request to DCCF.

f)	If no change of Analytics consumer is performed, the target NWDAF notifies the NWDAF service consumer about the re-location. If change of Analytics consumer is performed no notification is needed.

g)	Support the prepared analytics handover to inform target NWDAF(s) about a potential analytics handover. This message is to check the acceptability of the analytics handover at the target NWDAF(s) and allows the target NWDAF(s) to prepare to take over those Analytics subscriptions from the source NWDAF by requesting the related analytics context from the source NWDAF, creating subscriptions with the required data sources, etc. In case the source NWDAF has contacted multiple target NWDAF(s) about such prepared analytics handover, the source NWDAF has to confirm the handover to the selected target NWDAF and cancel the prepared analytics handover to the other candidate NWDAF(s).

h) Purging of UE related data in combination with or shortly after a UE has stopped being served by the source NWDAF is a good principle that shall be possible.

# * * * * End changes * * * *