SA WG2 Meeting #142E	S2-2009384

16 November - 20 November 2020, Elbonia	(revision of S2-2008980r1)

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#15, Sol.#76 update

Document for:	Discussion/Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Updates the solution to explicitly show the option where URDCF is hosted by DCCF.

# 1		Discussion

Based on discussion and feedback received in SA2#141E, the following changes are proposed for solution #76:

- Correct name of URDCF to " User consent Related Data Coordination Functionality"

- Add/update procedures to explicitly show the second alternative described in the Introduction, i.e. URDCF hosted by DCCF.

- Delete the Editor's Note related to coordination with SA WG3, as this coordination is captured in the proposed LS to SA WG3.

# 2	Proposal

It is proposed to modify TR 23.700-91 as follow.

*** Change 1 ***

## 6.76	Solution #76: Revocation of user consent

### 6.76.1	Introduction

This solution addresses KI #15: User consent for UE data collection/analysis.

User consent may be required to process certain UE related data. User consent may be collected and managed by the network operator and can be stored as part of the subscription data in UDR. User consent can also be updated or revoked at any time.

The proposed solution introduces a "User consent Related Data Coordination Functionality" (URDCF) that is tracking the distribution and usage of any user related information falling under the scope of user consent. If the user consent is updated/revoked, the URDCF will take care to inform all entities having received a copy of the UE/user related data and UE related Output Analytics about the updated or revoked user consent and request them to delete the data if the data is no longer covered by the user consent.

The URDCF logs Nnwdaf_AnalyticsInfo_Requests and Nnwdaf_AnalyticsSubscription_Subscribe Request messages. If the URDCF becomes aware of a revocation of user consent, the URDCF will check its logs to identify past Data Consumers affected by this revocation. The URDCF will then inform the identified Data Consumers about the revocation of the user consent and will request them to delete the user related data as it is no longer covered by a user consent.

The URDCF may be hosted by the NWDAF. Alternatively, the URDCF may be hosted by a Data Collection Coordination Function (DCCF) or Data Collection NF (DCNF) as described e.g. in solutions #9, #15, and #35 of the present document. The following procedures show the URDCF as being hosted by the NWDAF (alternative 1) or by the DCCF (alternative 2).

### 6.76.2	Procedures

#### 6.76.2.1	Alternative 1: Data Request procedure with URDCF hosted by NWDAF

Figure 6.76.2.1-1 shows the Data Request procedure with URDCF hosted by NWDAF.

Figure 6.76.2.1-1: Data request procedure with URDCF hosted by NWDAF

1.	A Data Consumer / Analytics Consumer (e.g. NWDAF or NWDAF service consumer) sends a Data / Analytics Request or Data / Analytics subscription (e.g. Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe Request) to the NWDAF with URDCF.

2.	If the requested Data is subject to user consent, the NWDAF with URDCF needs to check whether user consent is available for this request. The NWDAF with URDCF either already knows about the user consent (e.g. from a previous request) or needs to check with the UDM to learn about the user consent information available in the UDR subscription information. The NWDAF with URDCF will further subscribe to UDM to receive notifications for the UE or group of UEs related to changes on the user consent information.

3.	If the user consent is available and valid, the NWDAF sends Data collection requests / Event subscriptions to one or multiple Data Source(s), e.g. AF(s), NF(s), other NWDAF(s), depending on the Input Data required for the requested Analytics ID.

4.	Data Source(s) provide the requested Data / Events to the NWDAF (e.g. as Data response or as notifications related to the requested subscription). The response / notify messages may also include a validity time indicating the duration over which the Data may be stored / processed. At the expiry of this validity time, any Data Consumer (including the NWDAF) needs to either delete corresponding Data / Analytics, or needs to renegotiate a new validity time with the Data Source.

5.	If the collected Data relates to UE related Data, where user consent had not yet been checked in step 2, the NWDAF with URDCF needs to check whether user consent is available for this request. The NWDAF either already knows about the user consent (e.g. from a previous request) or needs to check with the UDM to learn about the user consent information available in the UDR subscription information.

6.	If the user consent is available, the NWDAF with URDCF keeps track of Data / Analytics Consumers that the NWDAF is providing UE related Data requiring user consent. Therefore, the NWDAF with URDCF will keep a record of the Data Consumer along with information to identify and contact the Data Consumer, e.g. by a callback URI provided by the Data Consumer at step 1.

7.	If the user consent is available and valid, the Data Source provides the requested Data to the Data Consumer (e.g. as Data response / Nnwdaf_AnalyticsInfo_Request response or as notifications related to the requested subscription). The response / notify message may also include a validity time indicating the duration over which the user consent is valid. At the expiry of this validity time, any Data Consumer needs to either delete corresponding Data / Analytics, or needs to renegotiate a new validity time.

#### 6.76.2.2	Alternative 2: Data Request procedure with URDCF hosted by DCCF

Figure 6.76.2.2-1 shows the Data Request procedure with URDCF hosted by DCCF.

Figure 6.76.2.2-1: Data request procedure with URDCF hosted by DCCF

1.	A Data Consumer / Analytics Consumer (e.g. NWDAF or NWDAF service consumer) sends a Data / Analytics Request or Data / Analytics subscription (e.g. Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe Request) to the Data Source via the DCCF with URDCF.

2.	If the requested Data is subject to user consent, the DCCF with URDCF checks whether user consent is available for this request. The DCCF with URDCF either already knows about the user consent (e.g. from a previous request) or checks with the UDM to learn about the user consent information available in the UDR subscription information. The DCCF with URDCF will further subscribe to UDM to receive notifications for the UE or group of UEs related to changes on the user consent information.

3.	If the user consent is available, the DCCF with URDCF keeps track of the Data / Analytics Consumers that requested the UE related Data requiring user consent. Therefore, the DCCF with URDCF will keep a record of the Data Consumer along with information to identify and contact the Data Consumer, e.g. by a callback URI provided by the Data Consumer at step 1.

4.	If user consent for the requested data / analytics is available and valid, the DCCF sends the data collection requests / event subscriptions to a Data Source (e.g. AF, NF, OAM, other NWDAF) that can provide the data / analytics as requested in step 1.

5.	The Data Source provides the requested Data / Analytics / Events to the Data Consumer / Analytics Consumer (e.g. as Data response or as notifications related to the requested subscription). The response / notify messages may also include a validity time indicating the duration over which the Data may be stored / processed. At the expiry of this validity time, any Data Consumer needs to either delete corresponding Data / Analytics, or needs to renegotiate a new validity time with the Data Source.

#### 6.76.2.3	User consent revocation procedure

Figure 6.76.2.3-1 shows the procedure related to user consent revocation / withdrawal for both alternative URDCF hosted by NWDAF (alternative 1) or URDCF hosted by DCCF (alternative 2).

Figure 6.76.2.3-1: User consent revocation procedure

1.	NWDAF with URDCF (alternative 1) or DCCF with URDCF (alternative 2) receives Data / Analytic requests from the Data / Analytic Consumer. URDCF checks the user consent for the requested/collected UE related Data. URDCF registers with UDM to be notified about changes on the subscription information related to user consent of the affected UE(s). URDCF keeps track of Data / Analytics Consumers that requested/received UE related Data requiring user consent. Step 1 is following the procedure as described in clause 6.76.2.1 (alternative 1) or clause 6.7.2.2 (alternative 2) of the present document.

2.	UDM determines that the user consent for a particular user has been revoked.

3.	UDM notifies to the URDCF about the user consent revocation.

4.	Based on its historical records of Data / Analytics Consumers and the Data provided to them, the URDCF will determine all Data Consumers to be notified as they had requested some Data that is affected by the withdrawn user consent.

5.	URDCF deletes any UE Data and/or Analytics related to the user that has revoked his consent. The URDCF will also terminate any related subscriptions with the Data Source(s) related to those Analytics.

6.	URDCF requests / notifies to all Data Consumers determined in step 5 to delete any UE Data and/or Analytics related to the user that has revoked his consent.

7.	Optionally, Data Consumers confirm to the URDCF that the UE related Data / Analytics have been deleted.

### 6.76.3	Impacts on services, entities and interfaces

NWDAF:

-	For alternative 1: Hosts functionality (URDCF) to keep track of Data requests / subscriptions and to notify Data Consumers about withdrawn user consent.

DCCF:

-	For alternative 2: Hosts functionality (URDCF) to keep track of Data requests / subscriptions and to notify Data Consumers about withdrawn user consent.

Data Consumer (e.g. NWDAF, NF):

-	Delete user data if user consent validity time has expired or upon request from URDCF if user consent was withdrawn.

*** End Change ***