3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2102404

Elbonia, 12 – 16 April, 2021

Title:	[DRAFT] Reply LS on Group Subscriptions

Response to:	LS (S2-2102107/C4-211369) on Group Subscriptions from CT4

Release:	Rel-15 onwards

Work Item:	5GS_Ph1-CT

Source:	SA2

To:	CT4

Cc:

Contact Person:

Name:	Fenqin Zhu

E-mail Address:	zhu.fenqin@huawei.com

Attachments:

1. Overall Description:

SA2 thanks CT4 for their consideration on group subscriptions and would like to provide the answer to the question raised in their LS:

Q1:	Does a group subscription (with or without a maximum number of reports) created in UDM by NEF apply to group members that register to the network after the group subscription is created? 

Does a group subscription (with or without a maximum number of reports) created in AMF by UDM apply to group members that register or move to the AMF after the group subscription is created?

A1:	both answers are Yes.

Q2:	If the answer of Q1 is yes, how can the NEF determine that the reporting for the group is complete?

A2:	As defined in clause 4.15.3.2.3, the NEF uses the Maximum number of reports to determine the reporting is complete for a particular individual UEs and further uses the Number of UEs to determine the reporting for the group is complete.

Q3:	If the answer of Q1 is no, group members registered in the network and their serving AMFs are typically different when the group subscription is created and deleted. So is the requirement "the UDM unsubscribes the monitoring event(s) to all AMF(s) serving the UEs belonging to that group" in clause 4.15.3.2.3 correct?

A3:	See A1.

Q4:	During an inter-AMF mobility of a group member of a group subscription in S-AMF, does the group subscription that T-AMF creates, when there was no corresponding group subscription in T-AMF, only apply to the group member just moving from S-AMF, and not to other group members registered in T-AMF?

A4：	No. The group subscription is established and if the UE is the first group member in the T-AMF. Then the created group subscription applies to other group members registered in T-AMF later.

Q5:	During an inter-AMF mobility of a group member of a group subscription in S-AMF, should the group subscription be signalled to T-AMF only if the maximum number of reports for this UE was not already reached in S-AMF and the maximum duration did not already expire?

A5:	When the maximum duration already expired, the group-based subscription is cancelled locally in the all impacted NFs. So the group-based subscription is signalled to the T-AMF when the maximum duration does not expire. But the group subscription with the remaining number of reports “0” shall be transferred to the T-AMF when the maximum number of reports for this UE has been already reached in S-AMF because the T-AMF will consider the UE registers the network after the subscription and continue to report the event for this UE if it is not transferred,.

Q6:	If the group subscription expires in an AMF as specified in clause 4.15.1 of TS 23.502, any later group unsubscribe request from NEF/UDM as specified in clause 4.15.3.2.3 will be rejected by the AMF since the group subscription should no longer exist. Is this the intended behaviour?

A6:	NO. SA2 has updated the related descriptions in clause 4.15.1 of TS 23.502 in SA2#143e by S2-2101579. When the maximum duration expired, the group-based subscription is cancelled locally in the all impacted NFs. When the maximum number of reports is reached for all the UEs belong to the group, the NEF unsubscribes the monitoring event(s) to the UDM and the UDM unsubscribes the monitoring event(s) to all AMF(s) serving the UEs belonging to that group. As the NEF knows the duration, the NEF does not unsubscribe the monitoring event(s) to the UDM even the maximum number of reports is reached.

2. Actions:

To CT WG4

ACTION:

SA2 kindly asks CT4 to take the above answer into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #145E		May 17-28, 2021				Elbonia