3GPP TSG-WG SA2 Meeting #144E e-meeting  	S2-2102419

# Elbonia, April 12 – 16, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2756 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SNPN discovers the capable AUSF to communicate with AAA server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the TR 23700-07 SNPN can host a function supporting primary authentication and authorization of SNPN UEs that use credentials from the separate entity in R17. The separate entity could be AAA server only. In this case, the AMF in the serving SNPN needs to find the AUSF which is capable of discovering and selecting the AAA server towards a separate entity, which means, not all of AUSF are capable of selecting AAA server. So AMF in the serving SNPN needs to find a way to select the capable AMF. According to the existing mechanism and TR conclusion, - The AMF discovers and selects the AUSF as described in TS 23.501 [4] clause 6.3.4 using the Home Network Identifier and Routing Indicator present in the SUCI provided by the UE. - The AUSF (e.g. enhanced AUSF or new NF) in SNPN selects the AAA hosted by the external entity. The discovery and selection functionality (within the AUSF) uses the domain name corresponds to the realm part (specified in the NAI format for SUPI in clauses 28.15.2 and 28.16.2) within Network specific identifier that identifies the external entity to perform discovery and selection of the AAA hosted by the external entity. So the AMF uses the Home Network Identifier and routing indicator in SUPI to select the specific AUSF for the UE, the routing indicator should be used to identify the capable AUSF. The SUPI may be pre-configured or provided by remote provisioning for a UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In case of SNPN using credentials owned by AAA server, the AMF uses the routing indicator contained in SUPI to find the AUSF capable to communicate with AAA server |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing new features as per conclusion of TR 23.700.07 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.9, 5.30.2.9.2, 6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.30.2.9.2	Credentials Holder using AAA Server for primary authentication and authorization

The AUSF in SNPN may support primary authentication and authorization of UEs that use credentials from an AAA Server in a Credentials Holder (CH).

-	If the UDM instructs AUSF that primary authentication by a AAA server in a CH is required, the AUSF shall discover and select the AAA server, and then forward EAP messages to the AAA server which is acting as the EAP Server for the purpose of primary authentication.

NOTE:	The specific AUSF is capable of communicating with AAA server in a CH as described above, and the AMF discovers and selects the AUSF as described in clause 6.3.4 using the Home Network Identifier and Routing Indicator present in the SUCI provided by the UE and then AMF requests the authentication from the capable AUSF when the authentication is performed.

-	The SUPI is used to identify the UE during primary authentication and authorization towards the AAA sever. SUPI privacy is achieved according to existing methods in Annex I.5 of TS 33.501 [29].

-	After completing the primary authentication with the AAA server successfully, the AMF and SMF shall retrieve the UE subscription data from UDM using SUPI.

Figure 5.30.2.X.Y-1 depicts the 5G System architecture for SNPN with Credentials Holder using AAA Server for primary authentication and authorization.

Figure 5.30.2.X.Y-1: 5G System architecture with access to SNPN using credentials from Credentials Holder using AAA Server

Editor’s note:	 Whether an intermediate function is needed between the AUSF and the AAA-S is FFS.

Editor’s note:	 the Interface between AAA Server and SNPN is FFS.

# * * * * Second change * * * *

6.3.4	AUSF discovery and selection

In the case of NF consumer based discovery and selection, the following applies:

-	The AMF performs AUSF selection to allocate an AUSF Instance that performs authentication between the UE and 5G CN in the HPLMN. The AMF shall utilize the NRF to discover the AUSF instance(s) unless AUSF information is available by other means, e.g. locally configured on AMF. The AUSF selection function in the AMF selects an AUSF instance based on the available AUSF instances (obtained from the NRF or locally configured in the AMF).

-	The UDM shall utilize the NRF to discover the AUSF instance(s) unless AUSF information is available by other means, e.g. locally configured on UDM. The UDM selects an AUSF instance based on the available AUSF instance(s) obtained from the NRF or based on locally configured information, and information stored (by the UDM) from a previously successful authentication.

AUSF selection is applicable to both 3GPP access and non-3GPP access.

The AUSF selection function in AUSF NF consumers or in SCP should consider one of the following factors when available:

1.	Home Network Identifier (e.g., MNC and MCC, realm) of SUCI/SUPI (by an NF consumer in the Serving network) along with the selected NID (provided by the NG-RAN) in the case of SNPN and Routing Indicator.

NOTE 1:	The UE provides the Routing Indicator to the AMF as part of the SUCI as defined in TS 23.003 [19] during initial registration. The AMF can provide the UE's Routing Indicator to other AMFs as described in TS 23.502 [3]. The Routing Indicator can be used to identify the AUSF which is capable of discovering and selecting the AAA server of Credentials Holder for primary authentication and authorization as defined in clause 5.30.2.9.2.

NOTE 2:	In the case of SNPN and if the UE provides IMSI as SUPI/SUCI to the AMF and if SUCI/SUPI is for an SNPN served by the AMF, the AMF uses the selected NID provided by the NG-RAN together with the selected PLMN ID (from SUCI/SUPI) as the SUCI/SUPI does not always include the NID. In the case of SNPN and the UE provides NSI as SUPI/SUCI to the AMF, the AMF uses the Home Network Identifier for selection of AUSF.

When the UE's Routing Indicator is set to its default value as defined in TS 23.003 [19], the AUSF NF consumer can select any AUSF instance within the home network for the UE.

2.	AUSF Group ID the UE's SUPI belongs to.

NOTE 3:	The AMF can infer the AUSF Group ID the UE's SUPI belongs to, based on the results of AUSF discovery procedures with NRF. The AMF provides the AUSF Group ID the SUPI belongs to other AMFs as described in TS 23.502 [3].

3.	SUPI; e.g. the AMF selects an AUSF instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI as input for AUSF discovery.

In the case of delegated discovery and selection in SCP, the AUSF NF consumer shall send all available factors to the SCP.

# * * * * End of changes * * * *