3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102462

Elbonia, Apr 12 – Apr 16, 2021	(revision of S2-210XXXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0286 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing EN on bulked data definitions and procedures. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Removal of FFSs Clause 6.2.6.0: Editor's note: The description of the proper information to obtain data collection shall be aligned with the enhancements for discovering and tracking of entities. Clause 6.2.6.1: Editor's note: Definition or "pre-process" data is FFS. Editor’s note: It is FFS the aligment of the description of the service parameters in Clause 6.2.S.1 with the procedures description in Clause 6.2.S.2, and the Nnwdaf_DataManagement service description in Clause 7.X. Clause 6.2.6.2: Editor's note: The discovery of the appropriated NWDAF providing the data collection service and the type of data exposed by the NWDAF shall be aligned with the extensions of the NWDAF discovery description. Editor’s Note: The description of the input parameters of Nnwdaf_DataManagement_Subscribe services when the required data is a list of events has to be aligned with description in Clause 5A4. Editor's note: Merge of the two sets of parameters above is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding text in 6.2.6.0 to align the description with the discovery of data sources done in 6.2.2.1, and Clause 6.2.6.3.6, therefore removing the FFS in 6.2.6.0. Definition of the term pre-processed data and removal of associated FFSs in Clause 6.2.6.1. Changing the text of Clause 6.2.6.1 to make clear that both Event IDs and Analytics ID can be exposed as bulked data, and alignment of the definitions of Data Specification for both cases. This enables to remove the FFS in Step 2a in Clause 6.2.6.2, therefore simplifying the description of the parameters used when subscribing to data from NWDAF with Nnwdaf_DataManagement_Subscribe. Updates in the parameters of Nnwdaf_DataManagement_XXX described in Clause 6.2.6.1 to provide bulked data, so that parameters described in the procedure in Clause 6.2.6.2 and Clause 7.4.x are also reflected in Clause 6.2.6.1. This also enables to remove the FFS in Clause 6.2.6.1 on alignment of description among 6.2.6.1, 6.2.6.2, 7.4.x. Add the new field “Recursive Event Clubbing” to align the Format&Processing definitions with the “Muting Event Mechanisms” and put back (in a reworded way) one of the parameters “DC Mode offline” removed during SA2#143E. This change also support the closing of FFS in Clause 6.2.6.0. Update text of procedure in Clause 6.2.6.2 including a note in Step 1 to align the discovery of NWDAF with data exposure capability with our proposal of changes in the discovery clauses. Therefore removing the FFS in Step 1 of the procedure. Updates in the text of Step 2a of procedure in Clause 6.2.6.2 to keep a simple description for the parameters either for Event IDs or Analtyics IDs. Additionally the changes in the description of the input parameters for bulked data request in Clause 6.2.6.1 enable the alignment among the Formatting and Processing described in the bulked data with the ones in Clause 5A.4. Therefore, enabling the removal of the 2 FFSs in Step 2a. Updates in the text of steps 2b of procedure in Clause 6.2.6.2 to reflect the use of the fetch flag as described in Clause 6.2.6.1. Updates in the text of step 4a of procedure in Clause 6.2.6.2 to clarify how an NWDAF can discover the sources of data collection when the data is not available. Updates in the text of steps 6a and 6b, 7(a,b) of procedure in Clause 6.2.6.2 to align with description in Clause 6.2.6.1. Update of the Figure 6.2.6.2-1 to reflect the same parameters described in Clause 6.2.6.1. Updates in clauses 7.4.1, 7.4.2, 7.4.4, 7.4.5 to reflect the same list of parameters defined in Clause 6.2.6.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of bulked data services and data provisioning from NWDAF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.0, 6.2.6.1, 6.2.6.2, 7.4.1, 7.4.2, 7.4.4, 7.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.6.0	General

Data collection may be performed via DCCF, MFAF, and ADRF, when such NFs are deployed or via NWDAF (hosting DCCF and/or ADRF).

DCCF, MFAF, ADRF, and NWDAF hosting DCCF and/or ADRF shall use the same services listed in clause 6.2.1 from OAM, and NFs (including AFs directly or via NEF) to collect data. Additionally the new services for data exposure from DCCF, MFAF, ADRF, and NWDAF may also be used for data collection.

The NWDAF, DCCF, ADRF shall obtain the proper information to perform data collection for a UE, a group of UEs or any UE following the principles of clause 6.2.1.

When NWDAF or DCCF need to discovery the sources of data collection they follow the principles defined in Clause 6.2.2.1 in case of data collection from NFs; Clause 6.2.6.3.6 in case of DCCF deployed in the network and requiring data from NWDAFs or ARDFs; Clause 5.2 in case of NWDAF (hosting DCCFs) and collecting data from other NWDAFs (also hosting DCCF).

# * * * * Next change * * * *

6.2.6.1	Bulked Data Collection

NWDAFs or DCCF or MFAF or ADRF may provide bulked data to consumers as an alternative to providing individual events (i.e. subscription to multiple event IDs to obtain the data required for an analytics generation).

The bulked data is the set of data samples from the collected event notifications to be used for an analytics ID for a consumer of NWDAF, DCCF, MFAF or ADRF. A data sample may be a notification, in which case the bulked data may comprise a group of received notifications, or a data sample may be information extracted from a notification and processed, in which case the bulked data comprises the processed information. The bulked data can be used for the purpose of analytics inference or ML model training.

The bulked data is generated based on the set of data samples from event notifications collected data from NFs/OAM and the properties for the selection of such data Consumers of bulked data or operators may define different rules (e.g. aggregation formats or processes) for generation of bulked data for training or inference.

NFs capable to expose bulked data have the following capabilities:

-	Exposing runtime collected data (e.g. data from NFs/AFs/OAM retrieved via notification mechanisms), or historical collected data (e.g. data from NFs/AFs/OAM that were at some point collected, then stored), or both;

-	Appling selection processes of data samples or processing mechanisms for the generation of the bulked data according to formatting and processing rules provided by consumers of the bulked data or defined by an operator. Such rules define the allowed and/or restricted properties or processes to be applied to the set of data from the collected event notifications to be used for the bulked data.

-	Examples of properties are network slices; area of interest; statistical attributes such as averages, mean, skewness; aggregation levels, such as geographical per cell, per UEs, or temporal such as per hours or days.

-	Examples of processes are: definition of data collection mode mechanisms; definition if data to be used for composing the bulked data is directly extracted from collected events, or the data is extracted from events and pre-processed , or both; applying anonymization of data fields in the bulked data to avoid exposing undesired information.

NOTE:	Pre-process data from collected events refers to the usage of data manipulation processes in order to aggregate, contactenate, process data from multiple collected events that results in a single processed value. Examples of data manipulation are the generation of: average, maximum, minimum, skewed value from the collected events.

-	Having the mapping of the Service Operation that have to be used for collecting data of the bulked data associated with an analytics ID.

NWDAF, DCCF, MFAF, ADRF may expose service(s), respectively, the Nnwdaf_DataManagement_Subscribe or Ndccf_DataManagement_Subscribe, Nmfaf_DataManagement_Subscribe, Nadrf_DataManagement_Subscribe service operation with the following input parameters in order to request bulked data:

-	Data Specification:

-	Event ID(s) or Analytics ID(s);

-	In case of Event IDs, the Data Specification fields includes the fields Target of Event Reporting and Event Filter Information as defined in TS 23.502 [3] Clause 4.15.1, and Bulked Data Type parameter, which can be set to ''raw data samples'' (i.e., data is directly extracted from collected events) or ''pre-processed data samples'' (i.e., data from collected events is processed and the processed data is included in the bulked data) or a combination of both;

-	In case the Analtyics ID(s), the Data Specification fields contain:

-	Target of Reporting including a tuple with analytics ID; Bulked Data Type, which can be set to ''raw data samples'' (i.e., data is directly extracted from collected events) or ''pre-processed data samples'' (i.e., data from collected events is processed and the processed data is included in the bulked data) or a combination of both; analytics stage (inference or training);

-	Filter Information may include fields related to the analytics ID such as: Target of Analytics Information (e.g., any UE, list of UEs, groups of UEs); Analytics Filter Information (e.g., area of interest, DNN, Application, S-NSSAI). The analytics ID also determines the Service Operation from NFs, OAM to be used and type of data (i.e., Event IDs, OAM measurements) to be collected and associated with the bulked data;

-	Service Operation in case of Event ID, defines the service operation to be used by NWDAF, DCCF, MFAF, or ADRF to request data (e.g.: Namf_EventExposure_Subscribe or OAM Subscribe)

-	Formatting and Processing: the parameters defined in TS 23.502 [3] clause 4.15.1 for Event Reporting Information are also part of the possible formatting and processing instructions, additionally the following parameters may be included:

-	Periodic bulked data  notification-	Notifications are sent periodically (e.g. every hour) and may also in a specified time window (eg: 2AM to 3AM), irrespective of the number of notifications from a Data Source or amount of data that have been bulked. Applicable when the Fetch Flag=false.

-	Feature type to be applied to the data from the collected event notificication in case the bulked data type includes ''pre-processed data samples''. Examples of feature type are maximum values, average values, etc.

-	Time Window – Specifies the start and stop time for the requested data or analytics.

- 	If the Time Window includes a period in the past, then the data or analytics collection is “historical”.

-	If the Time Window includes a period in the future, the data or analytics collection is “runtime”.

-	(Optional) Minimum and/or maximum number of samples to be included in the bulked data.

-	Fetch flag, when set to false, it indicates that the bulked data needs to be generated and included in notification messages; when set to true, it indicates that bulked data will not be returned in the notification messages and the consumer must fetch the bulk data. The default value of fetch flag is false.

-	Bulked data deadline, which indicates the limit of time for the consumer to fetch bulked data after receiving a notification that the data is available. Applicable when the fetch flag is set to true.

-	Notification Event Clubbing as defined in Clause 5A.4, indicates the number of notifications from a data source to be bulked before sending a notification containing the bulked data to the Consumer. Applicable when the Fetch Flag=False.

-	Recursive Event Clubbing as defined in Clause 5A.4, indicates that when further data collection needs to be peformed by the NF generating the bulked data, the Notification Event Clubbing and Recursive Event Clubbing flags or Event Muting Mechanism defined in Clause 6.2.S shall be used when interacting with further data sources.

-	Processing rules: an aggregation level (e.g., per UEs, per AoI) per feature type; anonymization rules (e.g., anonymization of UE identifications).

-	A Notification Target Address (+ Notification Correlation ID), where the Notification Correlation ID is the unique identification for the bulked data being generated for the requesting consumer.

-	ADRF ID or NWDAF ID (or ADRF Set ID or NWDAF Set ID) storing historical data (optional). If know to the consumer, this may be specified to direct a DCCF or an NWDAF to the repository containing historical data.

-	(Optional) ADRF information indicating whether the collected data for the generation of the bulked data are to be stored in an ADRF, and optionally an ADRF ID.

-	(Optional, in case the requested data is Event IDs) Data Source identification to collect the data, e.g. NF Instance (or NF Set) ID from which the data needs to be collected.

The output parameter of the Nnwdaf_DataManagement_Subscribe or Ndccf_DataManagement_Subscribe, or Nadrf_DataManagement_Subscribe service operation comprise the subscription correlation ID, which identifies the requested bulked data.

The input parameters of Nnwdaf_DataManagement_Notify or Ndccf_DataManagement_Notify, or Nmfaf_DataManagement_Notify, or Nadrf_DataManagement_Notify service operation shall contain the notification correlation ID, and the generated bulked when the fetch flag = false. When the fetch flag = true the notifications will contain the notification correlation ID and a target address where the data may be retrieved. In case of unsuccessful bulked data generation, the notification will contain an indication of an unsuccessful bulked data generation, optionally with expired bulked data deadline.

The input parameters for the service operation Nnwdaf_DataManagement_Fetch or Ndccf_DataManagement_Fetch, or Nmfaf_DataManagement_Fetch, or Nadrf_DataManagement_Fetch include: the subscription correlation ID (+ Notfication Correlation ID), which identifies the requested bulked data.

The output parameters for the service operation Nnwdaf_DataManagement_Fetch include:

-	the generated bulked data.

The generated bulked data exposed by the above listed service operations comprises:

-	the dataset (i.e., the resulting set of data samples and/or set of pre-processed data samples from the collected event notifications) generated based on the parameters of bulked data request and generation rules, for each data sample:

-	if the bulked data type is ''pre-processed data samples'' the data type (e.g., Event ID) and associated feature type (e.g., average values of Event ID) are included,

-	data value (when ''data samples" bulk data type is used) or processed values (when ''pre-processed data samples'' bulked data type is used),

-	timestamp when the data sample is associated with a bulked data.

# * * * * Next change * * * *

6.2.6.2	Procedure for Data Collection from NWDAF

The procedure in Figure 6.2.6.2-1 is used by NWDAF service consumer to invoke the data management services at NWDAFs in order to retrieve runtime and historical data.

Figure 6.2.6.2-1: Data Collection from NWDAF via Data Management Service

1.	NWDAF service consumer (e.g. NWDAF, DCCF) identifies that further data from an NWDAF is required in order to perform some operation related to analytics ID. The triggers for further data collection are related to:

a)	the local policies of NWDAF, DCCF (e.g. preparation for future requests for analytics ID as specified in clause 6.2.2.1);

b)	a request for analytics generation requiring data not available or not directly reachable via the NWDAF service consumer (e.g. out of the serving area);

c)	a request for model training;

d)	a request for data collection that NWDAF service consumer cannot provide by itself.

NOTE:	In case the NWDAF service consumer is a DCCF, the discovery of the proper NWDAF is defined in Clause 6.2.6.3.6. In case the NWDAF service consumer is a NWDAF, the discovery of proper NWDAFs is defined in Clause 5.2.

2a.	NWDAF service consumer invokes Nnwdaf_DataManagement_Subscribe service from NWDAF to request a required data. The request comprises the Data Specification as well as Formatting and Processing as defined in Clause 6.2.6.1, Notification Target Address (+ Notification Correlation ID).

When the required data is Event IDs, the NWDAF service consumer may include the Data Source, e.g. NF Instance (or NF Set) ID from which the data needs to be collected.

The NWDAF service consumer may include ADRF information indicating whether the data are to be stored in an ADRF, and optionally an ADRF ID.

The NWDAF service consumer may include ADRF ID or NWDAF ID (or ADRF Set ID or NWDAF Set ID) storing historical data (optional), directing NWDAF to the repository containing historical data.

2b. Based on the received request, NWDAF creates a new bulked data for the requesting consumer.

NWDAF sends Nnwdaf_DataManagement_Subscription service response with a confirmation of successful request and the subscription correlation ID identifying the requested data.

NOTE 1:	Subscription Correlation ID allows the NWDAF service consumer to request to NWDAF any changes in the generation of a requested data.

3.	NWDAF determines whether the request data is available at such NWDAF.

NWDAF maintains a local association of requested Event IDs or Analytics IDs to the list of triggered event subscription identifications from data sources to generate the requested data.  Based on this local association, the NWDAF checks if the data to be collected is available at itself. If the data is available, NWDAF uses such data to generate the bulked data.

4a.	(Optional) If NWDAF receives a request for data that is not available or not reachable by such NWDAF (e.g. out of serving area), NWDAF determines the sources for the data that is not available, if the information has not been included in the subscription to the requested data.

When data sources are NFs, the NWDAF discovers the proper NFs as defined in 6.2.2.1.

When the data sources are other NWDAFs, the NWDAF discovers the other NWDAFs as defined in Clause 5.2.

When the data source is DCCF, the NWDAF discovers the proper DCCF as defined in clause 6.3.19, TS 23.501 [2].

4b.	(Optional) NWDAF may trigger further data collection using any of the available mechanisms in Clause 6.2.2 and Clause 6.2.S (e.g., recursively using data collection services from other needed NWDAFs, DCCFs, ADRFs, NFs).

NWDAF updates its local association of the mapping of the requested data (Event ID or Analytics ID) to the identification of the request/subscription for data collection from the further data sources.

5.	Based on the properties of the received request and considering local rules, NWDAF generates the requested data including the available or collected data (e.g. from other NWDAFs, DCCFs or ADRFs, NFs).

6a. In case fetch flag is set to true in Step 2a, NWDAF waits until the requested data is ready and sends a Nnwdaf_DataManagement_Notify service message with the Notification Correlation ID and the target address where the data may be retrieved.

The requested data is ready when NWDAF generated the bulked data based on the available or collected data samples from the event notifications and the processing rules applicable.

6b. In case fetch flag is set to false in Step 2a, NWDAF uses the Nnwdaf_DataManagement_Notify service to send the Notification Correlation ID and requested data to the NWDAF service consumer.

NWDAF waits until the bulked data is generated or the bulked data deadline expires and uses Nnwdaf_DataManagement_Notify service with the indication of the outcome. In case of successful bulked data generation, NWDAF provides the bulked data via Nnwdaf_DataManagement_Notify service as defined in clause 6.2.6.1. In case of bulked data deadline expiration and unsuccessful bulked data generation, NWDAF provides the bulked data via Nnwdaf_DataManagement_Notify an indication of an unsuccessful bulked data generation with expired deadline. The Notification Target Address is used by the Nnwdaf_DataManagement_Notify service operation to deliver the message to the NWDAF service consumer.

7(a.b).	Alternatively, when the fetch flag is set to true in Step 2a, the NWDAF service consumer shall fetch the required data from NWDAF via Nnwdaf_DataManagement_Fetch service operation. The NWDAF service consumer invokes the Nnwdaf_DataManagement_Fetch service operation with the input parameters including the Subscription Correlation ID (+Notification Correlation ID), that identifies the data to be fetched and receives a response with the requested data, as defined in clause 6.2.6.1.

8.	The NWDAF service consumer uses the requested data for performing further processing. If the NWDAF service consumer is an NWDAF the requested data can be used for analytics generation or model training or for further exposing such data to other NWDAFs. If the NWDAF service consumer is a DCCF, the requested data can be used to be provided to a DCCF data consumer.

9.	When the NWDAF service consumer determines that no more data is required, it unsubscribe to the requested data from NWDAF. If NWDAF had triggered further data collection in Step 3a and 3b, NWDAF also unsubscribe to all data sources.

NOTE 2:	It is also possible that instead of providing the dataset of the generated bulked data in steps 6a, 6b, 7b, the NWDAF provides a reference to where the dataset can be retrieved by the NWDAF service consumer.

# * * * * Next change * * * *

7.4	Nnwdaf_DataManagement Service

7.4.1	General

Service Description: This service enables the consumer to subscribe/unsubscribe, be notified about data exposed by NWDAF, or fetch the subscribed data. It enables the consumer to request the generation of bulked data for Event IDs from NFs, as well as for Analytics IDs and retrieve the requested data.

# * * * * Next change * * * *

7.4.2	Nnwdaf_DataManagement_Subscribe service operation

Service operation name: Nnwdaf_DataManagement_Subscribe

Description: The consumer subscribes to receive data, or if the data is already defined in NWDAF, then the subscription is updated.

Inputs, Required: Data Specification, Notification Target Address (+ Notification Correlation ID).

When the required data is a bulked data for Event IDs received from NFs,  the Data Specification includes: set of Event IDs, Event Filter Information, Target of Event Reporting, and bulked data type as defined in Clause 6.2.6.1.

When the required data is a bulked data for Analytics ID, the Data Specification includes: Target of Reporting with the set of Analytics ID(s) to generate bulked data, bulked data type, and analytics stage; Filter Information with Target of Analytics Information, Analytics Filter Information as defined in Clause 6.2.6.1.

NOTE 1:	Event Filter Information, Target of Event Reporting and Bulked data type can be provided per individual Event ID in a set of Event IDs to generate bulked data.

NOTE 2:	Bulked data type, analytics stage, Target of Analytics Information, Analytics Filter Information can be provided per individual Analytics ID in a set of Analytics IDs to generate bulked data.

Inputs, Optional: Service Operation, Formatting and Processing, Data Source, ADRF information to store data used for generated bulked data, ADRF ID or NWDAF ID (or ADRF Set ID or NWDAF Set ID) storing historical data to be used for bulked data generation.

The Formatting and Processing parameters include: Periodic bulked data notification, Feature type, Time Window, Minimum and/or maximum number of samples, Fetch flag, Bulked data deadline, Notification Event Clubbing, Recursive Event Clubbing, Processing rules.

NOTE 2:	The Service Operation (in case of Event IDs), Formatting and Processing, Data Source, and ADRF information to store data used for generated bulked data, ADRF ID or NWDAF ID (or ADRF Set ID or NWDAF Set ID) storing historical data to be used for bulked data generation can be provided per individual Event ID or Analytics ID included in the Data Specification.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of the requested subscription).

Outputs, Optional: None.

# * * * * Next change * * * *

7.4.4	Nnwdaf_DataManagement_Notify service operation

Service operation name: Nnwdaf_ DataManagement_Notify.

Description: NWDAF notifies the consumer instance of the requested data according to the request.

Inputs, Required: Notification Correlation Information, time stamp, Requested Data or Target address where the requested data may be retrieved.

NOTE:	The Rquested Data or Target address can be provided per Event ID or per analytics ID included in the subscription.

Inputs, Optional: Unsuccessful bulked data generation, expired bulked data deadline.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * Next change * * * *

7.4.5	Nnwdaf_DataManagement_Fetch service operation

Service operation name: Nnwdaf_DataManagement_Fetch.

Description: Consumer requests to NWDAF to retrieve the subscribed data by such Consumer.

Inputs, Required: Subscription Correlation IDs, list of Notification Correlation Information.

Inputs, Optional: Set of Event ID or Analytics ID.

Outputs, Required: Requested data.

NOTE:	The requested data  can be provided per Event ID or Analytics ID.

Outputs, Optional: None.

# * * * * End of changes * * * *