3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102501

Elbonia, 12 – 16 April, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	Update of clause 6.3.3

Document for:	Approval

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC/Rel-17

Abstract: This contribution proposes to update description in clause 6.3.3.

# 1. Discussion

This contribution proposes the following changes:

1. Add descriptions on EAS IP replacement triggered by late notification.

Current clause 6.3.3 described the EAS IP replacement triggered by early notification. In Rel-16, both early notification and late notification can be used to trigger the AF to perform EAS relocation. Hence this solution should also cover the EAS relocation triggered by late notification.

- For early notification case, SMF sends notification to AF before the UP path is configured. AF selects target EAS and triggers EAS relocation. Once target EAS is ready, AF responds to SMF about the Target EAS IP address and port number. SMF adds/changes UL CL and Local PSA and configures the Local PSA to perform EAS IP replacement. This was described in current clause 6.3.3.

- For late notification case, the same procedure can be applied as well when the SMF get the late notification response from the AF. This paper proposes to add this procedure in clause 6.3.3.

2. Move the NOTE “AF decides when and how to stop the Source/old Target EAS from serving the UE based on its local configuration” to clause 6.3.3 to cover all procedures in clause 6.3.3.1.

3. Editorial changes.

# 2. Text Proposal

It is proposed to capture the following changes into TS 23.548.

# * * * * First change * * * *

### 6.3.3	Edge Relocation Using EAS IP Replacement

EAS IP replacement enables the Local PSA UPF to replace the source/old target EAS IP address and port number with the target/new target EAS IP address and port number for the Destination IP address and Destination Port number field of the uplink traffic and replace the target/new target EAS IP address and port number with the source/old target EAS IP address and port number for the Source IP address and Source Port number field of the downlink traffic. EAS IP replacement is based on the enhanced AF Influence information for EAS IP replacement (i.e. Support of EAS IP Replacement Capability, source EAS IP address and port number, target EAS IP address and port number).

EAS IP replacement requires support of TCP/TLS/QUIC context transfer between EASs.

NOTE 1:	The feasibility of this requirement, i.e. TCP/TLS/QUIC context transfer between EASs, is unclear and whether third party platforms would support this TCP/TLS/QUIC context transfer between EASs is unknown/not clear.

NOTE 2:	AF decides when and how to stop the Source/old Target EAS from serving the UE based on its local configuration.

#### 6.3.3.1	EAS IP Replacement Procedures

##### 6.3.3.1.1	Enabling EAS IP Replacement Procedure

UE

SMF

Remote

PSA

Local

PSA

PCF

NEF

AF

Target

EAS

Source

EAS

UL CL UPF

1. Establish a PDU Session

2. Discover EAS IP address (A Source EAS IP address is resolved by UE)

3. UL traffic (Src IP: UE IP, Dst IP: Source EAS IP) and DL traffic (Src IP: Source EAS IP, Dst IP: UE IP)

4. AF Influence procedure (SMF reconfigures UL CL and Local PSA) or UE Mobility

5. Early Notification with enhancement described in clause 6.3.3.2.2

7. Late Notification with enhancement described in clause 6.3.3.2.2

6b. UL traffic (Src IP: UE IP, Dst IP: Target EAS IP)

DL traffic (Src IP: Target EAS IP, Dst IP: UE IP)

6a. UL traffic (Src IP: UE IP, Dst IP: Source EAS IP)

DL traffic (Src IP: Source EAS IP, Dst IP: UE IP)

Figure 6.3.3.1.1-1: Enabling EAS IP Replacement Procedure

NOTE 1:	This procedure covers the scenarios that the UE moves from non-EC to EC or the AF decides to enable the EAS IP replacement in the middle of a session.

1.	UE requests to establish a PDU Session.

2.	UE is preconfigured with the Source EAS IP address or discovers the IP address of the application server for the service subject to Edge Computing, and the Source EAS IP address is returned to the UE via EAS Discovery procedure as described in clause 6.2.

3.	UE communicates with the Source EAS.

4.	When AF detects that the EAS is capable of runtime context mirroring and an optimal EAS is found, then AF decides to influence the traffic routing in 5GC. The EAS IP replacement information (i.e. Support of EAS IP Replacement Capability, source EAS IP address and port number, target EAS IP address and port number) is sent to the SMF within the AF Influence information and the SMF reconfigures the UL CL UPF for local traffic routing and Local PSA for EAS IP replacement. Or when UE moves to an area where the Local PSA has been configured to enforce EAS IP address replacement.

UL CL is configured by SMF to forward UL packet in which the destination IP address equals to the Source EAS IP address to Local PSA.

Local PSA is configured by SMF to enforce the "Outer Header Creation" and "Outer Header Removal" FARs as described in step 6. FARs "Outer Header Creation" and "Outer Header Removal" are reused for such an instruction from SMF to UPF.

Detailed enhancement to the AF Influence procedure is described in clause 6.3.3.2.

5.	When Early Notification happens, SMF notifies AF about the target DNAI. Based on the target DNAI, the AF selects a proper target EAS, then the AF triggers to mirror the runtime context between Source EAS and Target EAS. Once the Target EAS is ready, AF responds to SMF about the Target EAS IP address and port number. SMF adds UL CL and Local PSA as described in clause 4.3.5.4 of TS 23.502 [3]. During the procedure, SMF reconfigures Local PSA to perform EAS IP address replacement between Source EAS and Target EAS as described in step 6.

6.	Local PSA starts to perform "Outer Header Creation" and "Outer Header Removal" FARs as instructed by SMF, which results in EAS IP address replacement:

-	For UL traffic, the destination IP address and port number are replaced with the Target EAS IP address and port number;

-	For DL traffic, the source IP address and port number are replaced back with the Source EAS IP address and port number.

At this point, all subsequent uplink traffic of this EC service for this UE is forwarded to the target EAS.

NOTE 2:	In this solution, the PSA UPF need not to understand the logic of EAS IP replacement.

7.	When Late Notification happens, SMF adds UL CL and Local PSA as described in clause 4.3.5.4 of TS 23.502 [3]. The SMF notifies the DNAI Change event to the AF.

If Early Notification was used and the AF provided EAS IP information in step 5, the SMF also notifies AF about the start of the EAS IP replacement.

If Early Notification was not used, based on the target DNAI, the AF may select a proper target EAS, then the AF triggers to mirror the runtime context between Source EAS and Target EAS. Once the Target EAS is ready, AF responds to SMF about the Target EAS IP address and port number. SMF reconfigures Local PSA to perform EAS IP address replacement between Source EAS and Target EAS as described in step 6.

##### 6.3.3.1.2	EAS IP Replacement Update upon DNAI and EAS IP Change

UE

SMF

Local PSA2

Local PSA1

PCF

Target UL CL

AF

Old Target EAS

New Target EAS

Source UL CL

Steps 2-4 are same as steps 5-7 described in clause 6.3.3.1.1

1a. UL traffic (Src IP: UE IP, Dst IP: Source EAS IP)

DL traffic (Src IP: Source EAS IP, Dst IP: UE IP)

1b. UL traffic (Src IP: UE IP, Dst IP: Old Target EAS IP)

DL traffic (Src IP: Old Target EAS IP, Dst IP: UE IP)

Figure 6.3.3.1.2-1: EAS IP Replacement Update upon DNAI and EAS IP change

1.	For UL traffic, the destination IP address is replaced with the old Target EAS IP address at Local PSA; for DL traffic, the source IP address is replaced back with the Source EAS IP address at Local PSA.

Steps 2-4 are same as steps 5-7 described in clause 6.3.3.1.1 except that the UL CL, Local PSA and Target EAS in clause 6.3.3.1.1 are replaced by target UL CL, Local PSA2 and new Target EAS respectively.

##### 6.3.3.1.3	Disabling EAS IP Replacement Procedure

c

UE

SMF

Remote

PSA

Local

PSA

PCF

NEF

AF

Old Target EAS

Source

EAS

UL CL

UPF

3. UL traffic (Src IP: UE IP, Dst IP: Source EAS IP) and DL traffic (Src IP: Source EAS IP, Dst IP: UE IP)

2. Due to UE Mobility to a Non-EC environment, Early Notification is triggered, target DNAI is set to empty value, AF knows the UE moves out of EC environment and mirrors the runtime session context from Old Target EAS to Source EAS. Once ready, the AF indicates SMF to disable the EAS IP replacement at Local PSA for the PDU Session.

1a. UL traffic (Src IP: UE IP, Dst IP: Source EAS IP)

DL traffic (Src IP: Source EAS IP, Dst IP: UE IP)

1b. UL traffic (Src IP: UE IP, Dst IP: Old Target EAS IP)

DL traffic (Src IP: Old Target EAS IP, Dst IP: UE IP)

Figure 6.3.3.1.3-1: Disabling EAS IP Replacement Procedure

1.	Local PSA performs "Outer Header Creation" and "Outer Header Removal" FARs as instructed by SMF, which results in EAS IP address replacement:

-	For UL traffic, the destination IP address and port number are replaced with the old Target EAS IP address and port number;

-	For DL traffic, the source IP address and port number are replaced back with the Source EAS IP address and port number.

2.	Due to UE Mobility to a Non-EC environment, when Early or Late Notification is triggered, target DNAI is set to empty value, AF knows the UE moves out of EC environment and mirrors the runtime session context from old Target EAS to Source EAS. Once Source EAS is ready, the AF indicates SMF to disable the local routing at UL CL and the EAS IP replacement at Local PSA for this PDU Session.

3.	UL and DL traffic goes through Remote PSA, no EAS IP address replacement happens at Remote PSA.

#### 6.3.3.2	Enhancement to AF Influence

The AF may additionally include the capability indication for Support of EAS IP Replacement, Source and Target EAS IP address(es) and Port number(s) in the Nnef_TrafficInfluence_Create/Update request. Based on the Source EAS IP address(es) and port number(s), the SMF knows which service flow(s) is(are) subject to EAS IP Replacement.

SMF sends the "Outer Header Creation" and "Outer Header Removal" FARs to Local PSA UPF as described in step 6 of clause 6.3.3.2, and Local PSA UPF starts the EAS IP address replacement as described in clause 6.3.3.1. Then the SMF may notify AF about the start of EAS IP replacement using Late Notification procedure.

For load balancing purpose, the AF may move some UE(s) from the old Target EAS to the New Target EAS in the same local DN identified by the DNAI. For the abnormal condition of EAS, the AF may move all the UEs being served by the source EAS to a target EAS in the same local DN. For those purposes, the AF needs to include the capability indication for Support of EAS IP Replacement, List of impacted UE IP address and port number, the Source or old Target EAS IP address and port number for the impacted DNAI, the (new) Target EAS IP address and port number for the impacted DNAI in the Nnef_TrafficInfluence_Create/Update request.

The additional parameters for enabling the EAS IP Replacement are defined in clause 5.6.7.1 of TS 23.501 [2], clause 4.3.6.3 and 4.3.6.4 of TS 23.502 [3].

# * * * * End of changes * * * *