3GPP TSG-WG SA2 Meeting #144E e-meeting  	S2-2102571

# Elbonia, Apr 12 – 16, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2668 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates of NSACF Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, China Unicom, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The updates of Network Slice Admission Control function (NSACF) procedure and services to support multiple NSACFs instances. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed the procedure for multiple NSACFs by removing the following Editor’s notes in Clause 4.2.11. Editor's Note: if one S-NSSAI the number of the UE/PDU session is managed by serveral different NSACF, it is FFS how to assure the AMF reaches the same NSACF and how to coordinate the number of UE among the NSACF(s). Editor's Note: It is FFS how to trigger the notification without subscription. Also how to determine which AMF is to be notified need be clarified. It is proposed the procedure for multiple NSACFs by removing the following Editor’s note in Clause 5.2.21. Editor's Note: It is FFS how to support in case multi NSACF is supported, e.g. discover the same NSACF, coordination of the local maximum number among NSACF. The changes are applied to the following clauses: - 4.2.11 Network Slice Admission Control Function (NSACF) procedures - 5.2.21 Network Slice Admission Control Function (NSACF) services |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC cannot support the distributed NSACFs feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.11.2, 4.2.11.3, 4.2.11.x (new), 4.2.11.y (new), 5.2.21.2.1, 5.2.21.2.2, 5.2.21.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.2.11	Network Slice Admission Control Function (NSACF) procedures

#### 4.2.11.1	General

The Network Slice Admission Control Function procedures are performed for an S-NSSAI which is subject to Network Slice Admission Control (NSAC) as described in TS 23.501 [2].

Editor's note:	It is FFS how the NSAC works in the roaming case.

#### 4.2.11.2	Number of UEs per network slice availability check and update procedure

The number of UEs per network slice availability check and update procedure is to update (i.e. increase or decrease) the number of UEs registered with a S-NSSAI which is subject to NSAC. The AMF is configured with the information indicating which network slice is subject to NSAC.

Figure 4.2.11.2-1: Number of UEs per network slice availability check and update procedure

1.	The AMF triggers the Number of UEs per network slice availability check and update procedure to update the number of UEs registered with a network slice when a network slice subject to NSAC is included or removed from the Allowed NSSAI for a UE. The procedure is triggered in the following cases:

-	At UE Registration procedure, as per clause 4.2.2.2.2:

-	before the Registration Accept in step 21 if the EAC mode is active; or

-	after the Registration Accept message if the EAC mode is not active;

-	At UE Deregistration procedure, as per clause 4.2.2.3, after the Deregistration procedure is completed;

-	At UE Configuration Update procedure (which may result from NSSAA procedure):

-	before the UE Configuration Update message if the EAC mode is active; or

-	after the UE Configuration Update message if the EAC mode is not active;

NOTE:	Depending on the deployment, there may be different primary NSACF for different S-NSSAI subject to NSAC. However for one region, there are only one secondary NSACF handles Network Slice Admission Control for all S-NSSAI.

2.	If the S-NSSAI is verified that it can be included in the Allowed NSSAI, and the S-NSSAI is subject to NSAC, the AMF sends Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Request message to the secondary NSACF. The AMF includes in the message the UE ID, the S-NSSAI(s) for which the number of UEs registered per network slice update is required and the update flag which indicates whether the number of UEs registered with the S-NSSAI(s) is to be increased when the UE has gained registration to network slice(s) subject to NSAC or the number of UEs registered with the S-NSSAI(s) is to be decreased when the UE has deregistered from S-NSSAI(s).

3.	The secondary NSACF updates the current number of UEs registered for the S-NSSAI, i.e. increases or decrease the number of UEs registered per network slice based on the information provided by the AMF in the update flag parameter.

If the update flag parameter from the AMF indicates increase and the secondary NSACF finds the UE ID is already in the list of UEs registered with the network slice, the number of the UEs registered with the network slice is not increased as the UE has already been counted as registered with the network slice. If the UE ID is not in the list of UE IDs registered with the network slice and the local maximum number of UEs registered with the network slice has not been reached yet, the secondary NSACF adds the UE ID in the list of UEs registered with the network slice and increases the current number of the UEs registered with the network slice. If the UE ID is not in the list of UEs registered with that S-NSSAI and the local maximum number of UEs per network slice for that S-NSSAI has already been reached, the secondary NSACF performs slice availability check with the primary NSACF as described in Clause 4.2.11.x. If the primary NSACF also reach the maximum number of UEs, then the secondary NSACF returns a result parameter indicating that the maximum number of UEs registered with the network slice has been reached.

If the update flag parameter from the AMF indicates decrease, the secondary NSACF removes the UE ID from the list of UEs registered with the network slice for each of the S-NSSAI(s) indicated in the request from the AMF and also the secondary NSACF decreases the number of UEs per network slice that is maintained by the secondary NSACF for each of these network slices.

4.	The secondary NSACF returns the Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Response in which the secondary NSACF includes the S-NSSAI(s) for which the maximum number of UEs per network slice has already been reached along with a result parameter indicating that the maximum number of UEs registered with the network slice has been reached.

If for all the S-NSSAI(s) the secondary NSACF returned the maximum number of UEs per network slice has been reached, the AMF rejects the UE request for registration. In the Registration Reject message the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI parameter, a reject cause set to 'maximum number of UEs per network slice reached' for each S-NSSAI and optionally a back-off timer.

Editor's note:	It is FFS whether the AMF shall send the Registration Reject message when for all the S-NSSAI(s) that the UE requested to register for and were subject to NSAC the NSACF returned the maximum number of UEs per network slice has been reached.

Otherwise, the AMF returns Registration Accept message in which the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI list for which the secondary NSACF has indicated that the maximum number of UEs per network slice has been reached, a reject cause set to 'maximum number of UEs per network slice reached' for each rejected S-NSSAI and optionally a back-off timer.

Editor's note:	It is FFS whether and how to restrict the signalling sent from the AMFs to the NSACF in case the maximum number of UEs has been reached for prolonged time.

#### 4.2.11.3	Configuration for Early Admission Control (EAC) update procedure

The configuration for Early Admission Control (EAC) update procedure indicates to the AMF the activation or the deactivation of the EAC mode for the S-NSSAI subject to NSAC. EAC mode means that the AMF is required to perform the number of UEs per network slice availability check and update procedure before the S-NSSAI subject to NSAC is included in the Allowed NSSAI and sent to the UE.

Editor's note:	It is FFS if an O&M procedure can be used to configure EAC procedure as opposed to the SUBSCRIBE/NOTIFY method.

Figure 4.2.11.3-1: Early Admission Control (EAC) update procedure

1)	The number of UEs registered with a network slice subject to NSAC crosses a certain operator defined threshold. The secondary NSACF determines whether to activate or deactivate the EAC mode.

2)	The secondary NSACF triggers Nnsacf_NumberOfUEsPerSliceEACNotify operation including the S-NSSAI for which the EAC mode is to be activated or deactivated and a EAC flag set to activated if the number of UEs registered with the network slice is above certain threshold or set to deactivated if the number of the UEs registered with the network slice is below certain threshold.

Editor's note:	It is FFS how to trigger the notification without subscription. Also how to determine which AMF is to be notified need be clarified.

3)	The AMF uses the EAC flag to decide when to trigger the number of UEs per network slice availability check and update procedure so that delays to the registration procedure and impact to the already allowed network slices are avoided.

If the EAC flag indicates EAC mode activated, the AMF triggers the number of UEs per network slice availability check and update procedure before the Registration Accept step of the registration procedure or before the UE Configuration Update message as in EAC activated mode the secondary NSACF is to check whether the maximum number of UEs per network slice is reached which may impact the allowed S-NSSAI(s) confirmed by the Registration Accept message and the UE Configuration Update message.

If the EAC flag indicates EAC mode deactivated, the AMF triggers the number of UEs per network slice availability check and update procedure after Registration Accept step of the registration procedure or after the UE Configuration Update message as in EAC deactivated mode the secondary NSACF does not check whether the maximum number of UEs per network slice is reached, i.e. no impact is expected to the allowed S-NSSAI(s) confirmed by the Registration Accept message and the UE Configuration Update message.

4.2.11.x	Network Slice Admission Control and Update between NSACFs

The primary NSACF of a specific network slice can set and change the values for the local maximum numbers for the related secondary NSACFs individually. The secondary NSACF performs NSAC up to an amount defined by a local maximum number locally. If no local maximum number is available or the local maximum number is already reached, the secondary NSACF interacts with the primary NSACF.

5. Update the local maximum number of UEs or PDU Sessions.

NSACF

(Primary)

NSACF

(Secondary)

2. Nnsacf_SliceAdmission_AvailabilityCheckAndUpdate Request

4. Nnsacf_SliceAdmission_AvailabilityCheckAndUpdate Response

3. The network slice is checked and updated against the global maximum number of UEs or PDU Sessions.

1. A trigger to perform NSAC and update if the local maximum number (e.g., number of UEs or PDU sessions) is reached.

Figure 4.2.11.x-1: Network slice admission control and update procedure between NSACFs

- The secondary NSACF has no local maximum number or has already reached the local maximum number of allowed registered UEs or PDU Sessions of a network slice. Hence, the decision of the acceptance or rejection of the NSAC has to be taken by the primary NSACF. The secondary NSACF triggers to perform NSAC check with the primary NSACF.

For the indicated S-NSSAI, the secondary NSACF discovers and selects the primary NSACF based on local configuration or via the NRF.

- The secondary NSACF sends an NSAC request to the primary NSACF via the service operation < Nnsacf_SliceAdmission_AvailabilityCheckAndUpdate>. The request includes an Indication of check for maximum number of UEs or an Indication of check for maximum number of PDU Sessions, the S-NSSAI, and the secondary NSACF ID.

- The primary NSACF decides about the admission (i.e., accept or reject) by checking whether the overall global maximum number related to the received indication and S-NSSAI has already been reached.

If the global maximum number has not been reached for the indicated S-NSSAI, the primary NSACF derives a local maximum number for that secondary NSACF ID. Based on that, the primary NSACF updates the relation between the allocated local maximum number of registered UEs or PDU Sessions of the S-NSSAI and the secondary NSACF ID.

- The primary NSACF sends the < Nnsacf_SliceAdmission_ AvailabilityCheckAndUpdate> response to the requested secondary NSACF indicating the acceptance or rejection of the UE registration or PDU Session establishment.

In case of a rejection, the response includes a result parameter indicating that the global maximum number for the S-NSSAI has been reached. In case of an acceptance, the response include a local maximum number for UEs or PDU Sessions for the S-NSSAI to be applied by the secondary NSACF.

- [Conditional - depending on the step 4] In the case of an acceptance response, the secondary NSACF updates the local maximum number for the S-NSSAI if it is included in the response from the primary NSACF.

4.2.11.y	Network Slice Status Update between NSACFs

Figure 4.2.11.y-1: Network slice status update procedure between NSACFs

- The primary NSACF subscribes to notifications on the number of registered UEs, the number of established PDU Sessions, or both from the secondary NSACF via the service operation <Nnsacf_EventExposure_StatusSubscribe> with Event ID, Event Filter, Event Reporting information. The Event ID indicates the subscribed event ID, i.e.  Network Slice Registered UEs or Network Slice Established PDU Sessions. The Event Filter includes the information of S-NSSAI. The event reporting information includes reporting thresholds that can be periodically or threshold based (e.g. a certain percentage out of the local maximum number of allowed UEs or PDU Sessions is reached).

- The secondary NSACF monitors the event(s) and event reporting information for triggering the notification to the primary NSACF if the event is matched.

- The secondary NSACF notifies the primary NSACF via the service operation <Nnsacf_EventExposure_StatusNotify> when the conditions for the event ID are fulfilled, e.g., the status of registered UEs or established PDU sessions periodically or when a threshold is crossed according to the event reporting information described in the subscription request.

- The primary NSACF updates the current status of the overall registered UEs or PDU Sessions or both for specific S-NSSAI(s).

- The primary NSACF responds to the secondary NSACF. Per the current status of the overall registered UEs or PDU Sessions, the local maximum number of allowed registered UEs or PDU Sessions of a network slice may be included

- [Conditional - depending on the step 5] The secondary NSACF updates the local maximum number for the S-NSSAI if it is included in the response from the primary NSACF.

# * * * * Second change * * * *

5.2.21	Network Slice Admission Control Function (NSACF) services

5.2.21.1	General

The following table illustrates the NSACF services.

Table 5.2.21.1-1: List of NSACF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnsacf_NumberOfUEsPerSlice | AvailabilityCheckAndUpdate | Request/Response | AMF |
|  | EACNotify |  | AMF |
| Nnsacf_SliceAdmission | AvailabilityCheckAndUpdate | Request/Response | Secondary NSACF |

5.2.21.2	Nnsacf_NumberOfUEsPerSlice services

5.2.21.2.1	General

Service Description: The Nnsacf_NumberOfUEsPerSlice services control the number of UEs registered with a network slice for the network slices subject to NSAC. The AMF can request the NSACF to check whether the number of UEs registered with a network slice has reached the maximum number of UEs per network slice and the AMF can also request the NSACF to update the number of UEs registered with a network slice.

5.2.21.2.2	Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate service operation

Service Operation name: Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate

Description: Updates the number of UEs registered with a network slice (e.g. increase or decrease) when the UE registration status for a network slice subject to NSAC has changed. Also, if the number of the UEs registered with the network slice is to be increased and the Early Availability Check (EAC) mode in the NSACF is activated for that network slice (see Nnsacf_NumberOfUEsPerSliceEACNotify service operation), the NSACF first checks whether the number of UEs registered with the network slice has reached the maximum number of UEs per network slice threshold. If the maximum number of UEs registered with the network slice has already been reached, the UE registration for that network slice is rejected. If the EAC is not activated, the NSACF increases or decreases the number of UEs per network slice as per the input parameters below.

Inputs, Required: S-NSSAI(s), UE ID (SUPI), update flag.

The S-NSSAI(s) parameter is a list of one or more network slices for which the number of UEs registered with a network slice is to be updated and checked if the maximum number of UEs per network slice threshold has already been reached.

The UE ID is used by the NSACF to maintain a list of UE IDs registered with the network slice.

The update flag input parameter indicates whether the number of UEs registered with a network slice is to be:

-	increased when the UE registers to a new network slice subject to NSAC. If the UE ID is already in the list of UEs registered with the network slice, the number of UEs registered with the network slice is not increased as the UE has already been counted as registered with the network slice. If the UE ID is not in the list of UE IDs registered with the network slice and the maximum number of UEs registered with the network slice has not been reached yet, the NSACF adds the UE ID in the list of UEs registered with the network slice and increases the number of the UEs registered with the network slice. If the UE_ID is not in the list of UEs registered with that S-NSSAI and the maximum number of UEs per network slice for that S-NSSAI has already been reached, then the NSACF returns maximum number of UEs per network slice reached result;

-	decreased when the UE deregisters for a network slice that is subject to NSAC. The NSACF decreases the number of the UEs registered with the network slice and removes the UE ID from the list of UEs registered with the network slice.

The NSACF may optionally return the current status of the network slice availability (e.g. a percentage out of the max number of UEs registered with a network slice) in the availability status parameter. This information may be used for NSACF signalling and load balancing in case multiple NSACFs are serving the same network slice.

Outputs, Required: maximum number of UEs per network slice reached, availability status.

5.2.21.2.3	Nnsacf_NumberOfUEsPerSliceEACNotify service operation

Service Operation name: Nnsacf_NumberOfUEsPerSliceEACNotify

Description: The NSACF is configured with the information about which network slices are subject to NSAC. The NSACF may trigger notification to the AMF to indicate the activation of the Early Availability Check (EAC) mode for a certain network slice which is subject to NSAC when the number of the UEs registered with the network slice is above certain operator defined threshold (e.g. a percentage of the maximum number of UEs allowed to register with the network slice) and the deactivation of the EAC mode when the number of the UEs registered with the network slice is below certain operator defined threshold which may be same or different from the activation threshold.

Inputs, Required: S-NSSAI, EAC flag.

The S-NSSAI input parameter is the network slice for which the NSACF activates or deactivates the EAC mode.

The EAC flag input parameter indicates whether the Slice EAC mode is activated or deactivated.

5.2.21.y	Nnsacf_SliceAdmission Services

5.2.21.y.1	General

Service Description: The NF Service Consumer is required to query the primary NSACF to check the admission of the UE registration or PDU Session establishment whenever the NF Service Consumer has no local maximum number available or the local maximum number has been reached. The primary NSACF may optionally provide an updated local maximum number of allowed registered UEs or PDU Sessions as part of the response. Thus the NF Service Consumer can use the updated local maximum number without querying the NSACF again for the indicated S-NSSAI.

5.2.21.y.2	Nnsacf_SliceAdmission_AvailabilityCheckAndUpdate service operation

Service operation name: Nnsacf_SliceAdmission_AvailabilityCheckAndUpdate

Description: The consumer NF, e.g., secondary NSACF requests the primary NSACF to check the admission of a UE registration or PDU Session establishment to an S-NSSAI subject to quota information.

Inputs Required: S-NSSAI, Indication of check for maximum number of registered UEs, Indication of check for the maximum number of PDU Sessions, secondary NSACF ID.

Inputs, Optional: None.

Outputs Required: Operation execution result indication.

Outputs, Optional: local maximum number of the allowed registered UEs or PDU Sessions for the S-NSSAI.

# * * * * End of changes * * * *