3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102572

Elbonia, Apr 12 – 16, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2788 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Slice Admission Control on PDU Sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the conclusions for KI#2 in the TR 23.700-40, network slice admission control on PDU sessions needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Maximum number of PDU sessions per network slice admission control is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI#2 conclusion in the TR 23.700-40 is not implemented in TS 23.501 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.15.11.x	Maximum number of PDU Sessions per network slice admission control

The NSACF controls (i.e. increase or decrease) the current number of PDU Sessions established for a network slice so that it does not exceed the maximum number of PDU Sessions allowed to be established for that network slice.

When the current number of PDU Sessions associated with a network slice is to be increased, the secondary NSACF checks whether the local maximum number of PDU Sessions for that network slice has already been reached and if so, interacts with the primary NSACF. If the local maximum number has not been reached, the secondary NSACF increases the current number of PDU Sessions for that network slice.

When the primary NSACF receives a request for NSAC from the secondary NSACF, it checks whether the global maximum number of PDU Sessions for that network slice has already been reached. If this is not the case, the primary NSACF provides or changes the local maximum number for that network slice in the response to the secondary NSACF.

When the current number of PDU Sessions established for a network slice is to be decreased, the secondary NSACF decreases the current number of PDU Sessions established for that network slice.

The SMF triggers a request to a secondary NSACF instance for the maximum number of PDU Sessions per network slice admission control when the SMF’s PDU Session status for a network slice subject to NSAC is changed, i.e. during the PDU Session Establishment procedure in clause 4.3.2 in TS 23.502 [3], PDU Session Release procedure in clause 4.3.4 in TS 23.502 [3].

Editor's Note: For the change of SSC mode 2 and SSC mode 3 PDU Session Anchor, how does the NSACF bypass PDU session number availability check is FFS.

# * * * * End of changes * * * *