3GPP TSG-SA WG2 Meeting #144-e	S2-2102605

Elbonia, April 12 – 16, 2021

Title:	[Draft] Reply LS on NAS Non delivery for RRC_INACTIVE state

Response to:	Reply LS on NAS Non delivery for RRC Inactive state (S2-2102083/ R3-207170)

Release:	3GPP Rel-15/Rel-16

Work Item:	NR_newRAT-Core

Source:	SA2

To:	RAN3

Cc:	CT4

Contact Person:	Yunjing Hou

E-mail Address:	houyunjing (at) catt (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thanks RAN3 about the LS on NAS Non delivery for RRC Inactive state (R3-207170). SA2 discussed the questions and agreed the following answers.

Question 1: For a UE in RRC_INACTIVE state, is there any use case for AMF to piggyback a non-PDU session related NAS PDU in PDU SESSION RESOURE SETUP REQUEST?

Answer 1: The scenario described in TS 23.502 related to Question 1 is as follows:

“If the Service Request procedure is triggered by the Network (as described in clause 4.2.3.3) while the UE is in CM-CONNECTED state, only N2 SM information received from SMF is included in the N2 Request.”

Based on the description above, AMF will not piggyback a non-PDU session related NAS PDU in PDU SESSION RESOURCE SETUP REQUEST.

Question 2: Which solution is preferred to inform AMF the non-delivery of the non-PDU session related NAS-PDU in the "Initial Context Setup Request"?

- Solution 1: Use NAS NON DELIVERY INDICATION message to indicate the failure of the NAS delivery.

- Solution 2: Use the "Initial Context Setup failure” to implicitly indicate the failure of the NAS delivery.

Answer 2: Normally SA2 does not specify error handling, but exceptions can be made if there are system wide impact reasons to do so. One specific case is already defined for RRC inactivated state UE in clause 5.3.3.2.5 of TS23.501 as,

“If the RAN paging procedure, as defined in TS 38.300 [27], is not successful in establishing contact with the UE the procedure shall be handled by the network as follows:

-	If NG-RAN has at least one pending NAS PDU for transmission, the RAN node shall initiate the AN Release procedure (see TS 23.502 [3], clause 4.2.6,) to move the UE CM state in the AMF to CM-IDLE state and indicate to the AMF the NAS non-delivery.”

2. Actions:

To SA2 group:

ACTION: SA2 requests RAN3 and CT4 to take the above information into account.

3. Date of Next SA2 Meetings:

SA2#145-e	17 – 28 May 2021	Electronic