3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102833

Elbonia, April 12 – April 16, 2021	(revision of S2-2100xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0328 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Analytics subscription transfer - procedures update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1) Resolve "Editor's note: It is FFS whether this procedure is useful for any other analytics than UE related." As described in clause 6.1B.1, this procedure covers both internal triggers (e.g. load balancing, graceful shutdown) and external triggers (e.g. UE mobility). - Analytics subscription transfer can be triggered by UE mobility when either the data source is changing due to UE mobility, or when the analytics consumer is changing due to UE mobility. - Analytics subscription transfer triggered NWDAF internally can be applied to any kind of analytics ID. Therefore, this procedure is not limited to UE related analytics. 2) UE mobility can result in the data source changing and the analytics consumer changing at the same time, e.g. when an AMF is subscribed with the NWDAF for UE mobility analytics, where AMF is both the analytics consumer and the data source for the analytics generation. If the source NWDAF selects a new target NWDAF (e.g. NWDAF1), while at the same time, the new AMF does not take over the analytics subscriptions, but creates a new analytics subscription with a target NWDAF (e.g. NWDAF2), it might happen that two different NWDAFs (e.g. NWDAF 1 and NWDAF 2) are selected. In addition, as a new subscription is created by the new AMF, the analytics context of the UE related analytics is lost, except if it is recovered using a workaround solution. To avoid this situation, it is proposed that the new target AMF may determine to take over those analytics subscriptions received as part of the UEContext during AMF reallocation and firstly contact the old NWDAF (which had been serving the old AMF). The old NWDAF then can, if necessary, trigger an analytics subscription transfer to a new target NWDAF serving the new AMF. Using this preferred option, the UE related analytics subscription with its analytics context is not lost, but is handed over to the new target NWDAF. Further, no second analytics subscription is created by the new AMF, avoiding above mentioned problem that two different target NWDAFs might be selected by the new AMF and the old NWDAF (both triggered by the UE mobility event). 3) Resolve "Editor's note: It is preferably to have one procedure to cover the re-selection of NWDAF covering both when consumer change and when not. But it is FFS how." This CR provides a common procedure to cover the re-selection of NWDAF both when the analytics consumer changes and when not. 4) Resolve " Editor's note: It FFS how this aligns with the procedure when Analytics consumer changes." |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Remove the EN. 2) Extending the analytics subscription transfer procedure to allow an analytics subscription transfer from the old NWDAF to the new NWDAF following an AMF reallocation. 3)+4) Remove the ENs as the proposed extension of the procedure resolves the issue mentioned in the ENs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor's Notes are not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of the changes ***

## 6.1B	Procedures for analytics subscription transfer

### 6.1B.1	General

In multiple NWDAFs deployment scenario, it is possible that, within the time period of data collection and/or analytics computation, an NWDAF instance is required to transfer one or more of its analytics subscriptions to another NWDAF instance due to internal triggers (e.g. load balancing, graceful shutdown) or external triggers (e.g. UE mobility). For external triggers, NWDAF subscribes with NF(s) to be notified about corresponding events. As for UE mobility, upon an UE location change event notified by an AMF subscribed by NWDAF, NWDAF determines whether it can continue to provide the analytics service, or should trigger analytics subscription transfer to another NWDAF instance.

Procedures for analytics subscription transfer allow one NWDAF instance to transfer or handover its ongoing analytics subscriptions to another NWDAF instance. The transfer can be done for all subscriptions or just a selected subset of subscriptions related to specific area(s), specific Analytics ID(s), and/or specific UE(s).

The analytics subscription transfer can also be prepared if the source NWDAF instance anticipates that it will soon not be able to continue its current analytics tasks.

### 6.1B.2	Analytics Subscription Transfer

The procedure in Figure 6.1B.2-1 is used by an NWDAF instance to request the transfer of analytics subscription(s) to another NWDAF instance, using the Nnwdaf_AnalyticsSubscription_Transfer service operation defined in clause 7.2.5. If the NWDAF analytics consumer is of type AMF, this procedure also covers a potential change of the analytics consumer, which is triggering the NWDAF to check if it is still the appropriate NWDAF to serve the new analytics consumer or if the analytics subscriptions of the new analytics consumer should be transferred to another NWDAF; in the latter case, the NWDAF will initiate the analytics subscription transfer for those analytics subscriptions.

NOTE 1:	When the subscribed Analytics involves UE related data and the analytics consumer serves the same geographic areas as the NWDAF, crossing a certain tracking area boundary can cause both analytics consumer and serving NWDAF to change.

NOTE 2:	Handling of overload situation or preparation for a graceful shutdown are preferably executed inside an NWDAF Set, when available, therefore, not requiring an analytics subscription transfer as described in this clause. The procedure in Figure 6.1B.2-1 is applicable for analytics subscription transfer across NF Sets or if the NWDAF is not deployed in a Set.

Figure 6.1B.2-1: Analytics subscription transfer

1a.	Source analytics consumer subscribes, using Nnwdaf_AnalyticsSubscription_Subscribe service operation, to source NWDAF to receive analytics (e.g. UE related analytics) as specified in clause 6.1.1).

1b.	Source NWDAF starts data collection from relevant data source(s) (e.g. NFs or OAM) as specified in clause 6.2. Source NWDAF starts generating requested analytics.

2.	[Conditional] (Only if the source NWDAF does not serve the whole PLMN and the requested analytics involves UE related data) The source NWDAF subscribes, using Namf_EventExposure_Subscribe and/or Nsmf_EventExposure_Subscribe service operation, to receive notifications on UE mobility events from the AMF and/or SMF serving the UE, respectively.

NOTE 3:	The AMF and/or SMF serving the UE can be the analytics consumer and/or data source.

4.	[Optional] Source NWDAF determines, e.g. triggered by a UE mobility event notification, to prepare an analytics subscription transfer to target NWDAF(s), as specified in the procedure illustrated in Figure 6.1B.2-2.

5.	[Conditional] (Only if the analytics consumer is of type AMF) Due to UE mobility, the target analytics consumer starts serving the UE.

6a.	[Conditional] (Only if the analytics consumer is of type AMF) The target analytics consumer requests the transfer of UE context from the source analytics consumer, using Namf_Communication_UEContextTransfer service operation.

6b.	[Conditional] (Only if the analytics consumer is of type AMF) The source analytics consumer (i.e. AMF) responds with the requested UE context including the analytics subscriptions with source NWDAF and the identifier of the source NWDAF that the source analytics consumer is subscribed with to receive analytics related to the UE.

6c. [Conditional] (Only if the analytics consumer is of type AMF) The target analytics consumer (i.e. AMF) may determine to take over the analytics subscriptions received from the source analytics consumer. Alternatively, the target Analytics consumer may determine not to take over the analytics subscription and instead create a new analytics subscription with a new target NWDAF. In that case, it has to inform the source AMF such that the source AMF can terminate the analytics subscription with the source NWDAF.

NOTE x:	It is recommended that the AMF takes over the existing analytics subscription along with the analytics context associated with the analytics subscription. If a new analytics subscription is created by the target AMF with a new target NWDAF, the analytics context available at the source NWDAF might be lost. In addition, the target NWDAF selected by the target AMF may differ from the target NWDAF selected by the source NWDAF (triggered by the UE mobility event) in step 9.

7.	[Conditional] (Only if the analytics consumer is of type AMF) For the analytics subscription(s) received in step 6b and determined to take over in step 6c, the target analytics consumer informs the source NWDAF about the changed analytics consumer by notifying the new Callback address of the new analytics consumer using the Namf_EventExposure_Notify service operation to modify the analytics subscription. This message from the target analytics consumer to the source NWDAF may include a "preferred target NWDAF".

Editor's Note:	Whether the "preferred target NWDAF" is needed is FFS.

8.	If the source NWDAF determines, e.g. triggered by a UE mobility event notification, that the analytics consumer for analytics subscription(s) has changed to the target analytics consumer, it will update the affected analytics subscriptions to the target analytics consumer and the new subscription correlation ID.

9.	Source NWDAF determines, e.g. based on the UE location information received, the analytics subscription(s) to be transferred to a target NWDAF. Therefore, the source NWDAF discovers via NRF the target NWDAF supporting the requested analytics information for the target area. NWDAF discovery may be skipped if the target NWDAF had already been discovered as part of a prepared analytics subscription transfer in step 4 or if a "preferred target NWDAF" was provided by the target analytics consumer in step 7. NWDAF discovery and selection is specified in clause 6.3.13, TS 23.501 [2]

Editor's Note:	Whether the source NWDAF takes the "preferred target NWDAF" into account in its decision, or the source NWDAF will always use the indication provided by the target analytics consumer is FFS.

10a.	Source NWDAF requests, using Nnwdaf_AnalyticsSubscription_Transfer Request service operation, a transfer of the analytics subscription(s) determined in step 9 to the target NWDAF. The request contains a callback URI of the target analytics consumer. The request also contains Id(s) of active data sources related to the analytics subscriptions requested to be transferred, if not already provided as part of the prepared analytics subscription transfer in the preparation procedure (see step 4). The request message may also include "analytics context identifier(s)" indicating the availability of analytics context for particular Analytic ID(s).

NOTE 4:	If multiple notifications/requests are received in steps 3 and 7, indicating the necessity to transfer analytics subscriptions, the source NWDAF will only trigger a single analytics subscription transfer and ignore additional notifications/requests that would result in the same analytics subscription transfer.

10b.	Target NWDAF accepts the analytics subscription transfer and takes over the analytics generation based on the information received from the source NWDAF.

NOTE 5:	If not yet done during a prepared analytics subscription transfer, the target NWDAF allocates a new subscription Id to the received analytics subscriptions, following the behaviour of AmfEventSubscription as specified in TS 29.518 [25] clause 6.2.6.2.2 and UEContextTransfer as specified in TS 29.518 [25] clause 5.2.2.2.1.1.

NOTE 6:	The target NWDAF might already have received information on some/all of the analytics subscriptions as part of the prepared analytics subscription transfer request received in step 4 and, thus, might already have started to prepare for the analytics generation, e.g. by having already subscribed to relevant event notifications.

10c.	Target NWDAF informs the target analytics consumer about the successful analytics subscription transfer using a Nnwdaf_AnalyticsSubscription_Notify message and providing the new subscription Id, that had been assigned by the target NWDAF, in the Change Subscription Notification Correlation Id parameter of this message as specified in clause 7.2.4.

NOTE 7:	The existing Analytics context in the source NWDAF is not deleted directly but will be purged first when it was collected by the target NWDAF.

11.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_AnalyticsSubscription_Transfer Request received in step 10a, the target NWDAF, based on the provided "analytics context identifier(s)", shall request the "analytics context" from the source NWDAF. The analytics information transfer procedure is specified in clause 6.1B.3.

Editor's note:	It is FFS, whether procedure in clause 6.1B.3 is specified, or other new/existing service operations are used instead.

10d.	[Optional] Target NWDAF subscribes to relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the Analytics.

10e.	Target NWDAF confirms the analytics subscription transfer to the source NWDAF.

10f.	[Optional] Source NWDAF unsubscribes with the data source(s) that are no longer needed for the remaining analytics subscriptions.

12a,b.	Target NWDAF at some point derives new output analytics based on new input data and notifies the analytics consumer about the new analytics using a Nnwdaf_AnalyticsSubscription_Notify message as specified in clause 6.1.1.

NOTE 8:	Notification correlation information allows the analytics consumer to correlate the notifications to the initial subscription request made with the source NWDAF in step 1a.

The procedure in Figure 6.1B.2-2 is used by an NWDAF instance to request another NWDAF instance to prepare taking over analytics subscription(s) from the source NWDAF instance, using the Nnwdaf_AnalyticsSubscription_Transfer service operation defined in clause 7.2.5.

NOTE 9:	For simplification, this procedure visualizes when the analytics consumer does not change.

Figure 6.1B.2-2: Analytics subscription transfer with prepared analytics subscription transfer

The procedure of analytics subscription transfer comprises the following steps:

1a.	Analytics consumer subscribes to the source NWDAF for certain analytics as specified in clause 6.1.1.

1b.	Source NWDAF starts data collection from relevant data source(s) (e.g. NFs or OAM) as specified in clause 6.2. Source NWDAF starts generating requested analytics.

2.	[Conditional] (Only if the source NWDAF does not serve the whole PLMN and the requested analytics involves UE related data) The source NWDAF subscribes, using Namf_EventExposure_Subscribe and/or Nsmf_EventExposure_Subscribe service operation, to receive notifications on UE mobility events AMF and/or SMF, respectively.

If the source NWDAF determines that it needs to prepare to transfer analytics subscriptions to another NWDAF instance, the following steps 3 to 8 shall be executed:

NOTE 10:	The source NWDAF might determine that it needs to prepare to transfer analytics subscriptions to another NWDAF instance, as the source NWDAF estimates that the UE might enter an area which is not covered by the source NWDAF, e.g. by subscribing to AMF event exposure service for UE mobility event notifications, by performing UE mobility analytics, or by subscribing to another NWDAF providing UE mobility analytics.

Another example for source NWDAF to determine to prepare for an analytics transfer is that the source NWDAF wants to resolve an overload situation or prepare for a graceful shutdown.

NOTE 11:	Handling of overload situation or preparation for a graceful shutdown are preferably executed inside an NWDAF Set, when available.

3.	The source NWDAF discovers, via NRF, candidate target NWDAF instances (e.g. NWDAFy and NWDAFz) supporting the requested analytics information for the predicted target area(s). NWDAF discovery and selection is specified in clause 6.3.13, TS 23.501 [2].

NOTE 12:	In this procedure, NWDAFy and NWDAFz are examples for target NWDAF instances that are candidates to take over those analytic subscriptions.

4-5.	In the case of a prepared analytics subscription transfer, the source NWDAF requests, using Nnwdaf_AnalyticsSubscription_Transfer Request, to the candidate target NWDAFs (e.g. NWDAFy and NWDAFz) to prepare for an analytics subscription transfer by including a "prepared analytics subscription transfer indication" in the request message. The request message also includes information on the analytics subscriptions to be transferred. The request message may also include "analytics context identifier(s)" indicating the availability of analytics context for particular Analytics ID(s).

The candidate target NWDAFs (e.g. NWDAFy and NWDAFz) respond to the request from the source NWDAF using a Nnwdaf_AnalyticsSubscription_Transfer Response message.

6-7.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_AnalyticsSubscription_Transfer Request received in step 4, the determined target NWDAFs (e.g. NWDAFy and NWDAFz), based on the provided "analytics context identifier(s)", shall request the "analytics context" from the source NWDAF. The analytics information transfer procedure is specified in clause 6.1B.3.

Editor's note:	It is FFS, whether procedure in clause 6.1B.3 is specified, or other new/existing service operations are used instead.

NOTE 13:	The target NWDAFs (e.g. NWDAFy and NWDAFz) allocate a new subscription Id to the received analytics subscriptions, following the behaviour of AmfEventSubscription as specified in clause 6.2.6.2.2, TS 29.518 [25] and UEContextTransfer as specified in clause 5.2.2.2.1.1, TS 29.518 [25].

8.	[Optional] Based on the information received from the source NWDAF, the target NWDAFs (e.g. NWDAFy and NWDAFz) start data collection from NFs or OAM (as specified in clause 6.2) and analytics generation for the indicated analytics subscriptions.

The source NWDAF determines to transfer analytics subscription(s) to a target NWDAF instance NWDAF (e.g. NWDAFy) and the following steps are executed:

9.	Source NWDAF initiates the analytics subscription transfer to the target NWDAF as specified in steps 3 to 5b of the analytics subscription procedure illustrated in Figure 6.1B.2-1.

NOTE 14:	If the target NWDAF (e.g. NWDAFy) had accepted to prepare for an analytics subscription based on a prepared analytics subscription transfer request received from the source NWDAF in step 4, the "analytics context identifier(s)" provided in the Nnwdaf_AnalyticsSubcscription_Transfer request in step 9 indicate additional information / data collected or generated by the source NWDAF between step 6 and step 9.

10.	[Conditional] If a candidate target NWDAF (e.g. NWDAFz) had accepted to prepare for an analytics subscription based on a prepared analytics subscription transfer request received from the source NWDAF in step 5, but had not been selected as the new target NWDAF in step 9, the source NWDAF cancels the prepared analytics subscription transfer to this target candidate NWDAF (e.g. NWDAFz), using Nnwdaf_AnalyticsSubscription_Transfer Request include an "analytics subscription transfer cancel indication". The target NWDAF (e.g. NWDAFz) confirms the cancelation to the source NWDAF and, if applicable, deletes any analytics data that is no longer needed. If the target NWDAF (e.g. NWDAFz), as part of the analytics subscription preparation, had already subscribed to entities to collect data, it unsubscribes to those entities if the subscriptions are not needed for other active analytics subscriptions with the target NWDAF.

### 6.1B.3	Analytics Context Information Transfer

The procedure depicted in Figure 6.1B.3-1 is used by an NWDAF instance to request analytics context information from another NWDAF instance, using the Nnwdaf_AnalyticsInfo_Transfer service operation as defined in clause 7.3.3. This procedure, for example, can be called during an analytics subscription transfer procedure (as specified in clause 6.1B.2) to request the transfer of relevant analytics information.

Figure 6.1B.3-1: Analytics Context Information Transfer

The procedure of analytics information transfer comprises the following steps:

1.	The consumer NWDAF requests analytics context information by invoking Nnwdaf_AnalyticsInfo_Request service operation. The parameters that can be provided in the request message are listed in clause 6.1B.4.

2.	The provider NWDAF responds with analytics context information to the consumer NWDAF.

### 6.1B.4	Contents of Analytics Context Information

The consumers of the Nnwdaf_AnalyticsInfo_Transfer service operation (as specified in clause 7.3.3) provide the following input parameters listed below:

-	A list of analytics context identifier(s): identify a set of analytics context information that are available at the NWDAF instance providing this service and that are requested to be transferred to the consumer NWDAF instance.

NOTE:	A list of "analytics context identifier(s)" is received during an analytics subscriptions transfer request as described in clause 6.1B.2.

Editor's note:	It is FFS whether an enhanced Analytics request is to be used instead of transfer service for Analytics. And it is FFS whether bulk data is to be used for Data instead of transfer service.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsInfo_Transfer service operation (as specified in clause 7.3.3), the output information listed below:

-	(Set of) Analytics context information for each of the "analytics context identifier(s)" provided as input parameter. "Analytics context information" includes the following information parts:

-	Analytics related:

-	[OPTIONAL] Pending output analytics (i.e. not yet notified to the consumer) or historical output analytics information, if available at the source NWDAF. The content of the output analytics is specified in clause 6.1.3 as output information of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations.

-	Timestamp(s) of the last output analytics provided to the analytics consumer(s). Value is set to 0 if no output analytics had been sent yet.

-	Data collection related:

-	[OPTIONAL] Historical data that is available at the source NWDAF and that is related to the Analytics to be handed over to the target NWDAF. This includes not only the raw data collected from the data source(s), but also "analytics metadata" such as the time period of the collected data, information about the data source(s), etc. If historical data is available at the source NWDAF, this data shall be provided.

-	Information about subscriptions with data sources that are related to the analytics.

### *** End of the changes ***