SA WG2 Meeting #144E (e-meeting)	S2-2102837

# April 12 – 16, 2021 									                                 (revision of S2-210XXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2831 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the handling of AUSF and UDM selection for an Onboarding UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In order to support UE onboarding, the selected AUSF for the UE shall support the authenticaion procedure based on defautl credential and select the DCS. Hence, an AUSF instance or AUSF group ID used for Onboaring shall be preconfigured on the AMF supporting Onboaring, the AMF can use the AUSF group ID used for Onboaring to select an AUSF for the Onboaring UE. The SUPI/SUCI for an Onboarding UE includes the index information for DCS, so the information (e.g. realm) of the SUPI/SUCI cannot be used for AUSF selection or UDM selection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - An AUSF instance or AUSF group ID used for Onboaring shall be preconfigured on the AMF supporting Onboaring, the AMF can use the AUSF group ID used for Onboaring to select an AUSF for the Onboaring UE. - An NF (e.g. AUSF) supporting Onboarding as specified 5.30.2.X, shall not use the information (e.g. realm) in the of SUCI/SUPI of an Onboarding UE for selection of AUSF or UDM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AMF cannot support to select accurate AUSF for the Onboarding UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.4, 6.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start of changes *****

6.3.4	AUSF discovery and selection

In the case of NF consumer based discovery and selection, the following applies:

-	The AMF performs AUSF selection to allocate an AUSF Instance that performs authentication between the UE and 5G CN in the HPLMN. The AMF shall utilize the NRF to discover the AUSF instance(s) unless AUSF information is available by other means, e.g. locally configured on AMF. The AUSF selection function in the AMF selects an AUSF instance based on the available AUSF instances (obtained from the NRF or locally configured in the AMF).

-	The UDM shall utilize the NRF to discover the AUSF instance(s) unless AUSF information is available by other means, e.g. locally configured on UDM. The UDM selects an AUSF instance based on the available AUSF instance(s) obtained from the NRF or based on locally configured information, and information stored (by the UDM) from a previously successful authentication.

AUSF selection is applicable to both 3GPP access and non-3GPP access.

The AUSF selection function in AUSF NF consumers or in SCP should consider one of the following factors when available:

1.	Home Network Identifier (e.g. MNC and MCC, realm) of SUCI/SUPI (by an NF consumer in the Serving network) along with the selected NID (provided by the NG-RAN) in the case of SNPN and Routing Indicator.

NOTE 1:	The UE provides the Routing Indicator to the AMF as part of the SUCI as defined in TS 23.003 [19] during initial registration. The AMF can provide the UE's Routing Indicator to other AMFs as described in TS 23.502 [3].

NOTE 2:	In the case of SNPN and if the UE provides IMSI as SUPI/SUCI to the AMF and if SUCI/SUPI is for an SNPN served by the AMF, the AMF uses the selected NID provided by the NG-RAN together with the selected PLMN ID (from SUCI/SUPI) as the SUCI/SUPI does not always include the NID. In the case of SNPN and the UE provides NSI as SUPI/SUCI to the AMF, the AMF uses the Home Network Identifier for selection of AUSF.

NOTE X1: The AMF supporting Onboarding as specified 5.30.2.X, shall not use the information in the realm part of the SUCI/SUPI of an Onboaridng UE for selection of AUSF.

When the UE's Routing Indicator is set to its default value as defined in TS 23.003 [19], the AUSF NF consumer can select any AUSF instance within the home network for the UE.

2.	AUSF Group ID the UE's SUPI belongs to.

NOTE 3:	The AMF can infer the AUSF Group ID the UE's SUPI belongs to, based on the results of AUSF discovery procedures with NRF. The AMF provides the AUSF Group ID the SUPI belongs to other AMFs as described in TS 23.502 [3].

NOTE X2: The AMF supporting Onboarding shall be preconfigured with the AUSF Instance(s) used for Onboarding or AUSF Group ID(s) used for Onbaording, as specified in 5.30.2.X.

3.	SUPI; e.g. the AMF selects an AUSF instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI as input for AUSF discovery.

In the case of delegated discovery and selection in SCP, the AUSF NF consumer shall send all available factors to the SCP.

***** Next changes *****

6.3.8	UDM discovery and selection

The NF consumer or the SCP performs UDM discovery to discover a UDM instance that manages the user subscriptions.

If the NF consumer performs discovery and selection, the NF consumers shall utilize the NRF to discover the UDM instance(s) unless UDM information is available by other means, e.g. locally configured on NF consumers. The UDM selection function in NF consumers selects a UDM instance based on the available UDM instances (obtained from the NRF or locally configured).

The UDM selection functionality is applicable to both 3GPP access and non-3GPP access.

The UDM selection functionality in NF consumer or in SCP should consider one of the following factors:

1.	Home Network Identifier (e.g. MNC and MCC, realm) of SUCI/SUPI, along with the selected NID (provided by the NG-RAN) in the case of SNPN, and UE's Routing Indicator.

NOTE 1:	The UE provides the Routing Indicator to the AMF as part of the SUCI as defined in TS 23.003 [19] during initial registration. The AMF provides the UE's Routing Indicator to other NF consumers (of UDM) as described in TS 23.502 [3].

NOTE 2:	In the case of SNPN and the UE provides IMSI as SUPI/SUCI to the AMF and if SUCI/SUPI is for an SNPN served by the AMF, the AMF uses the selected NID provided by the NG-RAN together with the selected PLMN ID (from SUCI/SUPI) as the SUCI/SUPI does not always include the NID. In the case of SNPN and the UE provides NSI as SUPI/SUCI to the AMF, the AMF uses the Home Network Identifier for selection of UDM.

NOTE X2: The NF supporting Onboarding as specified 5.30.2.X, shall not use the information in the realm part of SUCI/SUPI of an Onboaridng UE for selection of AUSF.

When the UE's Routing Indicator is set to its default value as defined in TS 23.003 [19], the UDM NF consumer can select any UDM instance within the home network of the SUCI/SUPI.

2.	UDM Group ID of the UE's SUPI.

NOTE 3:	The AMF can infer the UDM Group ID the UE's SUPI belongs to, based on the results of UDM discovery procedures with NRF. The AMF provides the UDM Group ID the SUPI belongs to other UDM NF consumers as described in TS 23.502 [3].

3.	SUPI or Internal Group ID; the UDM NF consumer selects a UDM instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI or Internal Group ID as input for UDM discovery.

4.	GPSI or External Group ID; UDM NF consumers which manage network signalling not based on SUPI/SUCI (e.g. the NEF) select a UDM instance based on the GPSI or External Group ID range the UE's GPSI or External Group ID belongs to or based on the results of a discovery procedure with NRF using the UE's GPSI or External Group ID as input for UDM discovery.

In the case of delegated discovery and selection in SCP, NF consumer shall include one of these factors in the request towards SCP.

***** End of changes *****