# SA WG2 Meeting #S2-144E	S2-2102849

# 12 - 16 April, 2021, Electronic meeting

Title:	[draft] Reply LS on UE location aspects in NTN

Response to:	LS on UE location aspects in NTN (S2-2102115/R2-2102055)

Release:	Release 17

Work Item:	5GSAT_ARCH

Source:	SA2

To:	RAN2, RAN3

Cc:	SA3-LI, SA3, CT1

Contact Person:	Sherry (Yang) Shen

shenyang6@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	tbd

1. Overall Description:

SA2 thanks RAN2 for considering SA2 requirements with regards to regulatory services and for the clarification on how it is supported at NG-RAN.

SA2 would provide the following answer to the question raised by RAN2:

- Question 1: RAN2 would like to ask RAN3, SA3-LI and SA2 to confirm whether the current functionality identified above is sufficient for use in Non-Terrestrial Networks including initial registration procedure.

SA2 Answer: The assistance information as listed in R2-2102055 can provide sufficient granularity of UE location to determine the country (or province) at the present UE location for PLMN selection and for regulatory services (e.g. PSAP selection for emergency call). It is assumed that NG-RAN is able to provide User Location Information (ULI) of the UE in CM-CONNECTED state to the AMF with the granularity sufficient to determine the country where the UE is located.

Additionally, we would like to have the following issues clarified:

- Q1: Is it mandatory required that a gNB supporting satellite access constructs the Cell ID with the granularity accurate enough to determine the country where UE is located and provides it to the AMF as the ULI?

- Q2: Could the gNB supporting satellite access provide A-GNSS info to 5GC when requested?

- Q3: Since UE is only allowed to select a PLMN in the country it presents, how a UE in CM-IDLE state performs the PLMN selection?

2. Actions:

To RAN WG2 and RAN WG3.

ACTION: 	SA2 respectfully asks RAN WG2 and RAN WG3 to provide feedback on the above questions.

3. Date of Next SA2 Meetings:

SA2#145e	17th – 28th May 2021	Electronic meeting

SA2-Ad-hoc	tbd	Electronic meeting