SA WG2 Meeting #144E e-meeting	S2-2102877

April 12 – April 16, 2021, Elbonia

Source:	Qualcomm Incorporated

Title:	TS 23.304: clause 4.3.9.1 Layer-3 UE-to-Network Relay

Document for:	Approval

Agenda Item:	8.8.2

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution adds the ProSe Layer-3 UE-to-Network Relay function definition into TS 23.304.

# 1.	Introduction

Based on the KI#3 conclusion of TR 23.752, this contribution added ProSe Layer-3 UE-to-Network Relay function definition into TS 23.304 as clause 4.3.9.1.

# 2.	Proposal

It is proposed to capture the following changes into TS 23.304.

# * * * * First change * * * *

### 4.3.9.1	Layer-3 UE-to-Network Relay

Layer-3 UE-to-Network Relay entity provides the relaying functionality to support connectivity to the network for Remote UEs. It can be used for both public safety services and commercial services (e.g. interactive service).

Layer-3 UE-to-Network Relay supports the following functions:

- UE-to-Network Relay Discovery service as defined in clause 6.3.2.3, to allow discovery by the Remote UE;

- ProSe Direct Communication  as specified in clause 6.5.1, for the communication with the Remote UEs for the relay operation;

- access the 5GS as a UE as defined in TS 23.501 [x] with the enhancements as specified in clause 6.2 and 6.7;

- relays unicast traffic (uplink and downlink) between the Remote UE and the network, supporting IP, Ethernet or Unstructured traffic type;

- End-to-end QoS treatment for the Remote UE's traffic as defined in clause 5.7.2.1;

- IP address management for the Remote UE as defined in clause 5.6 in case the Remote UE uses IP traffic type;

- providing access to an N3IWF for the Remote UE when supported by the serving PLMN.

# * * * * End of changes * * * *