3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102949

Elbonia, April 12 – 16, 2021	(revision of S2-210xxxx)

Source:	Nokia, Nokia Shanghai-Bell

Title:	MB-SM Services

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This contribution

# Introduction

Requirements

TR 23.757 conclusions and flows in TS 23.247 require that the MB-SMF provides services:

- For the NEF/MBSF/AF to request TMGIs

- For the NEF/MBSF/AF to configure an MBS session

- For the SMF to obtain QoS information about the MBS session and to be notified about activation

- For the AMF to request delivery of multicast data via shared delivery towards RN nodes

Proposal: Define new Nmbsmf services

# 2. Text Proposal

It is proposed to agree the following changes for TR 23.757.

# 9	Network Function Services

Editor's note:		The NF and associating services are FFS.

### 9.X	MB-SMF Services

#### 9.X.1	General

The following table illustrates the MB-SMF Services for MBS.

Table 9.X.1-1: NF services provided by MB-SMF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Nmbsmf_Control | Create | Request/Response | NEF, MBSF, AF |
|  | Update | Request/Response | NEF, MBSF, AF |
|  | Delete | Request/Response | NEF, MBSF, AF |
| Nmbsmf_TMGI | Request | Request/Response | NEF, MBSF, AF |
|  | Release | Request/Response | NEF, MBSF, AF |
| Nmbsmf_Reception | Request | Request/Response | AMF, SMF |
|  | Release | Request/Response | AMF, SMF |
| Nmbsmf_Information | Request | Request/Response | SMF |
|  | Notify | Subscribe/Notify | SMF |
|  | Subscribe |  | SMF |
|  | Unsubscribe |  | SMF |

#### 9.X.2	Nmbsmf_Control service

##### 9.X.2.1	General

Service description: NF Service Consumer can use this service to request that an MBS service is configured, ,activated, deactivated, released, or deconfigured.

Editor's note:		Parameters of the service operations are FFS.

##### 9.X.2.2	Nmbsmf_Control_Create service operation

Service operation name: Nmbsmf_Control_Create.

Description: The NF Service Consumer requests and configures a context to control an MBS service. It can also request that the MBS session is activated.

Inputs, Required: MBS session ID

Inputs, Optional: DNN, Input Transport Address Request

Outputs, Required: Success or Failure. For Success MBS control context ID.

Outputs, Optional: Input Transport Address.

##### 9.X.2.3	Nmbsmf_Control_Update service operation

Service operation name: Nmbsmf_Control_Update.

Description: The NF Service Consumer can update the context to control an MBS service to request that the MBS session is activated, deactivated, or released.

Inputs, Required: MBS control context ID, type of update

Inputs, Optional: FFS.

Outputs, Required: Success or not.

Outputs, Optional: None.

##### 9.X.2.4	Nmbsmf_Control_Delete service operation

Service operation name: Nmbsmf_Control_Delete

Description: The NF Service Consumer requests that the context to control an MBS service is deleted. An established session is released.

Inputs, Required: MBS control context ID.

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: None.

#### 9.X.3	Nmbsmf_TMGI service

##### 9.X3.1	General

Service description: NF Service Consumer can use this service to request the allocation of TMGIs and release allocated TMGIs.

Editor's note:		Parameters of the service operations are FFS.

##### 9.X.3.2	Nmbsmf_TMGI_Request service operation

Service operation name: Nmbsmf_ TMGI_Request.

Description: NF Service Consumer can use this service to request the allocation of TMGIs.

Inputs, Required: Number of TMGIs

Inputs, Optional: None

Outputs, Required: TMGIs, Expiry Time

Outputs, Optional: None.

##### 9.X.3.3	Nmbsmf_TMGI_Release service operation

Service operation name: Nmbsmf_TMGI_Release.

Description: NF Service Consumer can use this service to request the allocation of TMGIs.

Inputs, Required: TMGIs

Inputs, Optional: FFS.

Outputs, Required: Success or not.

Outputs, Optional: None.

#### 9.X.4	Nmbsmf_Reception service

##### 9.X4.1	General

Service description: NF Service Consumer can use this service to request the reception of MBS data or to to terminate the reception of MBS data.

Editor's note:		Parameters of the service operations are FFS.

##### 9.X.4.2	Nmbsmf_Reception_Request service operation

Service operation name: Nmbsmf_Reception_Request.

Description: NF Service Consumer can use this service to request the reception of data of a multicast session

Inputs, Required: Multicast Session ID, AMF ID, RAN node ID

Inputs, Optional: Area Session ID, Unicast GTP Tunnel ID

Outputs, Required: Success or not

Outputs, Optional: Multicast GTP Tunnel ID

##### 9.X.4.3	Nmbsmf_Reception_Release service operation

Service operation name: Nmbsmf_Reception_Release.

Description: NF Service Consumer can use this service to request the termination of MBS data transmission

Inputs, Required: Multicast Session ID, AMF ID, RAN node ID

Inputs, Optional: Area Session ID

Outputs, Required: Success or not

Outputs, Optional: None.

#### 9.X.5	Nmbsmf_Information service

##### 9.X.5.1	General

Service description: NF Service Consumer can use this service to request information about an MBS session.

Editor's note:		Parameters of the service operations are FFS.

##### 9.X.5.2	Nmbsmf_Information_Request service operation

Service operation name: Nmbsmf_Information_Request.

Description: NF Service Consumer can use this service to request QoS information for a multicast session

Inputs, Required: Multicast Session ID

Inputs, Optional: Area Session ID

Outputs, Required: QoS information for multicast session

Outputs, Optional: None

##### 9.X.5.3	Nmbsmf_Information_Notify service operation

Service operation name: Nmbsmf_Information_Notify

Description: Provided by the MB-SMF to notify NF consumers of the subscribed events.

Inputs, Required: Event (Session activation), Notification Correlation Information (information to identify the application session).

Inputs, Optional: Event information (defined on a per Event ID basis).

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 9.X.5.4	Nmbsmf_Information_Subscribe service operation

Service operation name: Nmbsmf_Information_Subscribe.

Description: NF Service Consumer subscribes to notification of events

Inputs, Required: Multicast Session ID, Event Session activation)

Inputs, Optional: Area Session ID,.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID

Outputs, Optional: None

##### 9.X.5.5	Nmbsmf_Information_Unsubscribe service operation

Service operation name: Nmbsmf_Information_Unsubscribe

Description: Enable NF consumers to explicitly unsubscribe the notification.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: None.