3GPP TSG-WG SA2 Meeting #144-e 	S2-2102188r01

2021‑04‑12 -- 2021‑04‑16, e-meeting

Source: 	Ericsson, CMCC, Deutsche Telekom AG

Title: 	EPS Interworking Considerations

Document for: 	Agreement

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract of the contribution: This contribution discusses EPS Interworking aspects and propose a new Annex.

# 1 Discussion

The URSP rules in 5GC are defined to cover both NR/5GS access as well as for tight interworking with LTE/EPC access. In LTE/EPC access there is no possibility to provision new rules to the device, instead according to standards (3GPP TS 23.501, section 5.15.5.3 + section 5.17.1.2) the rules provisioned in SA-NR/5GS access are to be used also when the device is in LTE/NSA-NR access connected to the EPC.

The following text is captured in 3GPP TS 23.501, clause 5.15.5.3:

The UE shall store and use the URSP rules, including the NSSP, as described in TS 23.503 [45]

The following text is captured in 3GPP TS 23.501, clause 5.17.1.2:

If the UE does not have preconfigured rules for associating an application to a PDN connection (i.e. the UE does not have rules in UE local configuration and is not provisioned with ANDSF rules), the UE should use a matching URSP rule as defined in TS 23.503 [45], if available, to derive the parameters, e.g. APN, for the PDN connection establishment and associating an application to the PDN connection.

NOTE:      The mapping between the parameters in the URSP rules and the parameters used for PDN connection establishment is defined in TS 24.526 [110].

Based on these statements interworking with EPS do support using the URSP rules provisioned when in SA-NR coverage in 5GC. This also means that URSP rules will be used in LTE/NSA-NR coverage, if provisioned to the UE, when interworking with 5GC.

One can therefore conclude that interworking aspects may be documented in TS 23.548 to clarify the impact on the edge computing solutions for the multiple session solution. This is proposed to go into an annex of the specification.

This pCR updates TS 23.548 according to these conclusions.

# 2 Proposal

It is proposed to update TS 23.548 as follows (all new text):

*********************************Start Change (ALL NEW)********

Annex X (informative):
EPS Interworking Considerations

## X.1	General

5GC is specified to support interworking with EPC. Edge Computing deployments that use interworking need to consider the aspects outlined in this Annex.

When and if EPS interworking is applied, one need to consider that the number of PDU sessions available when UE is moving to EPS from 5GS, since the number PDN connections are limited in EPS.

NOTE:	EPS currently supports twelve bearers, but many implementations only support eight bearers.

## X.2	Distributed Anchor

SSC mode 3 cannot be used when the UE is registered in EPC as 5G-NAS is not available. Re-establishing a PDN connection after releasing a old one can be done in EPS using the "reactivation requested" cause value in EPS bearer context deactivation, if the feature is supported by EPS.

## X.3	Multiple Sessions

The URSP rules provided by 5GC to the UE are defined to cover both 5GS as well as EPS when interworking is applied. In EPSs there is no possibility to provide new URSP rules to the UE, instead according to  TS 23.501 [xx], clauses 5.15.5.3 and 5.17.1.2, the URSP rules provided to the UE when it was registered in 5GC may also be used when the UE is registered in EPC.

AF influence on URSPs may not take effect if the UE can also be on EPS, since the UE is not mandated to use the URSP rules on EPS.

*********************************Stop Change********