SA WG2 Meeting #S2-144E	S2-2102230r012

# 12 - 16 April, 2021 ; Electronic meeting

# Title:	Reply to LS on UE location aspects in NTN

# Response to:	R2-2102055/S2-2102115

# Release:	Rel-17

# Work Item:	5GSAT_ARCH

Source:	SA2

To:	RAN2

Cc:	SA3-LI, RAN3, SA3, CT1

Contact Person:

Name:	Jean Yves Fine

Tel. Number:

E-mail Address:	jean-yves.fine@thalesgroup.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 would like to thank RAN2 for their LS on UE location aspects in NTN (R2-2102055/S2-2102115).

In this LS, RAN2 is asking the following questions to which SA2 would like to provide the indicated responses below.

- Question 1: RAN2 would like to ask RAN3, SA3-LI and SA2 to confirm whether the current functionality identified above is sufficient for use in Non-Terrestrial Networks including initial registration procedure.

Answer from SA2:

SA2 confirms the requirement on UE’s location information for NG-RAN to perform appropriate Core Network selection in some scenarios and to determine Cell ID for all NGAP messages containing User Location Information.

SA2 believes that the methods indicated in the LS (mobility measurements, and/or UE position obtained from GNSS) may be sufficient to determine a CGI with sufficient accuracy to support services provided in 5GC depending on use case and the UE location accuracy.. SA2 notes that the the CGI constructed by the NTN based NG-RAN should correspond to a fixed geographical area. The accuracy requirement for the NAS procedures is comparable with a cell for TN.

SA2 further notes that it is necessary to provide an  accurate CGI to 5GC after a UE has entered CONNECTED state.

Additionally, AMF may initiate UE location procedure after registration in certain cases, e.g. emergency call procedures which may be used when an N2 provided ULI is considered insufficient.

For regulatory reasons, either network determined or network verified UE location is needed.

2. Actions:

To RAN2 group.

ACTION: 	SA2 kindly asks RAN2 to align their work with these answers.

3. Date of Next SA2 Meetings:

3GPP TSG SA2 #145 (E)	            17th – 28th May 2021

3GPP TSG SA2 #146 (E)	23rd – 27th August 2021