SA WG2 Meeting #144e	S2-2102231

# April 12 – 16, 2021; Elbonia                   		 	 				(revision of S2-210)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2707 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EASDF discovery and selection, and update of edge computing description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson, [Huawei, HiSilicon] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | EASDF discovery and selection is not specified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define EASDF discovery and selection Note that even though EASDF usage is defined in 23.548, EASDF discovery and selection is defined in 23.501 applying the same logic than for NWADF and 23.288 And adding reference to TS 23.548 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EASDF discovery and selection remains unspecified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | There is a companion PCR with the same title |  |  |  |  |  |  |  |  |

FIRST CHANGE (all text is new)

### 6.3.X	EASDF discovery and selection

Multiple instances of EASDF may be deployed in a network. NF consumers mentioned in this clause are SMF(s).

The NF consumers shall utilize the NRF to discover EASDF instance(s) unless EASDF information is available by other means, e.g. locally configured on the NF consumer. The EASDF selection function in NF consumers selects an EASDF instance based on the available EASDF instances.

The following factors may be considered by the NF consumer for EASDF selection:

-	S-NSSAI.

-	DNN

-	the N6 IP address of the EASDF and the N6 address of the PSA UPF

NOTE:	The IP address of the EASDF can be used is to select an EASDF that is “IP near” to the PSA of the PDU Session.

-	Locality (e.g. geographical location, data center) which may match the locality information for the PSA UPF

NEXT CHANGE (2)

END OF CHANGES