3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102267

Elbonia, April 12 – 16, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2716 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SNPN interface to Credentials Holder AAA Server for primary authentication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, NTT Docomo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-03-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current version of 23.501 has the following edtior’s notes: Editor’s note: Whether an intermediate function is needed between the AUSF and the AAA-S is FFS. Editor’s note: the Interface between AAA Server and SNPN is FFS. The 23.501 specifies ““Network functions within the 5GC Control Plane shall only use service-based interfaces for their interactions.” And currently the AUSF only has internal 5GC interfaces and thus is a pure SBI node and adding requirements for AAA protocols could prevent implementation of AUSF as a (micro)service in datacentres. The Network Slice-Specific Authentication and Authorization procedure (4.2.9.1 in TS 23.502) use the Network Slice Specific Authentication and Authorization Function (NSSAAF) that undertakes any AAA protocol interworking with the AAA protocol supported by the AAA server. This function provides the same functionality for AMF as AUSF would need for scenario when separate entity has AAA. Re-use of operator’s investment in NSSAAF also for this scenario is preferred. Even if a new AAA interworking function (AAA-IWF) is introduced, it could be co-located with NSSAAF for the similarities in functionality. Implementation can co-locate a new AAA interworking function and AUSF and then the standardized interface between AUSF and AAA-IWF is not exposed. If 3GPP mandates AUSF and protocol converter in same NF, it limits the vendor choices. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Due to the reasons above it is proposed to: - Introduce AAA Interworking Function (AIWF) to be used by AUSF during primary authentication. - Add service provided by AIWF that AUSF will be the primary consumer of - Remove the associated editor’s notes. Interface between SNPN and Credentials Holder AAA Server is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing new features as per conclusion of TR 23.700-07 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, 4.2.6, 4.2.7, 5.30.2.9, 5.30.2.9.1, 6.2.6.2, 6.2.X (NEW), 6.3.Y (NEW), 7.2.20 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

##### 5.30.2.9.2	Credentials Holder using AAA Server for primary authentication

The AUSF in SNPN may support primary authentication of UEs that use credentials from an AAA Server in a Credentials Holder (CH).

-	If the UDM informs AUSF that primary authentication by a AAA server in a CH is required, the AUSF shall discover and select the AAA server, and then forward EAP messages to the AAA server which is acting as the EAP Server for the purpose of primary authentication.

-	The SUPI is used to identify the UE during primary authentication towards the AAA sever. SUPI privacy is achieved according to existing methods in Annex I.5 of TS 33.501 [29].

-	After completing the primary authentication with the AAA server successfully, the AMF and SMF shall retrieve the UE subscription data from UDM using SUPI.

Figure 5.30.2.9.1-1 depicts the 5G System architecture for SNPN with Credentials Holder using AAA Server for primary authentication.

Figure 5.30.2.9.2-1: 5G System architecture with access to SNPN using credentials from Credentials Holder using AAA Server

The Interface between AAA Server and SNPN is  out of scope of 3GPP

# * * * * End of changes * * * *