3GPP TSG-SA2 Meeting #144E	S2-2102359

# April 12th - April 16th , 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2734 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Exposure of Time synchronization as a service - description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-04-12 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA2 FS_IIoT study has concluded the support for exposure of Time synchronization service and it need to be captured in TS 23.501. This CR progresses the description of time synchronization service exposure as agreed in TR 23.700-20. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Solve open ENs in clause 5.27.1.8 and progress description of time synchronization service within 23.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.8, 5.27.1.8.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

#### 5.27.1.8	Exposure of Time Synchronization

5G System supports time synchronization service that can be activated and deactivated by AF. Exposure of time synchronization comprises the following capabilities:

-	The AF may learn 5GS capabilities to support time synchronization.

-	The AF may request time synchronization service with specific requirements and supply information that can be used to configure time synchronization distribution for targeted UE(s). The AF controls activation and deactivation of the time synchronization service for the target UE(s) or provides a temporal validity condition to the request.

The NEF exposes the 5GS capabilities to support time synchronization service to the AF. To expose 5GS capabilities the NEF may be configured with the expected time synchronization accuracy within 5GS. The exposed information may include the supported time synchronization distribution methods (i.e. supported IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation modes or 5G clock sync), (g)PTP grandmaster capable, 5G Clock quality, 5G Clock accuracy.

The AF requests sent to the NEF may target a UE or multiple UEs, or any UE. The request may include the following information:

-	Combination of DNN and S-NSSAI. If the AF does not indicate a combination of DNN/S-NSSAI in the request, the NEF determines the DNN and S-NSSAI based on the AF identifier.

-	Target UE(s) Identification (optional). This may correspond to:

-	Individual UEs identified using GPSI, or an IP address/Prefix or a MAC address. If omitted, the NEF applies the request to any UE accessing the combination of DNN, S-NSSAI.

-	Requested time synchronization distribution method:

-	IEEE Std 1588-2019 [126] or IEEE Std 802.1AS [104] operation (i.e. as a Boundary Clock, peer-to-peer Transparent Clock, or end-to-end Transparent Clock or as a PTP relay instance).

-	5G clock sync. In this case, the time source is provided by the 5GS. 5G-AN provides a 5GS reference time to the UE via 3GPP radio access; UE/DS-TT may provide 5G reference timing information to end stations using implementation specific means. This method is applicable for both IP and Ethernet PDU Sessions.

-	Time synchronization accuracy: This parameter defines an upper bound for the synchronicity budget for the time distribution process within the 5GS.

-	For IEEE Std 1588-2019 [126] or IEEE Std 802.1AS [104] operation, information to be used by NEF to configure IEEE Std 1588-2019 or IEEE Std 802.1AS functionality in DS-TT or NW-TT, if applicable.

-	Temporal validity condition: This attribute is provided in the form of time interval(s) or duration(s) during which the AF request is to be applied.

Editor's Note: It is FFS which entity in 5GS is responsible to determine the validity of Temporal Validity Condition.

-	Spatial Validity Condition on the UE(s) location

Editor's Note: It is FFS which entity in 5GS is responsible to determine the validity of Spatial Validity Condition.

The NEF uses the Time Synchronization parameters received from AF. When IEEE Std 1588-2019 [126] or IEEE Std 802.1AS [104] operation have been selected, the NEF determines the necessary (g)PTP parameters to activate and control the service in DS-TT(s) and NW-TTs. For this purpose, the NEF uses the PMIC or BMIC to manage the IEEE Std 1588-2019 [126] or IEEE Std 802.1AS [104] operation in the DS-TT or NW-TT, respectively (see clause 5.27.1.4).

Editor's note:	FFS the time synchronization information provided by the AF that may be stored in the UDR and how the NEF may update or revoke information at the UDR for existing or future PDU Sessions.

For handling (g)PTP traffic, the PCF, according to PCC rule authorization, chooses a 5QI and dynamically set the PDB and/or MDBV according to requirements for (g)PTP protocol. The PCF provides the SMF with a PCC rule generated based on the AF request. The SMF may take the information in the PCC rule to may decide to modify or establish a PDU Session to create or modify or release a QoS Flow for transmitting the (g)PTP messages. The PCF acknowledges the time synchronization service request to the NEF..

## ---Start of the 2nd Change---

#### 5.27.1.8.1	Time Synchronization Assistance Information

Time synchronization assistance information describes time synchronization service characteristics for the distribution of 5G internal system clock. Time synchronization assistance information may be provided by SMF or AMF to the NG-RAN to assist the distribution of 5G internal system clock to the targeted UE(s) as described in TS 38.331 [28]. The knowledge of the time synchronization service characteristics is useful for NG-RAN to allow it to determine how to provide ReferenceTimeInfo IE (as described in TS 38.331 [28]), propagation delay compensation, time information transfer during mobility events to the targeted UE(s) within the cell.

Time synchronization assistance information may include the attributes listed in Table 5.27.1.8.1-1 on a UE or application basis. Application basis means the attribute applies to a specific time distribution service the AF configures for an individual UE or a group of UEs (e.g. one (g)PTP time domain or the 5G clock sync). UE basis means the attribute applies to the distribution of the 5G internal system clock that may map to one or multiple time synchronization processes configured at the UE(s) (e.g. multiple (g)PTP time domains).

Table 5.27.1.8.1-1: Time synchronization assistance information

| Assistance Information | Description |
| --- | --- |
| 5G clock sync indication (UE basis) | A flag that notifies the NG-RAN node the 5G clock sync is the source of time reference for the UE. |
| Time synchronization accuracy (optional, UE basis) | It refers to an upper bound for the E2E synchronicity accuracy budget supported by the 5G clock sync time distribution process. It may be provided by the AF when configuring the time synchronization error, if not provided, a value based on 5GS constrains is selected. |
| Uu synchronization error budget (optional, UE basis) | It refers to the Uu interface synchronicity accuracy budget for the 5G clock sync. |
| Temporal validity condition (optional, application basis) | It refers to time interval(s) or duration(s) during which the time synchronization process is to be applied. |

For a given synchronization accuracy requirement, the NEF provides the accuracy budget to the SMF via The SMF subtracts a preconfigured inaccuracy value for the network segment of the time distribution chain from the received accuracy budget to determine the remaining synchronization budget for Uu interface. The remaining synchronization accuracy budget is provided to the NG-RAN via AMF. If there is no network segment in the time distribution chain, the SMF can forward the received accuracy budget to the AMF without the correction.

When the AF request includes the temporal validity condition, it contains the start-time and stop-time attributes that describe the time period(s) when the time synchronization service is active and are based on the 5G system clock..

## ---End of Changes---