3GPP TSG-SA2 Meeting #144E	S2-2102363r02

# 12– 16 April 2021	(Revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2735 | rev | - | Current version: | 17.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Periodic TSC QoS Exposure - description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add description of Exposure for Periodic TSC QoS to TS23.501 to support TR23.700-20 Solution #5 adopted in TR conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clause 5.27.6 for TSC QoS Exposure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TSC QoS Exposure is not described |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** FIRST CHANGES (all new text) *****

5.27.6	Exposure for TSC QoS

Exposure for TSC QoS is intended for scenarios where endpoints require TSC but IEEE TSN is not deployed in the DN.

The AF may request QoS with specified requirements and supply information that can be used by the SMF to derive TSCAI for 5GS QoS flows.

The AF requests TSC QoS by providing QoS requirements and traffic pattern assistance parameters in the "Setting up an AF session with required QoS procedure" (see TS23.502 [3] Clause 4.15.6.6). The AF may send 5GS delay, priority, maximum TSC Burst Size and Guaranteed and Maximum Flow Bit Rate service requirements and TSC Assistance information as described in clause 5.27.2.

- The AF also provides UE identifier, AF Identifier and it may include a QoS reference. GPSI may be used to identify the individual UE in a manner similar to that used for AF influence on Traffic Routing (see clause 5.6.7).

-	The NEF authorizes requests received from an AF, determines the TSC QoS Information comprising a priority, maximum TSC Burst Size, delay and Maximum Flow Bitrate from the parameters provided by the AF and forwards them to the PCF. If traffic pattern parameters were provided by the AF, the NEF request to the PCF includes the TSC Assistance Container, which is used by the PCF and SMF as described in clause 5.27.2.

***** END OF CHANGES *****