3GPP TSG-WG SA2 Meeting #144E 	S2-2102372r02

# 12th April – 16th April 2021, Elbonia	(revision of S2-211xxxx)

Source:	Nokia, Nokia Shanghai Bell, OPPO, InterDigital

Title:	TS 23.256: UUAA Re-authentication/Re-authorization and Revocation procedure

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	ID_UAS / Rel-17

Abstract of the contribution: This contribution proposes procedures for UUAA Re-authentication/Re-authorization and UUAA Revocation.

# 1 Introduction

This pCR proposes procedures to handle the UUAA re-authentication/re-authorization for both UUAA-MM and UUAA-SM scenarios in clause 5.2.4.

This pCR also proposes procedures to handle the UUAA revocation for both UUAA-MM and UUAA-SM scenarios in clause 5.2.7.

Removed redundant clauses 5.2.2.3 and 5.2.2.4.

# 2 Proposal

It is proposed to approve the below content in the TS 23.256. All changes are marked with revision marks.

* * * * Begin of Changes * * * *

* * * * Next Change * * * *

* * * * Next Change * * * *

### 5.2.4	UUAA Re-authentication by USS/UTM

Figure 5.2.4.1-1: UAV Re-authentication procedure

UAS NF stores the UAV UEs UUAA context after successful UUAA procedure as explained in claise 5.2.2.2 for UUAA-MM and in clause 5.2.3 for UUAA-SM procedure. The UUAA context may be stored in the UDSF or may be stored locally in the UAS NF depending on deployments. The UAS NF shall also create an implicit subscription for notification towards the AMF (in UUAA-MM) or SMF (in UUAA-SM) after the successful UUAA procedure. This notification is used by the UAS NF to trigger re-authentication, update authorization data or revoke authorization of the UAV, upon receipt of such request from the USS.

1.	The USS sends a request to UAS NF for re-authentication of the UAV. The USS includes GPSI, CAA-Level UAV ID, PDU Session IP address if available in the re-authentication request.

2. 	UAS NF retrieves the UAV UE's stored UUAA context. From the stored UUAA context the UAS NF determines the target AMF or SMF for sending the notification.

3a or 3b.	The UAS NF sends Nnef_Auth_Notification request to notify the target NF, i.e. either the AMF or the SMF, to initiate re-authentication of the UAV.

4.	The UAS NF responds back to the USS indicating that re-authentication request has been successfully initiated

5.	If UE is in CM_Idle state, the target NF (i.e. either the AMF or the the SMF) initiates the Network Triggered Service Request procedures as described in TS 23.502 clause 4.2.3.3.

6a.	If  UUAA-MM was performed, the AMF initiates re-authentication of the UAV as described in steps 4c to 9 of UUAA-MM procedure, clause 5.2.2.2.

6b.	If  UUAA-SM was performed, the SMF then initiates re-authentication of the UAV as described in steps 3c to 6 of the  UUAA-SM procedure, clause 5.2.3.2.

* * * * Next Change * * * *

### 5.2.7	UUAA Revocation by USS/UTM

Figure 5.2.4.1-1: Procedure for UAV authorization revocation by USS

UAS NF stores the UAV UEs UUAA context after successful UUAA procedure as explained in claise 5.2.2.2 for UUAA-MM and in clause 5.2.3.2 for UUAA-SM procedure. The UUAA context may be stored in the UDSF or may be stored locally in the UAS NF depending on deployments. The UAS NF shall also create an implicit subscription for notification towards the AMF (in UUAA-MM) or SMF (in UUAA-SM) after the successful UUAA procedure. This notification is used by the UAS NF to trigger re-authentication, update authorization data or revoke authorization of the UAV, upon receipt of such request from the USS.

1.	The USS sends a authorization revocation request to UAS NFThe USS includes GPSI, CAA-Level UAV ID, PDU Session IP address if available in the authorization revocation request.

2. 	UAS NF retrieves the UAV UE's stored UUAA context. From the stored UUAA context the UAS NF determines the target AMF or SMF for sending the notification.

3a or 3b.	The UAS NF sends Nnef_Auth_Notification request to notify the target NF, i.e. either the AMF or the SMF that the UAV is not authorized anymore.

4.	The UAS NF responds back to the USS indicating that authorization revocation request has been successfully initiated.

5.	If UE is in CM_Idle state, the target NF (i.e. either the AMF or the the SMF) initiates the Network Triggered Service Request procedures as described in TS 23.502 clause 4.2.3.3.

6a.	If the target NF is AMF, the AMF initiates UCU procedure to invalidate the previous authorization information in the UE. The AMF shall also initiate the release of UAS service related PDU Sessions.

6b. If the target NF is AMF, based on network policy the AMF may start network initiated de-registration process as described in clause 4.2.2.3.3 in TS 23.502.

6c.	If the target NF is SMF, the SMF starts network initiated PDU session release prcoess as described in clause 4.3.4 of TS 23.502 to release the associated PDU session.

* * * * End of Changes * * * *