3GPP TSG-WG SA2 Meeting #144E e-meeting  	S2-2102411r01

# Elbonia, April 12 – 16, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2752 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | simultaneous data services from NPN and PLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To support simultaneous data services from NPN and PLMN, it needs to consider the following case: - The NPN is deployed as PNI-NPN, which relies on the PLMN When the NPN is deployed as PNI-NPN, at least two PDU Sessions should be used, one of them is established with the NPN dedicated DNN and/or dedicated S-NSSAI and the other one is not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the NPN is deployed as PNI-NPN, at least two PDU Sessions should be used, one of them is established with the NPN dedicated DNN and/or dedicated S-NSSAI and the other one is not. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear how to support simultaneous data services from PNI-NPN and PLMN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.3.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.30.3	Public Network Integrated NPN

#### 5.30.3.Y	Support simultaneous data services from PNI-NPN and PLMN

The simultaneous data service from PNI-NPN and PLMN is supported by using at least two PDU Sessions with one established with the PNI-NPN dedicated DNN and/or dedicated S-NSSAI while the other one is not.

# * * * * End of changes * * * *