3GPP TSG-WG SA2 Meeting #144E e-meeting  	S2-2102475r04

# Elbonia, April 12 – 16, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0293 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add bandwidth into Dispersion analytics per slice |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During the SA2#143E meeting, there is an FFS left for the Data Dispersion Analytics as follows: Editor Note: Consider the addition of “>>Bandwidth” into tables 6.x.3.1-3 and 6.x.3.1-4, which is the utilized bandwidth at the slice calculated by the NWDAF as the volume over the period. See also SA5 Liaison letter S2-2008361 (S5-205436). A related LS Response (S2-2101347) to the S2-2008361/S5-205436 has been approved to clarify that the utilized bandwidth at the slice can be calculated by using the Data Dispersion analytics with following formula: Utilized bandwidth = Data volume dispersed/Duration To align with the LS response, it is proposed to remove the FFS because the LS to SA5 has clarified that the Bandwidth per slice could be calculated by using the the Data Dispersion Analytics, so the OAM can just subscribe or request the Data Dispersion analytics per network slice. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An FFS is left in the TS 23.288. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.10.3.1	Data Dispersion Analysis

The data dispersion analytics results provided by the NWDAF can be UE data dispersion statistics as defined in table 6.10.3.1-1 and/or UE data dispersion predictions as defined in table 6.10.3.1-2.

Table 6.10.3.1-1: Data dispersion statistics bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Observed location statistics. |
| >> UE location | TA or cells where the UE dispersed its data. |
| >> Application ID (1..max) | To identify the application for the UE. |
| >>> Data volume dispersed | Data volume dispersed at this location and application, if application ID is provided. |
| >> Data classification | Data mobility classification for this location: fixed, camper, traveller. |
| >> Data usage ranking | Ranking of UE data usage at this location (See NOTE 2). |
| >> Data usage percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served at the location. |
| >> Ratio | Percentage of UEs in the group at the location and observation time. (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. |  |

Table 6.10.3.1-2: Data dispersion prediction bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Predicted location during the analytics target period. |
| >> UE location | TA or cells where the UE is predicted to disperse its data. |
| >> Application ID (1..max) | To identify the application for the UE. |
| >>> Data volume dispersion | Data volume dispersion prediction at this location and application, if application ID is provided. |
| >>>> Confidence | Confidence of this prediction (i.e. data to be dispersed at this location). |
| >> Data classification | Data mobility classification for this location: fixed, camper, traveller. |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this location). |
| >> Data usage ranking | Ranking of UE data usage at this location (See NOTE 2). |
| >> Data usage percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served at the location. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this location). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. |  |

The data dispersion analytics results provided by the NWDAF can also be for a UE or group of UEs data dispersion statistics at a given slice as defined in Table 6.10.3.1-3 and/or dispersion predictions as defined in Table 6.10.3.1-4:

Table 6.10.3.1-3: Data dispersion statistics bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Observed slice statistics. |
| >> S-NSSAI | Slice where the UE or group of UEs dispersed data. |
| >> Application ID (1..max) | To identify the application for the UE at the slice. |
| >>> Data volume dispersed | Data volume dispersed at this slice and application, if application ID is provided. |
| >> Data classification | Data mobility classification for slice: fixed, camper, traveller. |
| >> Data usage ranking | Ranking of UE data usage at this slice. (See NOTE 2). |
| >> Data usage percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served by the slice. |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. |  |

Table 6.10.3.1-4: Data dispersion prediction bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Predicted slice during the Analytics target period. |
| >> S-NSSAI | Slice where the UE is predicted to disperse its data. |
| >> Application ID (1..max) | To identify the application for the UE at the slice. |
| >>> Data volume dispersion | Data volume dispersion prediction at this slice and application, if application ID is provided. |
| >>>> Confidence | Confidence of this prediction (i.e. data to be dispersed at this slice). |
| >> Data classification | Data mobility classification for this slice: fixed, camper, traveller. |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this slice). |
| >> Data usage ranking | Ranking of UE data usage at this slice. See NOTE 2. |
| >> Data usage percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served by the slice. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this slice). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. |  |

NOTE:	The Application ID in the Data Dispersion Analytics is optional. When the Application ID is missing, the Data Dispersion Analytics is applied across all the applications in an AoI or a slice.

Editor's note:	Whether utilized bandwidth at a slice level (i.e. for all the UEs in the slice) could be calculated by using the Data dispersion analytics is FFS.

# * * * * End of changes * * * *