3GPP TSG-SA WG2 Meeting #144E 	S2-2102566r04

Elbonia, 12 – 16 April , 2021

Source:	Huawei, HiSilicon

Title:	Further clarification related to EAS Discovery with EASDF

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC /Rel-17

Abstract of the contribution: This contribution resolves editor’s notes in EAS discovery over Session Breakout connectivity model.

# Discussion

Resolving FFSs

In the EAS discovery procedure with EASDF, there are the following FFSs:

Editor's note:	It is FFS whether it is aligned with the usage of notification in SBI. This behaviour looks like SMF is requesting a message DNS handling rule (i.e. where to forward the DNS message or whether to include ECS option, what to include ECS option.). Normally, the Notification message is just sending ack or nack. The NF service model should be re-visited.

Editor's note:	it is FFS whether DNS message handling rule can be transferred in Notify Response message. Otherwise, SMF needs to invoke additional NF service similar to the step 6.

For these 2 FFSs, it is true that most notification responses do not carry any information, though there are exceptions. We are open on this issue. Here it is proposed to use a different message to provide ECS option/local DNS server address to EASDF.

Proposal 1: it is proposed not to carry the ECS option/local DNS server address in the response message. Instead, the SMF invokes update to provide the ECS option/local DNS server address.

DNS message handling rule

Different rule are used to describe how to handle the DNS message, e.g. DNS message forwarding rule or DNS message reporting rule, it is proposed to unify the rule for DNS message handling as the DNS message handling rule. Then the details of the DNS message handling rule is unclear.

It is proposed to further clarify what is DNS message handling rule:

The DNS message handling rule includes information used for DNS message detection and associated action(s). The information used for DNS message detection includes a DNS message detection template which are a list of parameters used by the EASNF to detect DNS messages, and the associated action(s) tell the EASNF how to handle the DNS message when the DNS message matches with the DNS message detection template.

# Proposal

It is proposed to update solution #43 as following in TS 23.458:

**************************** 1st Change ***************************

##### 6.2.3.2.2	EAS Discovery Procedure with EASDF

For the case that the UE DNS Query is to be handled by EASDF, the following applies.

-	During the PDU Session establishment procedure, the SMF selects an EASDF and provides its address to the UE as the DNS Server to be used for the PDU Session. The UE sends DNS Query to the EASDF. The SMF may configure the EASDF with DNS message handling rules to forward DNS messages of the UE to a relevant DNS server and/or report when detecting DNS messages.  The DNS message handling rule includes information used for DNS message detection and associated action(s). It is defined as following:

-	Precedence of the DNS message handling rule

-	DNS message detection template (includes at least one of the following):

-	DNS message type = DNS Query or DNS Response

-	If DNS message type = DNS Query

-  Source IP address (i.e. UE IP address)

-  Array of (FQDN ranges)

-	If DNS message type = DNS Response

-

- Array of FQDN ranges and/or array of EAS IP address ranges

-	Action(s) (include at least one of the following action):

-	Report EAS information to SMF

-	Send the DNS message to a preconfigured DNS server/resolver or an indicated DNS server as following (The indicated DNS server is included in the DNS handling rule):

-	Including the information to build optional ECS option in the DNS message (The informationfor the EASDF to build the ECS option is included in the DNS handling rule)

Editor’s Notes: it is FFS whether the information for the EASDF to build the ECS option may contain more than one IP address to deal with following cases: 1) some EAS may be deployed on the part of the DN with public IP addressing space while other EAS may be deployed on the part of the DN with private IP addressing space, 2) there may be multiple candidate L-PSA (+ULCL) UPF : for example some expensive L-PSA at the DU site related with fairly specific applications that are demanding in terms of delays and some more vanilla L-PSA + ULCL UPF that can provide traffic offload but that are more central and cheaper due to a better pooling effect in a more central location

-	Replacement of the DNS message target address with the indicated DNS Server Address; if no DNS Server Address is provided  by the SMF, then the EASDF is to forward the DNS message to a locally preconfigured DNS server/resolver

-	Buffer the DNS message and report to the SMF

-	Send the buffered DNS response message to UE

When the EASDF forwards a DNS request, it shall always ensure it receives the DNS answer (putting its own address as the source of the request)

The SMF may use following information to create DNS message handling rules associated with a PDU session:

-	 Local configuration associated with the (DNN, S-NSSAI) of the PDU Session,

-	AF influenced Traffic Steering Enforcement Control information within PCC rules, and/or

-	Information derived from the UE location such as a list of candidate L-PSA

The SMF may provide in a DNS message forwarding rule one or multiple IP addresses to be used by the EASDF to create DNS ECS option(s)

NOTE: 	For example, the SMF can derive these IP addresses based on the N6 IP address(es) associated with candidate L-PSA(s) in the NRF.

NOTE: 	Providing in DNS ECS option an IP address associated with the L-PSA UPF protects the privacy of the (IP address of the) UE.

-	If the FQDN in a DNS Query matches the FQDN(s) provided by the SMF, based on instructions by SMF, one of the following options is executed by the EASDF:

Editor's note:	It is FFS how non-UE specific EAS's FQDN information is configured with the SMF.

-	Option A: The EASDF includes the EDNS Client Subnet (ECS) option into the DNS Query message as defined in RFC 7871[6], and sends the DNS Query message to the DNS server for resolving the FQDN. The DNS server may resolve the EAS IP address considering the ECS option, and sends the DNS Response to the EASDF.

The SMF may provide in a DNS message handling rule IP address information  to be used by the EASDF to create DNS ECS options;.

-	Option B: The EASDF sends DNS Query message to a Local DNS server which is responsible for resolving the FQDN within the corresponding Local DN.  The EASDF receives the DNS Response message from the Local DNS server.

NOTE 1:	Option B does not support the scenario where the EASDF has no direct connectivity with the local DNS servers.

The SMF instructions for a matching FQDN may as well indicate EASDF to contact SMF. SMF then provides the EASDF with a DNS message handling rule.

-	If the DNS Query from the UE does not match a DNS message handling rules set by the SMF, then the EASDF may simply forward the DNS Query towards a preconfigured DNS server/resolver for DNS resolution.

-	When the EASDF receives a DNS Response message, the EASDF may notify the EAS information (i.e. EAS IP address(es) , optionally the EAS FQDN and optionally the corresponding IP address within the ECS DNS option) to the SMF if the DNS message reporting condition (i.e. the EAS IP address or FQDN is within the IP/FQDN range) provided by the SMF is met. The SMF may then trigger UL CL/BP and L-PSA insertion as specified in clause 6.3.3 in TS 23.501 [2] based on the Notification.

The information to build the ECS option or the Local DNS server address provided by the SMF to the EASDF are part of the DNS message handling rules to handle DNS queries from the UE. This information is related to candidate DNAI (s) for that FQDN(s) for the UE location.. The SMF may provide DNS message handling rules to handle DNS queries from the UE to the EASDF when the SMF establishes the association with the EASDF for the UE, and may update the rules at any time when the association exists. For the selection of the candidate DNAI for an FQDN for the UE, the SMF may consider the UE location, network topology and information of EAS deployment received as part of PCC rules. After the UE mobility, if the provided Information for ECS option or the Local DNS server address needs be updated, the SMF may send an update to DNS message forwarding rules to the EASDF.

Editor's note:	The sentence above means that the EASDF gets the FQDN(s) from SMF. How the SMF getting the FQDN(s) with DNS related information e.g. the ECS option or the Local DNS server address needs to be clarified in AF influence on traffic routing procedure specified in TS 23.502 [3].

Once the UL CL/BP and L-PSA have been inserted, the SMF may decide that the DNS messages for the FQDN are to be handled by local DNS resolver/server from now on. This option is further described in clause 6.2.3.2.3.

Figure 6.2.3.2.2-1: EAS discovery procedure with EASDF

1.	UE sends PDU Session Establishment Request to the SMF as shown in step 1 of clause 4.3.2.2.1 of TS 23.502 [3].

During the PDU Session Establishment procedure, the SMF selects EASDF as described clause 6.3 of TS 23.501 [2].

The SMF includes the IP address of the EASDF as DNS server in PDU Session Establishment Accept message as in step 11 of clause 4.3.2.2.1 of TS 23.502 [3]. The UE configures the EASDF as DNS server for that PDU Session.

2.	The SMF invokes Neasdf_DNSContext_Create Request (UE IP address, callback URI , DNS message handling rules) to the selected EASDF.

This step is performed before step 11 of PDU Session Establishment procedure in clause 4.3.2.2.1 of TS 23.502 [3].

The EASDF creates a DNS context for the PDU Session, and stores the UE IP address, the callback URI and DNS message handling rule(s) into the context.

-

The EASDF is provisioned with the DNS message handling rule(s), before the DNS Query message is received at the EASDF or as a consequence of the DNS Query reporting.

3.	The EASDF invokes the service operation Neasdf_DNSContext_Create Response () and with information allowing later the SMF to update or delete the context.

Editor's note:	How to guarantee that the UE uses the EASDF's IP address for the subsequent DSN Query in step 8 is FFS.

5.	The SMF may invoke Neasdf_DNSContext_Update Request (PDU Session Context ID,  DNS message handling rules) to EASDF. The update may be triggered by UE mobility, e.g. when UE moves to a new location, or by a reporting by EASDF of a DNS Query with certain FQDN, or, the update may be triggered by insertion/removal of Local PSA, e.g. to update rules to handle DNS messages from the UE or by new PCC rule information.

Editor's note:	It is FFS whether and how the AF-triggered DNS server information change trigger DNS context update procedure.

6.	The EASDF responds with Neasdf_DNSContext_Update Response.

7.	The UE sends DNS Query message to the EASDF.

If the EASDF can determine the information to build ECS option or local DNS server address based on the rules received from step 3 or 6, steps 8-9 are skipped.

8.	If the DNS Query message matches a DNS message handling rule for reporting, the EASDF sends the DNS message report to SMF by invoking Neasdf_DNSContext_Notify Request.

9.	The SMF responds with Neasdf_DNSContext_Notify Response.

10.

Editor's note:	it is FFS whether UL CL / BP insertion should be mentioned at this step

11. If DNS message handling rule for the FQDN received in the report need to be updated, e.g. provide updates to information to build the ECS option information, the SMF invokes Neasdf_DNSContext_Update Request (, DNS message handling rules) to EASDF.

Editor's note:	It is FFS whether EASDF Session ID is needed in step 11

For Option A, the DNS handling rule includes corresponding IP address to be used to build the ECS option. For Option B, the DNS handling rule includes corresponding local DNS Server IP address. The EASDF may as well be instructed by the DNS handling rule to simply forward the DNS Query to a pre-configured DNS server/resolver.

12. The EASDF responds with Neasdf_DNSContext_Update Response.

13.	The EASDF handles the DNS Query message received from the UE as the following:

-	For Option A, the EASDF adds the ECS option into the DNS Query message as specified in RFC 7871[6] and sends it to C-DNS server;

-	For Option B, the EASDF sends the DNS Query message to the Local DNS server.

If no DNS message detection template within the DNS message handling rule provided by the SMF matches the requested FQDN in the DNS Query, the EASDF may simply send a DNS Query to a pre-configured DNS server/resolver.

14.	EASDF receives DNS Responses from the DNS system and determines that a DNS Response can be sent to the UE.

15.	The EASDF may send a DNS message reporting to the SMF by invoking Neasdf_DNSContext_Notify request including EAS information if the EAS IP address or the FQDN in the DNS Response message matches the reporting condition provided by the SMF. The DNS message reporting may contain multiple EAS IP address if the EASDF has received multiple EAS IP address(es) from the DNS servers it has contacted.

Per the received DNS message handling rule, the EASDF does not send the DNS Response message to the UE but waits for SMF instructions (in step 16 or step 18), i.e. buffering the DNS Response message.

16.	The SMF invokes Neasdf_DNSContext_Notify Response service operation.

17.	The SMF may perform UL CL/BP and Local PSA selection and insert UL CL/BP and Local PSA.

Based on received EAS information received from the EASDF and other UPF selection criteria, as specified in clause 6.3.3 in TS 23.501 [2], the SMF may determine the DNAI based on EAS information received from EASDF and determine the associated N6 traffic routing information for the DNAI. The SMF may perform UL CL/BP and Local PSA selection and insertion as described in TS 23.502 [3].

18.	The SMF invokes Neasdf_DNSContext_Update Request (EASDF Session ID, DNS message handling rules).

Editor's note:	It is FFS whether EASDF Session ID is needed in step 18. More generally the usage of EASDF Session ID throughout the procedure is FFS

The  DNS message handling rule indicates the EASDF to send a DNS Response received in Step 12 to UE.

19.	The EASDF responds with Neasdf_DNSContext_Update Response.

20.	The EASDF sends the DNS Response to UE.

********************************* End Change ****************************