SA WG2 Meeting #144E e-meeting	S2-2102607

Elbonia, April 12 – 16, 2021

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2675 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add early admission control mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  |  | Date: |  | 2021-03-29 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the description in TS 23.502, when EAC mode is active, the AMF triggers slice availability check and update procedure during the registration and UCU procedures. When EAC mode is deactive, the AMF triggers the procedure after the registration and UCU procedures. The description in clause 5.15.11.1 only reflects the AMF behavior in one case, i.e. when EAC mode is active. So it is proposed to add the description of AMF behavior when EAC mode is inactive. To make the specification clear, the description of EAC mode is also added. Furthermore, based on the description in TS 23.502, the AMF does not trigger slice availability check and update procedure during the NSSAA procedures, but during or after the UCU procedure which is result from NSSAA procedures. So it is propose to simplify the description in clause 5.15.11.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description of early admission control mode and align the AMF behaviour with TS 23.502 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between TS 23.501 and TS 23.502. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

### 5.15.11	Network Slice Admission Control Function

#### 5.15.11.0	General

The Network Slice Admission Control Function (NSACF) monitors and controls the number of registered UEs per network slice for the network slices that are subject to Network Slice Admission Control (NSAC).

The NSACF is configured with the maximum number of UEs per network slice which are allowed to be served by each network slice that is subject to NSAC.

#### 5.15.11.1	Maximum number of UEs per network slice admission control

The NSACF controls (i.e. increase or decrease) the current number of UEs registered for a network slice so that it does not exceed the maximum number of UEs allowed to register with that network slice. The NSACF also maintains a list of UE IDs registered with a network slice that is subject to NSAC. When the current number of UEs registered with a network slice is to be increased, the NSACF first checks whether the UE Identity is already in the list of UEs registered with that network slice and if not, it checks whether the maximum number of UEs per network slice for that network slice has already been reached.

The AMF interacts with NSACF regarding the number of UEs per network slice admission control whenever the UE registration status for a network slice subject to NSAC changes.

#### 5.15.11.x	Early Admission Control mode

When Early Admission Control (EAC) mode is activated, the AMF is required to perform availability check and update procedure for the number of UEs per network slice before the S-NSSAI subject to NSAC is included in the Allowed NSSAI and sent to the UE.

The NSACF determines whether to activate or deactivate the EAC mode for a network slice based on the status of network slice, e.g. the number of UEs registered with the network slice subject to NSAC crosses a certain threshold.

* * * * End of Changes * * * *