SA WG2 Meeting #144E e-meeting	S2-2102680r04

Elbonia, 12 – 16 April 2021	(was S2-210xxxx)

Source:	Qualcomm Incorporated

Title:	Issues with DNS based EAS (re-)discovery: solution

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract: During SA2#143E, S2-2100745 proposed considerations on the User/Application/ HLOS overwriting the DNS settings provided by the MNO to the UE. This issue, if not solved, can potentially invalidate several optimizations introduced in the Rel-17 eEDGE_5GC work. This paper proposes a way forward.

# 1.	Discussion

As discussed in S2-2102679, it is essential to address the issue of application/user interface/HLOS overriding the DNS settings provided by the MNO to the UE.

# 2. 	Text proposal

It is proposed to agree the following changes vs. TS 23.548:

>>>>BEGINNING OF CHANGES<<<<

## 6.2	EAS Discovery and Re-discovery

### 6.2.1	General

Editor's note:	This clause describes general parts including e.g. privacy considerations, which DNS properties that are enabling DNS based Edge AS Discovery, recommendations/limitations for cases that OS/user overrides DNS setting.

In order for a UE to use the enhancements for EAS discovery and re-discovery it is essential that the DNS Query originated by the UE is sent to the DNS server/resolver or EASDF indicated by the HPLMN.

The solutions in this specification (e.g., EAS (re-)discovery) are applicable if the UE or the UE application client or both are aware of the usage of Edge Computing.

Based on the type of deployment used by the operator, different solutions apply:

1.	Edge Computing service is provided over dedicated PDU sessions. This approach requires the use of Edge-specific DNN(s)/S-NSSAI(s). In this case the HPLMN indicates to the UE that the used PDU session is Edge-specific. All the DNS queries sent over such DNN/S-NSSAI are directed to the proper DNS server/resolver or EASDF.

2.	Edge Computing service is provided over generic PDU sessions shared with other services (Edge as an internet service). This approach does not require the use of Edge-specific DNN(s)/S-NSSAI(s). In this case it is assumed that applications agreed with the HPLMN that the DNS queries are sent to the HPLMN Indicated DNS server/resolver or EASDF and that such indication cannot be overwritten by the user (via, e.g., user interface of the UE).

When indicating to the UE the address of the DNS server/resolver or of the EASDF, the HPLMN also indicates that such address is associated to Edge supporting features. Once received the UE shall forward such indication to the application layer and HLOS.

Editor's Note: How to enforce that the UE application and HLOS use the HPLMN's indicated DNS server/resolver of EASDF in case of DNS over HTTPS (DoH) or DNS over TLS (DoT) is FFS.

Editor's Note: How the 5GS enforces or can be notified whether the DNS Query is sent to the proper DNS server/resolver/EASDF is FFS.

>>>>END OF CHANGES<<<<