SA WG2 Meeting #144e	S2-2102692r03

# 12-16 April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0306 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ADRF functional description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Description of ADRF and ADRF services is missing from TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add section 5B to specify ADRF and section 10 to specify ADRF services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Description of ADRF is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (NEW) 5B, (NEW) 5B.1, (NEW) 10, (NEW) 10.1, (NEW) 10.2, (NEW) 10.2.1, (NEW) 10.2.2, (NEW) 10.2.X, 10.2.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (ALL NEW TEXT)

# 5B Analytics Data Repository Functional Description

## 5B.1	General

The ADRF offers services that enable a consumer to store and retrieve data and analytics. The analytics are produced by the NWDAF as described in clause 6.1, and data are collected as described in clause 6.2.

Data may be stored in the ADRF by:

- a consumer sending the ADRF an Nadrf_DataManagement_StorageRequest containing the data or analytics to be stored. The ADRF response provides a result indication.

Data may be retrieved from the ADRF by:

- a consumer sending an Nadrf_DataManagement_RetrievalRequest request to the ADRF to retrieve data or analytics for a specified data or analytics collection time window. The ADRF determines the availability of the data or analytics in its repository and sends in the response to the consumer either the data or analytics, or instructions for fetching the data or analytics.

Data may be deleted from the ADRF by:

- a consumer sending an Nadrf_DataManagement_Delete request.

An ADRF may be configured to register the data it is collecting with a DCCF. The registration uses the Ndccf_ContextManagement service specified in clause 8.2.3. The registration may subsequently be used by the DCCF to obtain data from the ADRF as described in clause 6.2.6.3.6

NEXT CHANGE (ALL NEW TEXT)

# 10	ADRF Services

#### 10.1	General

Table 10.1-1 shows the ADRF services and ADRF service operations.

Table 10.1-1: NF services provided by ADRF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadrf_DataManagement | StorageRequest | Request / Response | DCCF, NWDAF |
|  | RetrievalRequest | Request / Response | DCCF, NWDAF |
|  | Delete | Request / Response | DCCF, NWDAF |

Editor’s Note: Alignment of the ADRF services and service operation names is needed throughout TS 23.288.

Editor’s Note: Other NF consumer(s) for the ADRF services is FFS.

#### 10.2	Nadrf_DataManagement service

### 10.2.1	General

Service Description: This service enables the consumer to store, retrieve, and remove data or analytics from an ADRF.

### 10.2.2	Nadrf_DataManagement_StorageRequest service operation

Service operation name: Nadrf_DataManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store data or analytics. Data or analytics are provided to the ADRF in the request message.

Editor’s Note: Inputs and Outputs for Nadrf_DataManagement_StorageRequest service operation need to be defined.

### 10.2.X	Nadrf_DataManagement_RetrievalRequest service operation

Service operation name: Nadrf_DataManagement_RetrievalRequest

Description: The consumer NF uses this service operation to retrieve stored data or analytics from the ADRF. The Nadrf_DataManagement_RetrievalRequest response either contains the data or analytics, or provides instructions for fetching the data or analytics.

Editor’s Note: Inputs and Outputs for Nadrf_DataManagement_RetrievalRequest service operation need to be defined.

### 10.2.Y	Nadrf_DataManagement_Delete

Service operation name: Nadrf_DataManagement_Delete

Description: This service operation instructs the ADRF to delete stored data.

Editor’s Note: Inputs and Outputs for Nadrf_DataManagement_Delete need to be defined.

END OF CHANGES