3GPP TSG-WG SA2#144E Meeting emeeting 	S2-2102900r03

# emeeting, Apr 12-Apr 16, 2021	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2840 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Packet Loss Rate Measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is an issue for the counting start. The UE sends a PMF-PLR Count Request message. The UPF starts counting of the received UL packets over the target QoS Flow which the PMF-PLR Count Request message was received from. The UE starts counting the transmitted UL packets over the target QoS Flow and access network when it receives a PMF-PLR Count Response message from UPF. The counting result in the UE side may not be same as UPF side because of some UL packets are sent to UPF during the period of PMF-PLR Count Request message and PMF-PLR Count Response message but the UE have not started to count. Same issue for the counting stop. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UE start counting when it sends the PMF-PLR Count Request and stop the counting when it sends the PMF-PLR Report Request. . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Packet Loss Rate Measurements per QoS flow is not accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.5.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.32.5.2a	Packet Loss Rate Measurements

The UE and the UPF may decide to estimate the Packet Loss Rate (PLR) for an SDF over both accesses. For example, the UE may take this decision when an ATSSS rule in the UE requires the traffic of an SDF to be steered in accordance with a PLR-based threshold condition (e.g. PLR < 2%).

The UE and the UPF calculate the PLR for an SDF by exchanging PMF-PLR Report messages, as specified below. A PMF-PLR Report message is sent over 3GPP access or over non-3GPP access, using either the default QoS flow or a "target" QoS flow, as specified in clause 5.32.5.1.

The calculation of the PLR by the UE and by the UPF is based on the following mechanism. It is assumed that the PLR should be calculated for a target QoS flow, however, the same mechanism applies when the PLR should be calculated for the default QoS flow.

-	The UE requests from UPF to start counting the number of received UL packets by sending a PMF-PLR Count Request message over the target QoS flow. The UPF starts counting of the received UL packets over the target QoS Flow and over the access network which the PMF-PLR Count Request message was received from. The UE starts counting the transmitted UL packets over the target QoS Flow and access network when it sends a PMF-PLR Count Request message from UPF.

-	The UE requests from UPF to report the number of counted UL packets by ending a PMF-PLR Report Request message over the target QoS flow. The UPF stops the counting and sends a PMF-PLR Report Response message over the QoS flow including the number of UL packets counted since it received the last PMF-PLR Count Request message.

NOTE 1:	A PMF-PLR Report Request message can also indicate to UPF to start counting packets if the UE wants to measure the Packet Loss Rate again.

-	The UE calculates the UL packet loss ratio based on the local counting result of the number of transmitted UL packets and reported number of received UL packets in the UPF.

-	The UPF applies the same procedure for calculating the DL PLR

-	When the UP connection of the MA PDU session is deactivated on an access, no PMF-PLR messages are sent on this access. The PMF in the UPF shall not send PMF-PLR message on this access if the UP connection is not available or after it receives notification from the (H-)SMF to stop sending the PMF-PLR message on this access.

-	The UE and the UPF derive an estimation of the average PLR per QoS Flow over an access type by averaging the PLR measurements obtained over this access.

NOTE 2:	PLR measurements may not provide accurate loss rate because it does not consider e.g. buffering in lower layer, out of order delivery packets, etc.

# * * * * end of change * * * *