3GPP TSG-SA WG2 Meeting #144E	S2-2102938

# E-Meeting, 12th – 16th April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2853 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SNPN UE configuration and subscription aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Two corrections: - UE configuration section states the following: An SNPN-enabled UE is configured with the following information for each subscribed SNPN: Last part is not needed as credentials may be assigned either by the SNPN or by the CH. Also, in case the credentials are assigned by the CH, UE need not repeat storage of the same credential for every SNPN. Furthermore, the following is an optional feature that is supported on top: - UE supports access to an SNPN using credentials from a Credentials Holder Thus, indented the related bullet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove reference to „each subscribed SNPN“ and indent the bullet related to CH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Bullets that can apply to architecture with SNPN only, SNPN and CH are limited to SNPN only arch. – unintended restriction, limitation exists in the text. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.3, 5.30.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******Start of changes*******

#### 5.30.2.3	UE configuration and subscription aspects

An SNPN-enabled UE is configured with the following information for each subscribed SNPN:

-	PLMN ID and NID of the SNPN assigning the credential for the UE;

-	Subscriber identifier (SUPI) and credentials;

-	Optionally, an N3IWF FQDN and an identifier of the country where the configured N3IWF is located;

-	Optionally, if the UE supports access to an SNPN using credentials from a Credentials Holder:

-	User controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of GINs.

The Credentials Holder controlled prioritized lists of preferred SNPNs and GINs can be updated by the Credentials Holder.

Editor's note:	Whether the Credentials Holder controlled lists can be updated using the UE Parameters Update (UPU) via UDM Control Plane Procedure as defined in TS 23.502 [3] clause 4.20 or the Steering of Roaming (SoR) procedure as defined in TS 23.122 [17] Annex C will be determined based on feedback from CT WG1 and SA WG3.

A subscriber of an SNPN is either:

-	identified by a SUPI containing a network-specific identifier that takes the form of a Network Access Identifier (NAI) using the NAI RFC 7542 [20] based user identification as defined in TS 23.003 [19] clause 28.7.2. The realm part of the NAI may include the NID of the SNPN; or

-	identified by a SUPI containing an IMSI.

In the case of access to an SNPN using credentials owned by a Credentials Holder as specified in clause 5.30.2.9.3, the SUPI shall also contain identification for the Credentials Holder (i.e. the realm in the case of Network Specific Identifier based SUPI or the MCC and MNC in the case of an IMSI based SUPI).

NOTE 1:	When Credentials Holder is an SNPN, and the MCC and MNC of the SNPN is not unique, then IMSI based SUPI is not supported as the MCC and MNC need not be unique always; instead USIM credentials are supported using Network Specific Identifier based SUPI.

NOTE 2:	Network Specific Identifier are not supported for the case the Credentials Holder is provided by a PLMN.

An SNPN-enabled UE that supports access to an SNPN using credentials from a Credentials Holder and that is equipped with a PLMN subscription may additionally be configured with the following information for SNPN selection and registration using the PLMN subscription in SNPN access mode:

-	User controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of preferred GINs.

The Credentials Holder controlled prioritized lists of preferred SNPNs and GINs can be updated by the Credentials Holder.

Editor's note:	Whether the Credentials Holder controlled lists can be updated using the UE Parameters Update (UPU) via UDM Control Plane Procedure as defined in TS 23.502 [3] clause 4.20 or the Steering of Roaming (SoR) procedure as defined in TS 23.122 [17] Annex C will be determined based on feedback from CT WG1 and SA WG3.

##### 5.30.2.4.2	Automatic network selection

NOTE 1:	If the UE has multiple SNPN subscriptions it is assumed that the subscription to use for automatic selection is determined by implementation specific means prior to network selection.

For automatic network selection the UE selects and attempts registration on available and allowable SNPNs in the following order:

-	the SNPN the UE was last registered with (if available);

-	the SNPN identified by the PLMN ID and NID for which the UE has SUPI and credentials.;

-	If the UEs supports access to an SNPN using credentials from a Credentials Holder then the UE continues by selecting and attempting registration on available and allowable SNPNs which broadcast the indication that access using credentials from a Credentials Holder is supported in the following order:

-	SNPNs in the user controlled prioritized list of preferred SNPNs (in priority order);

-	SNPNs in the Credentials Holder controlled prioritized list of preferred SNPNs (in priority order);

-	SNPNs, which additionally broadcast a GIN contained in the Credentials Holder controlled prioritized list of preferred GINs (in priority order);

NOTE 2:	If multiple SNPNs are available that broadcast the same GIN, the order in which the UE selects and attempts a registration with those SNPNs is implementation specific.

-	SNPNs, which additionally broadcast an indication that the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN, i.e. the broadcasted NID or GIN is not present in the Credentials Holder controlled prioritized lists of preferred SNPNs/GINs in the UE.

NOTE 3:	If multiple SNPNs are available that broadcast the indication that the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN, the order in which the UE selects and attempts a registration with those SNPNs is implementation specific.

When a UE performs Initial Registration to an SNPN, the UE shall indicate the PLMN ID and NID as broadcast by the selected SNPN to NG-RAN. NG-RAN shall inform the AMF of the selected PLMN ID and NID.