3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102944r01

Elbonia, April 12 – 16, 2021	(revision of S2-210xxxx)

Source:	Nokia, Nokia Shanghai-Bell, Ericsson

Title:	Completing description of location dependent services

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This contribution

- Introduction

R01: Updated the discussion and merge content from S2-21002283, S2-21002353 and S2-21002436

The following conclusions for key issue 6 in TS 23.757 are not yet fully captured:

-	If different location-dependent content is provided for an MBS session, the network shall be able to support multiple MB-UPFs as ingress points for an MBS session.

NOTE 1:	The solution for different location-dependent content can also be applied in a scenario where the same content is provided in distributed islands of the network to enable distinct local distribution trees with local ingress points.

-	Information about the location dependent multicast service may be provided by an AF, or may be configured.

-	For location dependent MB service, the MB-SMF allocates the Area Session identifier. The Area Session identifiers allocated by different MB-SMFs shall not overlap for a multicast session.

It is already captured that different MB-UPFs are assigned. This is to allow different ingress points for different location-dependent data sources, and thus different AFs for different location areas within a multicast session should also be allowed to register themselves as data sources

At SA2#143E there was agreement that ETSUN will not be supported in REl-17, although the related PCR to capture this in 29.757 was noted due to disagreement about related implications.

Preliminary discussions identified are some likely 5MBS impacts of ETSUN:

- I-SMF updates, e.g. to forward UE join requests for multicast

- When UE moves between different SMF service areas, how can the MB-SMF in the new SMF service are be selected, e.g. how the Anchor-SMF selects MB-SMF in the visiting region?

There is agreement that this will not be addressed.

However, there is a disagreement on how to document the limitation when multiple SMF service areas are deployed in  in Rel-17:

The following Discussion is from S2-21002283:

In SA2#143E, “Support of deployments topologies with specific SMF Service Areas” is moved out from Rel-17. As a result, location dependent service is not supported if the UE moves from one SMF service area to another. Below is an example:

In the example (see Figure 1),

-	the SMF Service Areas belonging to different regions managed by different subsidiaries, therefore separate MB-SMF and MB-UPF must be deployed for each region.

-	UE originally joins the MBS Session in Region A and shared delivery is applied.

-	When UE moves from Region A to Region B, the SMFA in Region A is not able to discover MB-SMFB as SMFA is not supposed to have knowledge of SMF service area of Region B.

Figure 1 UE moves from Region A to Region B, Shared Delivery in Region B cannot be established

Based on the above,

[Proposal-1] It is proposed to add a note that location dependent service is not supported if a UE moves from one SMF service area to another.

For location dependent service in different MBS areas within the same SMF service area, if multiple MB-SMFs for an MBS Session is required, it means that TMGI needs to be coordinated among multiple MB-SMFs with the same SMF service area, which will increase complexity both in 5GC and in the AS. For simplicity,

[Proposal-2] For location dependent service in different MBS areas within the same SMF service area, it is proposed that one MB-SMF be used for one MBS Session. Multiple MB-UPFs are used to transfer different content data in different MBS area.

2. Text Proposal

It is proposed to agree the following changes for TR 23.757.

## 6.2	Local MBS service

A Local MBS service is an MBS service provided in one or several MBS service area(s). An MBS service area is identified by a cell list or a tracking area list. Only UEs within the MBS service area may receive content data, while UEs outside the MBS service area are not allowed to receive location specific content.

For multicast communication, UEs outside the MBS service area are not allowed to join the MBS service, and the network shall not deliver location specific content anymore to the UEs moving out of the MBS service area. During UE joins into a local multicast session, the multicast service area information of the multicast session is provided to the UE via NAS message. When the join fails due to outside the multicast service area, the UE does not join to the multicast session again until the UE moves inside the multicast service area. When the join succeeds, the UE does not perform monitoring the information related to the multicast session identified by an MBS Session ID over the radio if outside the multicast service area.

A location dependent MBS is a local MBS that is provided in several MBS service areas. The location dependent MBS service enables distribution of different content data to different MBS service areas. The same MBS Session ID is used but a different area session ID is used for each MBS area. The area session ID is used, in combination with MBS Session ID, to uniquely identify the MBS service within 5GS. The network supports the location-dependent content distribution for the location dependent MBS services, while UEs are only aware of the MBS service. When UEs move to a new MBS service area, content data from the new MBS service area shall be delivered to the UE, and the network ceases to deliver the content data from the old MBS service areas to the UE. However, when the UE moves from non-local service area (i.e. not using Area Session ID), the network may keep to deliver the location-independent content data to the UE.

Information about different location areas for a location dependent multicast service may be provided by one or several AFs,.

NOTE 1: In this release, deployments topologies with specific SMF Service Areas is not supported, as a result, location dependent service using multicast communication is not supported when a UE moves from one SMF service area to another.

NOTE 2:	An example of Location-dependent MBS is a nationwide weather forecast service with local weather reports.

NOTE 3:	Area Session ID is equivalent to Flow ID as specified in TS 23.246 [8].

### 7.3.4	Support for Local Broadcast Service and Location-dependent Broadcast Service

Editor's note:	This clause needs a revisit, e.g. the content can be moved to clause 7.2.1 where the steps are described.

Local broadcast service and location-dependent broadcast service are described in clause 6.2. For broadcast sessions, MBS service area is always required.

Different AFs may provide contents for different location areas, and different ingress points for the broadcast session are supported for different location areas. For location dependent service provided in different MBS areas belonging to different SMF service areas, different MB-SMFs and MB-UPFs  are required for a broadcast session.

The area session ID is allocated by MB-SMF in MBS Session Establishment procedure. MB-SMF needs to further ensure there is no area overlapping for service areas that share the same TMGI.

NOTE 1: For location dependent service provided in different MBS areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session.