3GPP TSG-SA WG2 Meeting #144E (e-meeting)	S2-2102953r01

# April 12 – April 16, 2021, Elbonia											 (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2854 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Slice Admission Control - EPC interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following conclusion was agreed in clause 8.1 and 8.2 of TR 23.700-40: NOTE 2: The detailed operation for the network slice access control with the consideration of different access types and EPC interworking will be considered during the normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | EPC interworking aspects for network slice admission control shall be specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EPC interworking aspects for network slice admission are not covered by the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 5.15.11.x	Network slice admission control during EPC interworking

Network Slice Admission Control for maximum number of UEs and for maximum number of PDU Sessions per network slice is performed at the time of EPC to 5GC interworking procedure.

The NSACF controls (i.e. increases or decreases) the number of registered UEs per network slice based on request from the AMF when the UE performs mobility Registration procedure from EPC to 5GC. If the selected S-NSSAI by the AMF is subject to the NSAC, the AMF triggers a request to NSACF for maximum number of UEs per network slice admission control as per clause 5.15.11.1.

The NSACF controls (i.e. increases or decreases) the current number of PDU Sessions per network slice based on request from the SMF+PGW-C when the UE's PDN connections are moved from EPC and 5GC. If the selected S-NSSAI by the SMF+PGW-C is subject to the NSAC, the SMF+PGW-C triggers a request to NSACF for maximum number of PDU sessions per network slice control during PDU session establishment/release procedures as per clause 5.15.11.x.

Based on operator policy, the AMF and SMF+PGW-C may indicate to NSACF the control request was triggered due to EPC to 5GC interworking.

Editor’s Note: it is FFS how the NSACF should treat a request when the EPC to 5GC interworking indication is present. For example, if the operator uses a unified quota for 5GC and EPC interworking, the NSACF could treat a request with EPC to 5GC interworking indication with priority over requests without this indication. Alternatively, the operator can use separate quotas for 5GC, and EPC to 5GC interworking scenarios. In yet another alternative, the operator can decide for the NSACF to always honour requests with EPC to 5GC interworking indication.

* * * * End of Change * * * *