3GPP TSG-SA2 Meeting #145E	S2-2103817

# Electronic,  May 17 – May 28, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0388 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User consent to data collection and analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Conclusions for KI#15 are not validated or confirmed by SA3 yet, this WG said that their SID is in an early stage, therefore we propose to add a NOTE that acknowlege the fact that user consent is needed and that SA3 will defined it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A note is added in the general description of the Analytics functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions for KI#15 are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------First Change---------------------------------------------

## 5.1	General

The NWDAF provides analytics to 5GC NFs, and OAM as defined in clause 7. An NWDAF can be decomposed into:

-	Analytics logical function (AnLF): An NWDAF containing the Analytics logical function, denoted as NWDAF(AnLF), can perform inference, derive analytics information (i.e. derives statistics and/or predictions based on Analytics Consumer request) and expose analytics service i.e. Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo.

-	Model Training logical function (MTLF): An NWDAF containing the Model Training logical function, denoted as NWDAF(MTLF), trains ML models and exposes new training services (e.g. providing trained model) as defined in clause 7.

NOTE 1:	NWDAF can contain Model Training logical function, Analytics logical function, or both. An NWDAF contains both logical functions and is denoted as NWDAF, unless explicitly mentioned as NWDAF(AnLF) and NWDAF(MTLF).

NOTE 2:	Pre-trained ML model storage and provisioning to NWDAF is out of the scope of 3GPP.

NOTE 3:	Sharing of models or model meta data is limited to single vendor environments in this Release of the specification.

Analytics information are either statistical information of the past events, or predictive information.

Different NWDAF instances may be present in the 5GC, with possible specializations per type of analytics. The capabilities of a NWDAF instance are described in the NWDAF profile stored in the NRF.

In order to support NFs to discover and select an NWDAF instance containing MLTF, AnLF, or both, that is able to provide the required service (e.g. analytics exposure or ML model provisioning) for the required type of analytics, each NWDAF instance should provide the list of supported Analytics ID(s) (possibly per supported service) when registering to the NRF, in addition to other NRF registration elements of the NF profile. NFs requiring the discovery of an NWDAF instance that provides support for some specific service(s) for a specific type of analytics may query the NRF for NWDAFs supporting the required service(s) and the required Analytics ID(s).

The consumers i.e. 5GC NFs and OAM decide how to use the data analytics provided by NWDAF.

The interactions between 5GC NF(s) and the NWDAF take place within a PLMN.

The NWDAF has no knowledge about NF application logic. The NWDAF may use subscription data but only for statistical purpose.

The NWDAF architecture allows for arranging multiple NWDAF instances in a hierarchy/tree with a flexible number of layers/branches. The number and organisation of the hierarchy layers, as well as the capabilities of each NWDAF instance remain deployment choices.

In a hierarchical deployments, NWDAFs may provide data collection exposure capability for generating analytics based on the data collected by other NWDAFs, when DCCF, MFAF are not present in the network.

NOTE: The procedures for data collection for UE related analytics need to take user consent into account, how to do this depends on the outcome of the SA3 WG.

----------------------------------------End of Changes--------------------------------------------