3GPP TSG-SA2 Meeting #145E	S2-2103815r04

# Electronic,  May 17 – May 28, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0335 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Data collection from UE Application |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Inconsistent description of the Data collection from UE Application functionality. - Clarify terminology on the Application ID provided by NWDAF and by the UE Application. - Clarify that the ASP provides to the UE Application the Application ID that will be used later when collecting data from the UE Application. - The LS response from SA4 clarifies that there may be a NAT deployed in the operator´s domain, given that the use of NATs has not been studied, the proposal is to state that NATs are not supported in this Release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The specification text is corrected to describe that the Data is collected fom the UE Application (and not from the UE). - Move AF discovery and selection to 23.501 as for the rest of the NF discovery and selection. - Replace “internal application ID” “Application ID known in the core network”. - Clarify that “external application ID” is “Application ID configured in the UE Application”. Clarify that the Application ID configured in the UE Application may be OSid specific. Then the OSAppId defined in 23.503 is used: “An identifier associated with a given application and uniquely identifying the application within the UE for a given operating system.” , but it may also be no OSid specific when the application is running on a web browser. - Clarify that NATs are not supported in this Release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions for KI#8 are not defined in normative phase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.8, 6.2.8.1, 6.2.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------First Change---------------------------------------------

### 6.2.8	Data Collection from the UE Application

#### 6.2.8.1	General

The NWDAF may interact with an AF to collect data from UE Application(s) as an input for analytics generation. The AF can be in the MNO domain or an AF external to MNO domain. The data collection request from NWDAF may trigger the AF to collect data from the UE Application.

The UE Application establishes a connection to the AF in the MNO domain or external to MNO domain over user plane via a PDU session. The AF communicates with the UE Application and collects data from UE Application.

For both an AF in trusted domain and an AF in untrusted domain (which supports to collect data from a UE Application), the SLA between the operator and the Application Service Provider (i.e. ASP) determines per Application ID in use by the ASP:

-	The AF for the UE Application to connect to (e.g. based on an FQDN).

-	The information that the UE Application shares with the AF, subject to user consent.

-	Possible Data Anonymization, Aggregation or Normalization, algorithms (if used).

-	The authentication information that enable the AF to verify the authenticity of the UE's Application that provides data.

NOTE 1:	The mutual authentication info that is used by the UE Application and the AF and how user consent is obtained is out of SA WG2's scope.

The AF (which supports the data collection) is configured based on the SLA above.

A UE Application (which supports to providing data to an AF) is configured by the ASP with the Application ID to use in the communication with the AF and then the UE Application is configured per Application ID with the below information:

-	The address of the AF to contact.

-	The parameters that the UE Application is authorized to provide to the AF.

-	The authentication information to enable the UE Application to verify the authenticity of the AF that requests data.

NOTE 2:	The authentication and authorization info that is used by the UE Application and the AF for collection and how user consent is obtained is out of SA2's scope.

NOTE 3:	The configuration procedure for the above information from the ASP to the UE's Application is out of SA WG2 scope.

NOTE 4:	The Application ID configured in the UE Application can either be an OSAppId as defined in TS 23.503 [4] or an OS independent Application Identifier (e.g. for applications running on a web browser).

The Target for Event Reporting may be set as a GPSI in the Naf_EventExposure. The GPSI may be an External Identifer for individual UE as defined in TS 23.501[2] that includes the domain name. This domain name and the Application ID configured in the UE Application are different from each other.

#### 6.2.8.2	Procedure for data collection from the UE Application

##### 6.2.8.2.1	Connection establishment between UE Application and AF

The UE Application receives the data collection configuration from ASP. The configuration information is as described in clause 6.2.8.1.

The UE Application establishes a user plane connection to the AF. Data collection procedure from the UE Application is performed via the user plane connection.

NOTE 1:	Whether multiple user plane connections are established, or a single user plane connection is established for different applications between each UE Application  and AF is based on implementation that is out of 3GPP scope.

NOTE 2:	The Connection establishment procedure from the UE Application to the AF as above is out of scope of the present specification. For the 3GPP defined services, the Connection establishment procedure is in the scope of SA WG4. For the non-3GPP defined services, the Connection establishment procedure is out of 3GPP's scope.

NOTE 3:	In order to preserve resources (e.g. battery, quota) for the end user, a user plane connection to the AF should only be established when the UE has an active PDU Session for the UE Application and it is actively using the network (i.e. it should not be established when the UE Application is inactive, or used in an off-line mode).

Both direct data collection procedure (from the UE Application to the AF, either in trusted domain or untrusted domain) and indirect data collection procedure (from the UE Application to the Application server and from the Application server to the AF) shall be supported. The data collection procedure is out of scope of the present specification.

Editor's note:	Appropriate reference to SA4 specification for data collection procedure is needed.

The UE Application provides the Application Id configured in the UE Applicationto the AF. The AF stores it,  along with the retrieved IP address of the UE (in the PDU session used) in order to request data collection from the UE Application. The UE IP address is used by the AF to identify the user plane connection.

NOTE 4:	An operator can deploy NAT functionality in the network; Data collection from the UE Application when NAT is deployed is not specified in this release of the specification.

Editor's note:	The procedure on how AF obtains the Application ID configured in the UE Application will be added once SA4/SA3 develops the procedure.

##### 6.2.8.2.2	AF registration and discovery

The AF registers its available NF profile to the NRF. The AF in trusted domain registers to the NRF by using the Nnrf_NFManagement service that is defined in clause 5.2.7.2 of TS 23.502 [3]. The AF in untrusted domain registers the available NF profile to the NRF via the NEF as described in clause 6.2.2.3.

The AF discovery and selection is described in TS 23.502 [3].

##### 6.2.8.2.3	Data Collection Procedure from UE

Figure 6.2.8.2.3-1: Data Collection Procedure from UE

1.	An NF subscribes to Analytics from the NWDAF as described in clause 6.1.1.1, that includes Analytics ID, Analytics Filter Information including e.g. AoI, Internal Application ID(s) and Target of Analytics Reporting. NWDAF may also initiate the data collection prior to this subscription.

2.	NWDAF discovers the AF that provides data collection (based on the AF profiles registered in NRF) as described in TS 23.502 [3].

Step 3a is used for the AF in trusted domain while step 3b is used for the AF in untrusted domain.

3a.	NWDAF subscribes to the AF in trusted domain for UE data collection input data for analytics, by using Naf_Event_Exposure_Subscribe as defined in clause 5.2.19.2 of TS 23.502 [3]. The NWDAF requests contains an Application ID known in the core network and the UE Application provides the Application ID configured in the UE Application The AF binds the NWDAF request for an Application Id and the UE data collection for an Application Id configured in the UE.

3b.	NWDAF subscribes to the AF in untrusted domain for UE data collection input data for analytics, by using step 2 and step 3 of the procedure that is described in Figure 6.2.2.3-1).

4.	The AF collects the UE data using either direct or indirect data collection procedure in clause 6.2.8.2.1. The establishment of the connection can be performed at any time prior to this. The AF links the data collection request from step 3 to the user plane connection as described in clause 6.2.8.2.4.

NOTE 1:	The Direct data collection and indirect data collection procedure is decided by SA4.

Editor's note:	Appropriate reference to SA4 specification for data collection procedure is needed.

Step 5a is used for the AF in trusted domain while step 5b is used for the AF in untrusted domain.

5a.	The AF in trusted domain receives the input data from the UE, and processes the data (e.g. anonymizes, aggregates and normalizes) according to the SLA that is configured in the AF described in clause 6.2.8.1, Event ID(s) and Event Filter(s) set during step 3a. The trusted AF then notifies the NWDAF on the processed data collected according to the NWDAF subscription in step 3a.

5b.	The AF in untrusted domain receives the input data from the UE, and processes the data (e.g. anonymizes, aggregates and normalizes) according to the SLA that is configured in the AF described in clause 6.2.8.1, Event ID(s) and Event Filter(s) set during step 3b. The untrusted AF notifies the NWDAF on the processed data collected by using step 5b (i.e. Step 4 and step 5 of the procedure that described in Figure 6.2.2.3-1).

NOTE 2:	If the target of analytics reporting is "any UE", the AF either before step 5a (the AF in trusted domain) or before step 5b (the AF in untrusted domain) can process (e.g. anonymize, aggregate and normalize) the data from multiple UEs according to the SLA configured in the AF, Event ID(s) and Event Filter(s) set during step 3a or 3b before notifying the NWDAF on the processed data.

6.	The NWDAF produces Analytics.

7.	The NWDAF provides analytics to the consumer NF.

Editor's note:	Any steps related to architecture enhancement (e.g. DCCF) in this procedure will need further updates.

---------------------------------------- End of Changes------------------------------------------