3GPP TSG-SA2 Meeting #145-e e-meeting	S2-2103878r02

Elbonia, May 17th – 28th, 2021	(was S2-2103250, merges s2-2104031)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0299 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME |  | Radio Access Network |  |  |  |  |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Resolve ENs for clause 6.1A.3.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | CATT, Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  |  |  |  |  | Date: |  |  | 2021-05-17 |  |  |  |
| Category: | F |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-17 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |  |  |
| Reason for change: |  | Regarding “Editor's note: It is FFS if other mechanism can be used, such as a query to UDM, that doesn't require the location query”, As the TS 23.288 allows a DCCF to query UDM to obtain the NWDAF serving UE, it is propose to also allow the aggregator NWDAF to query UDM to obtain the NWDAF serving the UE. Regarding “Editor's note: Whether the NF service area is obtained from that NF or NRF is FFS.” In our views, the NF service area should be retrieved from NRF by requesting NF profile, since there is no procedure for a NWDAF to request NF service area from NF itself. Regarding “Editor's note: It is FFS how to find an NWDAF that already receives data from an NF, and thereby minimizing the data collection from the NFs.” This should be a general issue, since it is applicable for any NWDAF discovery case. We propose to remove it or move it into general part for NWDAF discovery. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. removing some Ens 2. add a NOTE to clarify how the NWDAF obtain UE location. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The issues indicated by the ENs still exist |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1A.3.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 2 adds solves an EN by adding the option for the aggregator NWDAF to query UDM for discoverying an NWDAF serving the UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

6.1A.3.2	Procedure for Analytics Aggregation without Provision of Area of Interest

The procedure depicted in figure 6.1A.3.2-1 is used to address cases where an NWDAF service consumer requests Analytics ID(s) without providing an Area of Interest, but requires multiple NWDAFs to collectively serve the request.

Figure 6.1A.3.2-1: Procedure for analytics aggregation without provision of Area of Interest

1.	This step is a NWDAF discovery procedure without providing any area of interest. NRF may return multiple NWDAF candidates matching the requested capabilities and supported Analytics ID(s).

Depending on the requested Analytics ID, the NWDAF service consumer, e.g. based on internal logic, can be able to determine which NWDAF should be selected for providing the required data analytics. If not, the NWDAF service consumer should select a NWDAF with large serving area from the candidate NWDAFs which supports analytics aggregation, e.g. NWDAF 1.

2.	NWDAF service consumer sends Analytics information or analytics subscription request to the aggregator NWDAF, i.e. NWDAF 1 in the Figure 6.1A.3.2-1. In the request, NWDAF service consumer provides the requested Analytics ID(s), e.g. Analytics ID 1.

Once receiving the request, the Aggregator NWDAF 1 may decide to subscribe data analytics from other NWDAF instances which can provide the requested data analytics. Based on the analytics ID, there are two cases for the Aggregator NWDAF 1 to subscribe data analytics from other NWDAF instances:

3a.	If the data analytics requires UE location information, e.g. for the analytics ID "UE Mobility", "Abnormal behaviour", or "User Data Congestion", the Aggregator NWDAF 1 can:

-	query UDM to discover the NWDAF serving the UE first if itsupports; or

-	determine which NWDAF can provide the required analytics for the area that UE locates by querying NRF.

NOTE 1:	If the Aggregator NWDAF 1 does not get the required NWDAF serving the UE from the UDM due to, e.g., no suitable NWDAF serving UE registered to the UDM, it has to still query the NRF.

NOTE 2:	The Aggregator NWDAF 1 firstly determines the AMF serving the UE as specified in the clause 6.2.2.1, then requests UE location information from the AMF, and queries the NRF with the tracking area where the UE is located.

NOTE 3:	If an Aggregator NWDAF receives an Analytics request for a group of UEs, i.e. the Target of Analytics Reporting set to an Internal Group ID, it performs NWDAF discovery based on location information of all UEs in the group, and then requests all discovered NWDAFs to provide the required analytics.

3b.	If the data analytics does not require to collect UE location information, e.g. for the analytics ID "Service Experience", "NF load information", or "UE Communication", the Aggregator NWDAF 1 can determine the NFs to be contacted for data collection as specified in the clause 6.2.2.1, then discover other NWDAFs that can collect data from such NFs to generate the required analytics. In order to discover such NWDAFs, the Aggregator NWDAF 1 firstly retrieves NF service area from each of the data source NF from NRF.

4.	If the Aggregator NWDAF 1 determines to query the NRF to discover the required NWDAF, it sends a NF discovery request(s) including UE location (e.g. TAI-1) or NF serving area (e.g. TAI list-1) as a filter to NRF, and obtains candidates target NWDAF(s) that can provide the required analytics. There are two cases:

Editor's note:	It is FFS how to find an NWDAF that already receives data from an NF, and thereby minimizing the data collection from the NFs.

5a.	If a single target NWDAF (e.g. NWDAF 2) can provide the requested analytics data, the Aggregator NWDAF (e.g. NWDAF 1) can redirect the AnalyticsInfo_Request to that target NWDAF or request an analytics subscription transfer to that target NWDAF, depending on the type of the analytics request/subscription received by the NWDAF Service Consumer.

5b.	If the Aggregator NWDAF decides to request data analytics from one or more target NWDAFs, the steps 4-9 of the analytics aggregation procedure in clause 6.1A.3.1 are executed.

* * * End of changes * * *