3GPP TSG-SA WG2 Meeting #145E	S2-2103882r03

# e-meeting, May 17 – 28, 2021			(merge of S2-2103830, S2-2103882 and S2-2104571)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0260 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating clause 5.2 for the discovery of NWDAFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT? , NTT DOCOMO? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_ph2 |  |  |  |  |  | Date: |  |  | 2021-05-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is a need to update clause 5.2 on discovery of NWDAF |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Moved all text that is procedure like to 6.1x (new clause for selection and re-selection procedures for NWDF). Such text shall be described in clause 6. -Adding UDM as possible discovery source -Adding that PCF may receive information from AMF and SMF on used NWDAFs thereby solving following ENs in clause 5.2: Editor's note: How the selected NWDAF and / or NRF is aware of the tracking area where the UE is located is FFS. Editor's note: If multiple NWDAFs are serving the same tracking area, it is FFS to determine which one should be selected. Editor's note: It is FFS whether the selected NWDAF can query UDM to obtain the target NWDAF. Editor's note: It is FFS if other mechanism can be used to determine the target NWDAF, e.g. other parameters in the query to NRF. This version includes a sentence for selection of a co-located NWDAF and using the locality parameter as well |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NWDAF discovery is underspecified |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.288 CR 0261 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | As a consequence of approving this CR, it shall be implemented before CR0290R1. And when implementing CR0290R1 doing it as indented, since it connects well with last indented part in this CR. And at the end of the clause, CR0291R1 should be implemented. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | With respect to the original version of the CR, the deployment options in an Annex have been removed. |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested Machine Learning (ML) model information by using the NWDAF discovery principles defined in clause 6.3.13, TS 23.501 [2]. This clause describes how to perform the NWDAF discovery and selection, e.g. which parameters, from those specified in clause 6.3.13, TS 23.501 [2], to be used.

Different deployments (see Annex X) may require different discovery and selection parameters. Different ways to perform discovery and selection mechanisms depend on different types of analytics/data (NF related analytics/data and UE related analytics/data). NF related refers to analytics/data that do not require a SUPI nor group of SUPIs (e.g. NF load analytics). UE related refers to analytics/data that requires SUPI or group of SUPIs (e.g. UE mobility analytics).

A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signaling described in TS 23.502 [3] clause 4.16. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

In order to discover an NWDAF using the NRF:

If the analytics is related to NF(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer selects an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF with aggregation capability, the consumer selects the NWDAF with aggregation capability with large serving area. If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF queries the NRF with the service area of the NF to be contacted to determine another target NWDAF.

If the analytics is related to UE(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer selects an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF with  aggregation capability, the consumer select the NWDAF with aggregation capability with large serving area.

If the selected NWDAF cannot provide analytics for the UE(s) (e.g, the NWDAF serves a different serving area), the selected NWDAF determines the AMF serving the UE as specified in the clause 6.2.2.1, requests UE location information from the AMF, and queries the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

In order to discover an NWDAF that has registered in UDM for a given UE:

NWDAF service consumers or other NWDAFs interested in UE related data or Analytics, if support, may make an first query to UDM to discover an NWDAF instance that is already serving a UE.

In order to discover an NWDAF based on NWDAF set ID and NF type parameters in NRF:

When a data source is associated to one NWDAF, the NWDAF collects the data from this data source. The NWDAF registers this association in NRF by including the NF Set ID or the NF type of the data source. NWDAF service consumers or other NWDAFs may query NRF, making use NWDAF Set ID or NF type to discover an NWDAF instance that is associated to a data source NF. This query to NRF requires that the consumer has previously determined the NF Set ID or NF Type of the data source serving the UE, this can be learned, for a specific UE, as indicated in Table 5A.2-1.

* * * End of Changes * * * *