3GPP TSG-SA WG2 Meeting #145E	S2-2103883r06

# e-meeting, May 17 – 28, 2021	(was part of S2-2102185, partial merge of S2-2104550 and S2-2104275)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0339 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF re-selection for Multiple NWDAF deployments – procedures -update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT?, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Re-selection with transfer of Analytics Context information procedure has been introduced in meeting SA#143. Some updates to the re-selection procedure is done to encompass preparation phase. It is also included how this procedure can live together with selection of NWDAF procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Resolving the EN whether to combine the selection and re-selection procedure or not, by making clear the difference between the two procedures. Adding an indication in the Analytics Subscription on the consumer’s serving area. Solving following ENs Editor's note: It is preferably to have one procedure to cover the re-selection of NWDAF covering both when consumer change and when not. But it is FFS how. Editor's note: It FFS how this aligns with the procedure when Analytics consumer changes -Adding the use case to the re-selection procedure to also encompass re-selection in conjunction with discovery and selecton of NWDAF (was earlier part of clause 5.2 NWDAF discover and selection. -Putting the prepared Analytics Subscription procedure into separate clause |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | -Procedure for re-selection with transfer of Analytics Context information is inconsistent with selection procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1B.1, 6.1B.2, 7.2.2, 7.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

6.1B	Procedures for analytics subscription transfer

6.1B.1	General

In multiple NWDAFs deployment scenario, it is possible that, within the time period of data collection and/or analytics computation, an NWDAF instance is required to transfer one or more of its analytics subscriptions to another NWDAF instance due to internal (e.g. load balancing, graceful shutdown) or external triggers (e.g. UE mobility, request from another NWDAF instance). For external triggers, NWDAF may subscribe with NF(s) to be notified about corresponding events. As for UE mobility, upon the UE location change event notified by AMF subscribed by NWDAF, NWDAF determines whether it can continue to provide the analytics service, or should trigger reselection of NWDAF to make analytics subscription transfer, e.g., when the serving area of the target NWDAF is within the serving area of the analytics consumer NF.

Procedures for analytics subscription transfer allow one NWDAF instance to transfer or handover its ongoing analytics subscriptions to another NWDAF instance. The transfer can be done for all subscriptions or just a selected subset of subscriptions related to specific area(s), specific Analytics ID(s), and/or specific UE(s).

The analytics subscription transfer can also be prepared if the source NWDAF instance anticipates that it will soon not be able to continue its current analytics tasks.

6.1B.2	Analytics Subscription Transfer

6.1B.2.1	Analytics Subscription Transfer initiated by source NWDAF

The procedure in Figure 6.1B.2.1-1 is used by an NWDAF instance to request the transfer of analytics subscription(s) to another NWDAF instance, using the Nnwdaf_AnalyticsSubscription_Transfer service operation defined in clause 7.2.5.

Editor's note:	It is FFS whether this procedure is useful for any other analytics than UE related.

NOTE 1:	Handling of overload situation or preparation for a graceful shutdown are preferably executed inside an NWDAF Set, when available, therefore, not requiring an analytics subscription transfer as described in this clause. The procedure in Figure 6.1B.2.1-1 is applicable for analytics subscription transfer across NF Sets (when available) or if the NWDAFs are not deployed in a Set.

Figure 6.1B.2.1-1: Analytics subscription transfer initiated by source NWDAF

0.	Source analytics consumer subscribes to Analytics from source NWDAF. The Analytics Consumer may send its service area information to the NWDAF,. Optionally the source NWDAF subscribes to UE mobility events.

1.	[Optional] Source NWDAF determines, e.g. triggered by a UE mobility event notification, to prepare an analytics subscription transfer to target NWDAF(s), as specified in the procedure illustrated in clause 6.1B.2.2.

2.	Source NWDAF determines, e.g. based on the UE location information and the NF consumer’s serving area received, the analytics subscription(s) to be transferred. NWDAF discovery may be skipped if the target NWDAF had already been discovered as part of a prepared analytics subscription transfer in step 4. NWDAF discovery and selection is specified in clause 6.3.13, TS 23.501 [2]

3.	Source NWDAF performs an NWDAF discovery and selects the target NWDAF. NWDAF discovery may be skipped if the target NWDAF had already been discovered as part of a prepared analytics subscription transfer in the preparation procedure (see step 1).

4.	Source NWDAF requests, using Nnwdaf_AnalyticsSubscription_Transfer Request service operation, a transfer of the analytics subscription(s) determined in step 2 to the target NWDAF. The request contains a callback URI of the analytics consumer. The request also contains Id(s) of active data sources related to the analytics subscriptions requested to be transferred, if not already provided as part of the prepared analytics subscription transfer in the preparation procedure (see step 1). The request message may also include "analytics context identifier(s)" indicating the availability of analytics context for particular Analytic ID(s).

NOTE 2:	If multiple notifications/requests are received, indicating the necessity to transfer analytics subscriptions, the source NWDAF will only trigger a single analytics subscription transfer and ignore additional notifications/requests that would result in the same analytics subscription transfer.

NOTE 3:	If Source NWDAF learns that the analytics consumer is not serving the UE any more (e.g. based on the analytics consumer's service area and UE location), Source NWDAF does not initiate transfer of the analytics subscription(s) subscribed by the analytics consumer to a new NWDAF.

5.	Target NWDAF accepts the analytics subscription transfer and takes over the analytics generation based on the information received from the source NWDAF.

NOTE 4:	If not yet done during a prepared analytics subscription transfer, the target NWDAF allocates a new subscription Id to the received analytics subscriptions, following the behaviour of AmfEventSubscription as specified in TS 29.518 [25] clause 6.2.6.2.2 and UEContextTransfer as specified in TS 29.518 [25] clause 5.2.2.2.1.1.

NOTE 5:	The target NWDAF might already have received information on some/all of the analytics subscriptions as part of the prepared analytics subscription transfer request received in step 1 and, thus, might already have started to prepare for the analytics generation, e.g. by having already subscribed to relevant event notifications.

6.	Target NWDAF informs the analytics consumer about the successful analytics subscription transfer using a Nnwdaf_AnalyticsSubscription_Notify message and providing the new subscription Id, that had been assigned by the target NWDAF, in the Subscription Change Notification Correlation Id parameter of this message as specified in clause 7.2.4.

NOTE 6:	The existing Analytics context in the source NWDAF is not deleted directly but will be purged first when it was collected by the target NWDAF.

7.	[Conditional] The target NWDAF, based on the "analytics context identifier(s)" or Subscription Correlation ID provided in step 4, requests the "analytics context". The analytics context information transfer procedure is specified in clause 6.1B.3.

Editor's note:	It is FFS, whether procedure in clause 6.1B.3 is specified, or other new/existing service operations are used instead.

8.	[Optional] Target NWDAF subscribes to relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the Analytics.

9.	Target NWDAF confirms the analytics subscription transfer to the source NWDAF.

10.	[Optional] Source NWDAF unsubscribes with the data source(s) that are no longer needed for the remaining analytics subscriptions.

11-12.	Target NWDAF at some point derives new output analytics based on new input data and notifies the analytics consumer about the new analytics using a Nnwdaf_AnalyticsSubscription_Notify message as specified in clause 6.1.1.

NOTE 7:	Notification correlation information allows the analytics consumer to correlate the notifications to the initial subscription request made with the source NWDAF in step 0.

6.1B.2.2	Analytics subscription transfer with prepared analytics subscription transfer

The procedure in Figure 6.1B.2.2-1 is used by an NWDAF instance to request another NWDAF instance for a prepared analytics subscription(s) transfer from the source NWDAF instance, using the Nnwdaf_AnalyticsSubscription_Transfer service operation defined in clause 7.2.5.

NOTE 1:	The source NWDAF might determine that it needs to prepare to transfer analytics subscriptions to another NWDAF instance, as the source NWDAF estimates that the UE might enter an area which is not covered by the source NWDAF, e.g. by subscribing to AMF event exposure service for UE mobility event notifications, by performing UE mobility analytics, or by subscribing to another NWDAF providing UE mobility analytics.

NOTE 2:	The source NWDAF might also determine that it needs to prepare to transfer analytics subscriptions to another NWDAF instance, as the source NWDAF wants to resolve an overload situation or prepare for a graceful shutdown.

NOTE 3:	Handling of overload situation or preparation for a graceful shutdown are preferably executed inside an NWDAF Set, when available.

Figure 6.1B.2.1-1: Analytics subscription transfer with prepared analytics subscription transfer

1a.	Analytics consumer subscribes to the source NWDAF for certain analytics as specified in clause 6.1.1.

1b.	[Optional] Source NWDAF starts data collection from relevant data source(s) (e.g. NFs or OAM) as specified in clause 6.2. Source NWDAF starts generating requested analytics.

2.	[Optional] The source NWDAF subscribes, using Namf_EventExposure_Subscribe and/or Nsmf_EventExposure_Subscribe service operation, to receive notifications on UE mobility events from AMF and/or SMF, respectively.

2b.	The source NWDAF determines that it needs to prepare to transfer analytics subscriptions to another NWDAF instance. This may be the case, e.g., due to the UE moving to a location outside the serving area of the source NWDAF serving area.

3.	The source NWDAF discovers candidate target NWDAF instances (e.g. NWDAFy and NWDAFz) supporting the requested analytics information for the predicted target area(s). NWDAF discovery and selection is specified in clause 6.3.13, TS 23.501 [2].

NOTE 4:	In this procedure, NWDAFy and NWDAFz are examples for target NWDAF instances that are candidates to take over those analytic subscriptions.

4-5.	In the case of a prepared analytics subscription transfer, the source NWDAF requests, using Nnwdaf_AnalyticsSubscription_Transfer Request, to the candidate target NWDAFs (e.g. NWDAFy and NWDAFz) to prepare for an analytics subscription transfer by including a "prepared analytics subscription transfer indication" in the request message. The request message also includes information on the analytics subscriptions to be transferred. The request message may include "analytics context identifier(s)" indicating the availability of analytics context for particular Analytics ID(s).

The candidate target NWDAFs (e.g. NWDAFy and NWDAFz) respond to the request from the source NWDAF using a Nnwdaf_AnalyticsSubscription_Transfer Response message.

6-7.	[Conditional] The determined target NWDAFs (e.g. NWDAFy and NWDAFz), based on the "analytics context identifier(s)" or Subscription Correlation ID provided in step 4, may request the "analytics context" from the source NWDAF. The analytics context information transfer procedure is specified in clause 6.1B.3.

Editor's note:	It is FFS, whether procedure in clause 6.1B.3 is specified, or other new/existing service operations are used instead.

NOTE 5:	The target NWDAFs (e.g. NWDAFy and NWDAFz) allocate a new subscription Id to the received analytics subscriptions, following the behaviour of AmfEventSubscription as specified in clause 6.2.6.2.2, TS 29.518 [25] and UEContextTransfer as specified in clause 5.2.2.2.1.1, TS 29.518 [25].

8.	[Optional] Based on the information received from the source NWDAF, the target NWDAFs (e.g. NWDAFy and NWDAFz) start data collection from NFs or OAM (as specified in clause 6.2) and analytics generation for the indicated analytics subscriptions.

The source NWDAF determines to transfer analytics subscription(s) to a target NWDAF instance NWDAF (e.g. NWDAFy) and the following steps are executed:

9.	Source NWDAF initiates the analytics subscription transfer to the target NWDAF as specified in steps 3 to 5b of the analytics subscription procedure illustrated in Figure 6.1B.2-1.

NOTE 136:	If the target NWDAF (e.g. NWDAFy) had accepted to prepare for an analytics subscription based on a prepared analytics subscription transfer request received from the source NWDAF in step 4, the "analytics context identifier(s)" provided in the Nnwdaf_AnalyticsSubcscription_Transfer request in step 9 indicate additional information / data collected or generated by the source NWDAF between step 6 and step 9.

10.	[Conditional] If a candidate target NWDAF (e.g. NWDAFz) had accepted to prepare for an analytics subscription based on a prepared analytics subscription transfer request received from the source NWDAF in step 5, but had not been selected as the new target NWDAF, the source NWDAF cancels the prepared analytics subscription transfer to this target candidate NWDAF (e.g. NWDAFz), using Nnwdaf_AnalyticsSubscription_Transfer Request include an "analytics subscription transfer cancel indication". The target NWDAF (e.g. NWDAFz) confirms the cancelation to the source NWDAF and, if applicable, deletes any analytics data that is no longer needed. If the target NWDAF (e.g. NWDAFz), as part of the analytics subscription preparation, had already subscribed to entities to collect data, it unsubscribes to those entities if the subscriptions are not needed for other active analytics subscriptions with the target NWDAF.

* * * Next Change * * * *

7.2.2	Nnwdaf_AnalyticsSubscription_Subscribe service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Subscribe.

Description: Subscribes to NWDAF analytics with specific parameters.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2, Target of Analytics Reporting, Notification Target Address (+ Notification Correlation ID), Analytics Reporting Parameters, Analytics target period.

NOTE 1:	Target of Analytics Reporting can be provided per individual Analytics ID in a set of Analytics IDs.

Inputs, Optional: Analytics Filter Information, Subscription Correlation ID (in the case of modification of the analytics subscription), preferred level of accuracy of the analytics, accuracy level per analytics subset, Reporting Thresholds, Maximum number of objects requested (max), Preferred order of results, Maximum number of SUPIs requested (SUPImax), time when analytics information is needed, Analytics Metadata Request, Dataset Statistical Properties, Output strategy and Data time window, the consumer NF's serving area.

NOTE 2:	Analytics Filter Information, Reporting Thresholds, Maximum number of objects requested (max), Maximum number of SUPIs requested (SUPImax), Analytics Metadata Request, Dataset Statistical Properties, Output strategy and Data time window can be provided per individual Analytics ID in a set of Analytics IDs.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription).

Outputs, Optional: None.

* * * Next Change * * * *

7.2.5	Nnwdaf_AnalyticsSubscription_Transfer service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Transfer.

Description: Requests to NWDAF to transfer analytics subscription(s) from the consumer NWDAF.

Inputs, Required:

-	Transfer type: indicates the type of the transfer request. The following values are supported:

-	Analytics subscription transfer preparation: requests the NWDAF to prepare for taking over the analytics subscription(s) and/or prepare for collecting Analytics Context(s).

-	Analytics subscription transfer: requests the NWDAF to take over the analytics subscription(s).

-	Analytics subscription transfer cancel: cancels a prepared analytics subscription request.

Inputs, Optional:

-	If this service operation is for "analytics subscription transfer preparation", the following parameters are provided:

-	(Set of) analytics subscription(s) with the following parameters:

-		All input parameters for the analytics exposure as specified in clause 6.1.3.

-	[OPTIONAL] Active data source ID(s): Instance Id or Set Id of the active data source(s) the consumer NWDAF is currently using for the analytics of this analytics subscription.

-	Subscription Correlation Id.

-	Subscription callback URI: indicates the callback URI of the analytics consumer of this subscription, i.e. the URI the analytics notifications are to be sent to.

-	Id of analytics consumer that is subscribed to receive analytics.

-	If this service operation is to request analytics subscriptions transfer and if analytics subscription transfer preparation is not performed, the same parameters as those for Transfer type of "Analytics subscription transfer preparation" shall be provided.

-	If this service operation is to request analytics subscriptions transfer and if analytics subscription transfer preparation is performed, the same parameters as those for Transfer type of "Analytics subscription transfer preparation" can be provided with updated parameter values.

-	If this service operation is to request analytics subscriptions transfer cancel, the following parameter shall be provided:

-	Subscription Correlation Id.

-	(Set of) analytics context identifier(s): identifies analytics context information available at the consumer NWDAF.

Outputs Required: Operation execution result indication.

Outputs, Optional: None.

* * * End of Changes * * * *