SA WG2 Meeting #145E (e-meeting)	S2-2103981

# May 17 – 28, 2021 									                               (revision of S2-2102843)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2833 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of architecture for AF requested support of Time Sensitive Communication and Time Synchronization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIOT |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A lefover EN need to fixed for the architecture for AF requested support of Time Sensitive Communication and/or Architecture for Time Synchronization Editor's note: The architecture for AF requested support of Time Sensitive Communication and/or Time Synchronization is FFS. To support the gPTP/PTP based Time Synchronization service, the DS-TT, NW-TT and NEF are required. When the gPTP/PTP based Time Synchronization service is terminated at the UE, the required DS-TT is collocated with UE. For the other scenario (e.g. non gPTP and/or PTP based Time Synchronization, AF requested support of Time Sensitive Communication without the need of gPTP/PTP), the DS-TT and NW-TT may not present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify and correct the AF requested support of Time Sensitive Communication and/or Architecture for Time Synchronization. Separate the time syncrhonization service locig from the NEF; introduce Time Synchronization NF that contains the time-synchronization service logic, and controls the DS-TT(s) and NW-TT to form (g)PTP instances. Trusted AF may use the services exposed by the TS NF directly, untrusted AF uses the services provided by NEF, which uses the services from TS NF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The architectures for AF requested support of Time Sensitive Communication and/or Architecture for Time Synchronization, is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.4.8.3	Architecture for support of Time Synchronization

This clause describes the architecture to support time synchronization services (see Figure 4.4.8.3-1). The Time Synchronization related features that are supported based on AF request are described in clause 5.27.1.

As shown in Figure 4.4.8.3-1, to support Time Synchronization service based on IEEE Std 802.1AS [104] or IEEE Std 1588-2019 [126] for Ethernet or IP type PDU Sessions, the DS-TT, NW-TT and TS NF (Time Synchronization Network Function) are required in order to support the features in IEEE Std 802.1AS [104] or IEEE Std 1588-2019 [126] as described in clause 5.27. The NEF exposes 5GS capability to support Time Synchronization service as described in clause 5.27.1.8. TS NF controls the DS-TT(s) and NW-TT for the (g)PTP based time synchronization service. Trusted AF may use the time synchronization service capabilities exposed by the TS NF.

For scenarios when the AF requests for5G reference time distribution only, the DS-TT, NW-TT and TS NF may not be present.

Figure 4.4.8.3-1: Architecture for Time Synchronization

Editor's note:	The architecture for AF requested support of Time Sensitive Communication and/or Time Synchronization is FFS.

END OF CHANGES