SA WG2 Meeting #145e	S2-2103999r04

# May 17 – 28th, 2021; Elbonia               			(revision of S2-210)

Source:	Nokia, Nokia Shanghai Bell, Ericsson

Title:	ECS Address Configuration

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract: This contribution clarifies that the ECS information that comes from the H-SMF may point the UE to an ECS that is hosted in the VPLMN.

- Discussion

a few updates on ECS provisioning

- NOTE 2:	Although the Service Provisioning procedure with the ECS can take place over a HR session, the UE needs to establish an LBO PDU Session to access the EES(s) and EAS(s) in VPLMN. As the UE is not aware of whether a PDU Session is working in LBO or in HR mode, in this case the PDU session used to access the EES(s) would need to use another combination of (DNN, S-NSSAI) than the PDU Session working in HR mode.

- The SMF does not need to be aware of ECS Provider ID, ECS ID

- Is ECS ID really useful?

- Text reshuffling, no hanging §

- “ a 3rd party AF can use Nnef_ParameterProvision to provide, update, or delete ECS Address Configuration Information for a UE (See TS 23.502 [3], clause 4.15.6.2) or any UE for a (DNN, S-NSSAI)”. I’d claim that a 3rd party AF needs to provide ECS Address Configuration Information for any UE or for a group of UE but not for a single UE

- Proposal

This contribution proposes to implement the following updates to TS 23.548

* * * Start of Change * * *

6.5.2	ECS Address Provisioning

6.5.2.1	ECS Address Configuration information

The ECS Address Configuration Information consists of one or more FQDN(s) and/or IP Address(es) of Edge Configuration Server(s), and of an ECS Provider ID . It may be associated with spatial validity conditions. It is further described in TS 23.502 [3]. A UE may receive multiple instances of ECS Address Provisioning information (e.g. corresponding to different ECS Provider ID).

The SMF does not need to be aware of the internal structure of the ECS Address Configuration Information but may use the spatial validity conditions to determine when to deliver ECS Address Configuration Information to the UE;

6.5.2.2	ECS Address Configuration information Provisioning to the UE

If the UE hosts an EEC and supports transferring the ECS address received from the 5GC to the EEC, the UE indicates in the PCO at PDU Session establishment or modification that it supports the ability to receive ECS address(es) via NAS and to transfer the ECS Address(es) to the EEC(s). As described in TS 23.502 [3], if the UE supports the ability to receive ECS Address Configuration Information via NAS and to transfer the ECS Address(es) to the EEC(s), the UE may receive ECS Address Configuration Information from the SMF via PCO during PDU Session Establishment and/or during PDU Session Modification procedures.

The SMF ECS Address Configuration Information

The SMF may receive ECS Address Configuration Information and associated spatial validity conditions from the UDM together with SM subscription information. The UDM in the HPLMN may provide the SMF (in HPLMN in HR case, in VPLMN in LBO case) with ECS address configuration information that depends on the serving PLMN of the UE.

The SMF may determine the ECS Address Configuration Information to be sent to the UE based on UE subscription information received from UDM(as described in 4.15.6.3d-2 in TS 23.502[3]);,

The SMF may decide to send updated ECS Address Configuration Information to the UE based on locally configured policy, updated UE subscription information, or a change of UE location (e.g. in non-roaming scenarios, if the UE moves out of the Spatial Validity of ECS Address Configuration Information delivered to the UE, then updated ECS Address Configuration Information may be sent by the SMF to the UE via PDU Session Modification procedure). The PDU Session Modification procedure is used to send updated ECS Address Configuration Information to the UE as described in clause 4.3.3 in TS 23.502 [3].

NOTE 1:	In home routed sessions, the ECS Address Configuration Information comes from the H-SMF. The traffic to the indicated Edge Configuration Server(s) can be transmitted via a PDU session with local breakout.

NOTE 2:	Although the Service Provisioning procedure with the ECS can take place over a HR session, the UE needs to establish an LBO PDU Session to access the EES(s) and EAS(s) in VPLMN. As the UE is not aware of whether a PDU Session is working in LBO or in HR mode, in this case the PDU session used to access the EES(s) would need to use another combination of (DNN, S-NSSAI) than the PDU Session working in HR mode.

NOTE 3:	The Service Provisioning procedure is described in TS 23.558 [5].