3GPP TSG-WG SA2 Meeting #145E e-meeting  	S2-2104084r07

# Elbonia, May 17 – 28, 2021	(revision of S2-2103271)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0293 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add bandwidth into Dispersion analytics per slice |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During the SA2#143E meeting, there is an FFS left for the Data Dispersion Analytics as follows: Editor Note: Consider the addition of “>>Bandwidth” into tables 6.x.3.1-3 and 6.x.3.1-4, which is the utilized bandwidth at the slice calculated by the NWDAF as the volume over the period. See also SA5 Liaison letter S2-2008361 (S5-205436). A related LS Response (S2-2101347) to the S2-2008361/S5-205436 has been approved to clarify that the utilized bandwidth at the slice can be calculated by using the Data Dispersion analytics with following formula: Utilized bandwidth = Data volume dispersed/Duration To align with the LS response, it is proposed to remove the FFS because the LS to SA5 has clarified that the Bandwidth per slice could be calculated by using the the Data Dispersion Analytics, so the OAM can just subscribe or request the Data Dispersion analytics per network slice. Rev 2: According to the conclusion of eNS_Ph2 KI#5, the PCF needs to consume a new Analytics ID or an available Analytics ID that provides UL/DL average throughput per S-NSSAI (i.e. utilized bandwidth). Currently the Data Dispersion Analytics supports the target of analytics reporting as any UE. The outputs of data dispersion statistics/prediction bound by slice should be able to support the case with the target of analytics reporting for any UE. This can also satisfy the requirements from eNS_Ph2 on policy control for data rate per network slice with assistance of the NWDAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the FFS. Rev 2: Align the specification to describe the outputs of data dispersion analytics bound by slice in case that the “target of analytics reporting” is any UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An FFS is left in the TS 23.288 and requirement from eNS_Ph2 cannot be satisfied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.1, 6.10.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.10.1	General

Dispersion analytics identifies the location (i.e. areas of interest, TAs, cells) or network slice(s) where a UE, or a group of UEs, or any UE disperse most (if not all) of their data volume and sessions transactions (i.e. MM and SM messages).

The NWDAF shall be able to provide dispersion statistics or predictions and shall be able to collect UE dispersion related information from NFs.

Dispersion is the percentage of activity that a UE, or group of UEs, or any UE, generates at a location or a slice during a period of interest. This enables the operator to rank hot locations or slices by the various activities and to identify the top contributors (i.e. UEs) for that activity. When heavy UEs at a particular location or slice are detected/predicted and reported to a policy entity, their QoS attributes, or access to an area of interest or a slice can then be limited.

-	Data dispersion - The percentage of data traffic volume that a UE, or a group of UEs, or any UE, generated at a location or in a slice during the period of interest.

-	Transaction dispersion - The percentage of MM and SM messages that a UE, or a group of UEs, or any UE, generated at a location or in a slice during the period of interest.

The operator may classify and assign one of the three mobility classes per dispersion characteristics for a UE or a group of UEs or any UE as fixed, camper or traveller. The classification is based on either Data dispersion or Transaction dispersion.

-	Data-Classification - fixed, camper, traveller data-classification per thresholds assigned by the operator.

EXAMPLE:	When a UE disperses, during the period of observation, most (threshold=95%) of its data at a location or a slice, the dispersion data-classification of the UE at that location or slice is "fixed".

-	Transaction-Classification - fixed, camper, traveller transactions-classifications per thresholds assigned by the operator. Top-Heavy UE is one who's dispersion percentile rating, at a location or a slice, is higher than a threshold value defined by the operator (e.g. 0.9 percentile), see percentile ranking in the output tables, clause 6.10.3.

EXAMPLE:	When a UE disperses, during the period of observation, (threshold=40%) of its session transactions at a location or a slice, the dispersion transaction-classification of the UE is camper at that location or slice.

Based on the dispersion analytics, the analytics consumer can determine that a data or transaction hot spot is formed when the amount of data or amount of transactions dispersed by most of the UEs at the area of interest or slice exceeds a certain data or transaction threshold established through trend analysis or operator policy.

The consumer of dispersion analytics may indicate in its request:

-	Analytics ID set to "Dispersion Analytics"

- Dispersion Analytics type set to Data Dispersion Analytics (DDA) or Transaction Dispersion Analytics (TDA).

-	Target of analytics reporting: single UE, any UE, or a group of UEs. "Any UE" is only supported for Data Volume Dispersion Analytics.

-	Analytics Filter Information: optional list of TA(s), Area(s) of Interest, Cells, or S-NSSAI, Top-Heavy UEs, Fixed UEs, Camper UEs, Dispersion Analytics type (Data Volume Dispersion Analytics (DVDA) orTransaction Dispersion Analytics (TDA)).

-	Analytics target period indicating the time period over which the statistics or predictions are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

NOTE:	Care must be taken with regards to load when requesting analytics for "Any UE". This could be achieved by utilizing event filters (e.g. Area of Interest for AMF) and possible analytics filters including Top-Heavy UEs, and/or fixed and/or camping UEs and/or Analytics Reporting Information (e.g. SUPImax), or sampling ratio as part of Event Reporting Information.

# * * * * Second change * * * *

#### 6.10.3.1	Data Dispersion Analysis

The data dispersion analytics results provided by the NWDAF can be UE data dispersion statistics as defined in table 6.10.3.1-1 and/or UE data dispersion predictions as defined in table 6.10.3.1-2.

Table 6.10.3.1-1: Data dispersion statistics bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Observed location statistics. |
| >> UE location | TA or cells where the UE dispersed its data. |
| >> Application ID (1..max) | To identify the application for the UE. |
| >>> Data volume dispersed | Data volume dispersed at this location and application, if application ID is provided. |
| >> Data classification | Data mobility classification for this location: fixed, camper, traveller. |
| >> Data usage ranking | Ranking of UE data usage at this location (See NOTE 2). |
| >> Data usage percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served at the location. |
| >> Ratio | Percentage of UEs in the group at the location and observation time. (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. |  |

Table 6.10.3.1-2: Data dispersion prediction bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Predicted location during the analytics target period. |
| >> UE location | TA or cells where the UE is predicted to disperse its data. |
| >> Application ID (1..max) | To identify the application for the UE. |
| >>> Data volume dispersion | Data volume dispersion prediction at this location and application, if application ID is provided. |
| >>>> Confidence | Confidence of this prediction (i.e. data to be dispersed at this location). |
| >> Data classification | Data mobility classification for this location: fixed, camper, traveller. |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this location). |
| >> Data usage ranking | Ranking of UE data usage at this location (See NOTE 2). |
| >> Data usage percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served at the location. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this location). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. |  |

The data dispersion analytics results provided by the NWDAF can also be for a UE or group of UEs data dispersion statistics at a given slice as defined in Table 6.10.3.1-3 and/or dispersion predictions as defined in Table 6.10.3.1-4:

Table 6.10.3.1-3: Data dispersion statistics bound by slice (for a UE or UE group)

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Observed slice statistics. |
| >> S-NSSAI | Slice where the UE or group of UEs dispersed data. |
| >> Application ID (1..max) | To identify the application for the UE at the slice. |
| >>> Data volume dispersed | Data volume dispersed at this slice and application, if application ID is provided. |
| >> Data classification | Data mobility classification for slice: fixed, camper, traveller. |
| >> Data usage ranking | Ranking of UE data usage at this slice. (See NOTE 2). |
| >> Data usage percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served by the slice. |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. |  |

Table 6.10.3.1-4: Data dispersion prediction bound by slice (for a UE or UE group)

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Predicted slice during the Analytics target period. |
| >> S-NSSAI | Slice where the UE is predicted to disperse its data. |
| >> Application ID (1..max) | To identify the application for the UE at the slice. |
| >>> Data volume dispersion | Data volume dispersion prediction at this slice and application, if application ID is provided. |
| >>>> Confidence | Confidence of this prediction (i.e. data to be dispersed at this slice). |
| >> Data classification | Data mobility classification for this slice: fixed, camper, traveller. |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this slice). |
| >> Data usage ranking | Ranking of UE data usage at this slice. See NOTE 2. |
| >> Data usage percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served by the slice. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this slice). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. |  |

When the Target of analytics reporting is set as "any UE", the data volume dispersion analytics results provided by the NWDAF can be for all UEs data volume dispersion statistics at a given slice as defined in Table 6.10.3.1-x and/or data volume dispersion predictions as defined in Table 6.10.3.1-y:

Table 6.10.3.1-x: Data dispersion statistics bound by slice (for any UE)

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Observed slice statistics. |
| >> S-NSSAI | Slice where all UEs dispersed data. |
| >> Data volume dispersed | Data volume dispersed at this slice and application, if application ID is provided. (see NOTE 1). |
| NOTE 1: Utilized bandwidth at the slice can be calculated by using the Data volume dispersed for the Duration. |  |

Table 6.10.3.1-y: Data dispersion prediction bound by slice (for any UE)

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Predicted slice during the Analytics target period. |
| >> S-NSSAI | Slice where the UE is predicted to disperse its data. |
| >> Data volume dispersion | Data volume dispersion prediction at this slice and application, if application ID is provided. (see NOTE 1). |
| >>>> Confidence | Confidence of this prediction (i.e. data to be dispersed at this slice). |
| NOTE 1: Utilized bandwidth predication at the slice can be calculated by using the Data volume dispersion for the Duration. |  |

NOTE:	The Application ID in the Data Dispersion Analytics is optional. When the Application ID is missing, the Data Dispersion Analytics is applied across all the applications in an AoI or a slice.

# * * * * End of changes * * * *