3GPP TSG SA WG2 Meeting SA2#145E 		S2-2104157

17 – 28 May, 2021, Electronic, Elbonia

Title:	[Draft] Reply LS on UPF support for multiple network slices

Response to:	LS on UPF support for multiple network slices (S2-2103766/C4-212560)

Release:	Rel-17

Work Item:	BEPoP

Source:	China Telecom (to be changed to SA2)

To:	CT4

Cc:	SA5

Contact person: 	Jiayifan Liu, Yubing Liu

liujyf@chinatelecom.cn

liuyb9@chinatelecm.cn

1	Overall description

SA2 would like to thank CT4 for the LS on UPF support for multiple network slices.

SA2 has discussed the questions and would like to provide the following replies:

CT4 Q1: Do SA2 and SA5 think that the second solution is a valuable optional alternative to the existing solution?

SA2 reply: Yes. There are different requirements under various business scenarios where one UPF is shared by multiple network slices. Solution 1 requires the ratio of S-NSSAI and Network Instance should be 1:1, which increase the required provisioning in SMF and UPF to fulfil other requirements. Solution 2 can provide the flexibility to fulfil the various scenarios without changing the mapping of Network Instance in SMF. And in the case where multiple slices share the same NI, the separate S-NSSAI in solution 2 can help to better differentiate the resources. Therefore, it is considered as a valuable alternative.

CT4 Q2: If the answer to Q1 is YES, do SA2 and SA5 see major impacts to support the second solution and/or have issues with doing so?

SA2 reply: .The impact is that UPF should have a logic/configuration to allocate resources using S-NSSAI together with Network Instance. There are no major impacts.

2	Actions

To CT4

ACTION: 	SA2 kindly requests CT4 to take the above information into account

3	Dates of next TSG SA WG2 meetings

SA WG2 Meeting #146E 		August 16-27, 2021

SA WG2 Meeting #147E		October 18-22, 2021