SA WG2 Meeting #145E e-meeting	S2-2104207r01

Elbonia, May 17 – 28, 2021

Source:	vivo

Title:	TS 23.304: Layer 2 ID for group member discovery

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution.

1.	Discussion

In group member discovery, considering there is a configuration of Layer-2 Group ID being used as Layer-2 ID, thus it is no need to pre-configure UE with a mapping of ProSe service to Destination Layer-2 ID anymore. The Announcing UE just takes the Layer-2 Group ID as the Destination Layer-2 ID and self-assigns a layer-2 ID as the Source layer-2 ID to send the Group Member Discovery message, and then the monitoring UEs just takes the Layer-2 Group ID as the Destination Layer-2 ID for Announcement message reception.

2.	Proposal

It is proposed to capture the following changes into TS 23.304.

# * * * * First change * * * *

#### 5.9.1.2	Destination Layer 2 ID

Destination Layer 2 ID is defined in clause 5.6.1 of TS 23.287 [2].

The Destination Layer-2 ID for ProSe Direct Discovery with Model A is selected based on the configuration as described in clause 5.1.2.1. The Destination Layer-2 ID for a Solicitation message for Model B is selected based on the configuration as described in clause 5.1.2.1.

For Group member discovery in clause 6.3.2.2, the Layer-2 Group ID is taken as the Destination Layer-2 ID for sending and receiving the Group Member Discovery Announcement message and Group Member Discovery Solicitation message.

#### 5.9.1.3	Source Layer 2 ID

Source Layer 2 ID is defined in clause 5.6.1 of TS 23.287 [2].

The UE self-selects a Source Layer-2 ID for ProSe Direct Discovery and Group member discovery.

# * * * * Next change * * * *

#### 6.3.2.2	Group Member Discovery

##### 6.3.2.2.1	General

Group Member Discovery is applicable to public safety use and commercial services. To perform Group Member Discovery, the UE is configured with the related information as described in clause 5.2.

Group Member Discovery is a form of restricted discovery in that only users that are affiliated with each other can discover each other (e.g. only users sharing the same Discovery Group ID).

In the case of Public Safety use, the ProSe Restricted Code is not used for Group Member Discovery, and pre-configured or provisioned information for the Discovery procedures as defined in clause 5.2 is used.

NOTE:	The Group Member Discovery performed by Application Layer in coordination with Application Server is out of scope of this specification.

Both Model A and Model B discovery are supported:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

##### 6.3.2.2.2	Procedure for Group Member Discovery with Model A

Depicted in Figure 6.3.2.2.2-1 is the procedure for Group Member Discovery with Model A.

Figure 6.3.2.2.2-1: Group Member Discovery with Model A

1.	The announcing UE sends a Group Member Discovery Announcement message. The parameters contained in this message are described in clause 5.9.1.

The Destination Layer-2 ID and Source Layer-2 ID used to send the Group Member Discovery Announcement message are specified in clause 5.9.1.2 and clause 5.9.1.3.

The Monitoring UE determines the Destination Layer-2 ID for signalling reception as specified in clause 5.9.1.2.

NOTE:	A UE may send multiple Group Member Discovery Announcement messages (Model A) if the UE belongs to more than one discovery group.

##### 6.3.2.2.3	Procedure for Group Member Discovery with Model B

Depicted in Figure 6.3.2.2.3-1 is the procedure for Group Member Discovery with Model B.

Figure 6.3.2.2.3-1: Group Member Discovery with Model B

1.	The discoverer UE sends a Group Member Discovery Solicitation message. The parameters contained in this message are described in clause 5.9.1.

The Destination Layer-2 ID and Source Layer-2 ID used to send the Group Member Discovery Solicitation message are specified in clause 5.9.1.2 and clause 5.9.1.3.

How the Discoveree UE determines the Destination Layer-2 ID for signalling reception is specified in clause 5.9.1.2.

2.	The discoveree UEs that match the values of the parameters contained in the solicitation message based on the Discovery Group ID, respond to the discoverer UE with a Group Member Discovery Response message. The parameters contained in this message are described in clause 5.9.1.

The Source Layer-2 ID used to send the Group Member Discovery Response message is specified in clause 5.9.1.3. The Destination Layer-2 ID is set to the Source Layer-2 ID of the received Group Member Discovery Solicitation message.

# * * * * End of changes * * * *